NEW ENGLAND POWER POOL

MARKET RULE 1

FERC Electric Rate Schedule No. 7

1.	MARKET OPERATIONS	3

1.1	Introduction.	3

1.2	[Reserved.]	3

1.3	Definitions.	3

1.4	[Reserved.]	24

1.5	[Reserved.]	24

1.6	[Reserved.]	24

1.7	General.	24

1.8	[Reserved.]	28

1.9	Pre-scheduling.	28

1.10	Scheduling.	29

1.11	Dispatch.	40

1.12	Dynamic Scheduling.	43

2.	CALCULATION OF LOCATIONAL MARGINAL PRICES	44

2.1	Introduction.	44

2.2	General.	44

2.3	Determination of System Conditions Using the State Estimator.	45

2.4	Determination of Energy Offers Used in Calculating Real-Time Prices.	46

2.5	Calculation of Real-Time Nodal Prices.	47

2.6	Calculation of Day-Ahead Nodal Prices.	48B

2.7	Reliability Regions, Load Zones, Zonal Prices and External Nodes.	50

2.8	Hubs and Hub Prices.	50A

2.9	Final Real-Time Prices and Day-Ahead Energy Market Results.	51

2.10	Performance Evaluation.	51A

3.	ACCOUNTING AND BILLING	52

3.1	Introduction.	52

3.2	Participants.	52

3.3	[Reserved.]	63

3.4	Non-Participant Transmission Customers.	63

3.5	[Reserved.]	64

3.6	Data Reconciliation.	64

3.7	Eligibility for Billing Adjustments.	65

4.	RATE TABLE	67

4.1	Offered Price Rates.	67

4.2	[Reserved.]	67

4.3	Emergency Energy Transaction.	67

5.	CALCULATION OF TRANSMISSION CONGESTION REVENUE AND CREDITS	68

5.1	Non-Participant Transmission Congestion Cost Calculation.	68

5.2	Transmission Congestion Credit Calculation.	68

6.	RELIABILITY MUST RUN RESOURCES	71

6.1	Definition.  ..	71

6.2	Day-Ahead and Real-Time Energy Market.	71

6.3	[Reserved.]	71

6.4	RMR Operating Reserve Charges.	71

7.	FINANCIAL TRANSMISSION RIGHTS AUCTIONS	73

7.1	Auctions of Financial Transmission Rights.	73

7.2	Financial Transmission Rights Characteristics.	74

7.3	Auction Procedures.	75

8.	INSTALLED CAPACITY	79

8.1	Annual Installed Capacity Requirement.	79

8.2	Requirements Applicable to Participants.	80

8.3	Requirements Applicable to ICAP Resources.	82

8.4	Unforced Capacity Auctions.	90

8.5	Capacity Deficiencies and Deficiency Auctions.	91

9.	FORWARD RESERVE MARKET	94

9.1	Forward Reserve Purchase Amount and Timing.	94

9.2	Timing of Forward Reserve Auctions.	94

9.3	Forward Reserve Auction Offers.	94

9.4	Evaluation of Forward Reserve Auction Offers and Forward Reserve Clearing Prices.	94

9.5	Product Definition and Resource Requirements.	95

9.6	Delivery of Reserve.	97

9.7	Consequences of Delivery Failure.	102

9.8	Forward Reserve Credits and Charges.	104

10.	GAP RFPs FOR RELIABILITY PURPOSES	106

10.1	Request For Proposals for Load Response and Supplemental Generation Resources for Reliability Purposes	106

Appendix A	Market Monitoring, Reporting and Market Power Mitigation	201

Appendix B	Imposition of Sanctions by the ISO	301

Appendix C	Auction Revenue Rights (ARRs) and Qualified Upgrade Awards	401

Appendix D	Alternative Dispute Resolution (ADR) Review	501

Appendix E	Load Response Program (LRP)	601

Appendix F	Operating Reserve Accounting	701

NEPOOL STANDARD MARKET DESIGN

# MARKET OPERATIONS

## Introduction.

This Market Rule sets forth the scheduling, other procedures, and certain general provisions applicable to the operation of the NEPOOL Market within the NEPOOL Control Area.  The ISO shall operate the NEPOOL Market in compliance with NERC, NPCC and NEPOOL reliability criteria.  This Market Rule addresses each of the three time frames pertinent to the daily operation of the NEPOOL Market:  Pre-scheduling, Scheduling, and Dispatch. This Market Rule (except for the provisions relating to the Installed Capacity Market) shall become effective on the SMD Effective Date.  The rules relating to the Installed Capacity Market shall become effective on the ICAP Effective Date.

## [Reserved.]

## Definitions.

### Existing Definitions.

Unless otherwise provided for in Section 1.3.2 of this Market Rule, capitalized terms used but not defined in this Market Rule are as defined in the NEPOOL Filed Documents.

### Definitions.

For purposes of this Market Rule, the following capitalized terms shall have the meanings set forth below.

“ADR“ is alternative dispute resolution.

“Administrative Sanctions” is defined in Section 4.1.2 of Appendix B of this Market Rule.

“Affiliate Resources” is defined in Section 3.3.2 of Exhibit 2 to Appendix A of this Market Rule.

“Accepted Electric Industry Practice”, sometimes referred to as Good Utility Practice, shall mean any of the practices, methods, and acts engaged in or approved by a significant portion of the electric utility industry during the relevant time period, or

any of the practices, methods, and acts which, in the exercise of reasonable judgment in light of the facts known at the time the decision was made, could have been expected to accomplish the desired result at a reasonable cost consistent with good business practices, reliability, safety and expedition.  Accepted Electric Industry Practice is not limited to a single, optimum practice method or act to the exclusion of others, but rather is intended to include practices, methods, or acts generally accepted in the region.

“Agreement” is the Restated NEPOOL Agreement and attachments, including the NEPOOL Tariff, as amended and restated from time to time.

“Auction Revenue Right (ARR)“ is a right to receive FTR Auction Revenues in accordance with Appendix C of this Market Rule.

“Auction Revenue Right Allocation (ARR Allocation)” is defined in Section 1 of Appendix C of this Market Rule.

“Auction Revenue Right Holder (ARR Holder)“ is an entity which is the record holder of an Auction Revenue Right in the register maintained by the ISO.

“Cancellation Fee” is defined in Section (d).

“Capability Period” shall mean a period of time defined by the ISO for the purposes of rating and auditing Installed Capacity Resources.  There are two Capability Periods, a Summer Capability Period and a Winter Capability Period.  The dates defining the start and end of these periods are set forth in the NEPOOL Manuals.

“Capability Year” shall mean a year’s period beginning on June 1 and ending May 31, for which the ISO will assign, on a monthly basis, each Participant an Unforced Capacity Requirement.

“CARL Data” is Control Area reliability data submitted to the ISO to permit an assessment of the ability of an external Control Area to provide energy to the NEPOOL Control Area in support of Unforced Capacity offered to the NEPOOL Control Area by that external Control Area.

“Commission” is the Federal Energy Regulatory Commission.

“Congestion Component” is the component of the nodal price that reflects the marginal cost of congestion at a given Node or External Node relative to the reference point.  When used in connection with Zonal Price and Hub Price, the term Congestion Component refers to the Congestion Components of the nodal prices that comprise the Zonal Price and Hub Price weighted and averaged in the same way that nodal prices are weighted to determine Zonal Price and averaged to determine the Hub Price.

“Congestion Cost” on and after the SMD Effective Date, is the cost of congestion as measured by the difference between the Congestion Components of the Locational Marginal Prices at different Locations and/or Reliability Regions on the NEPOOL Transmission System.

“Congestion Paying LSE” is, for the purpose of the allocation of FTR Auction Revenues to ARR Holders as provided for in Appendix C of this Market Rule, a Participant or Non-Participant that is responsible for paying for Congestion Costs as a Transmission Customer paying for Regional Network Service or Long-Term Firm Point-to-Point Transmission Service under the NEPOOL Tariff, unless such Transmission Customer has transferred its obligation to supply load in accordance with NEPOOL System Rules, in which case the Congestion Paying LSE shall be the Participant supplying the transferred load obligation.  The term Congestion Paying LSE shall be deemed to include, but not be limited to, the seller of internal bilateral transactions that transfer Real-Time Load Obligations under the NEPOOL System Rules.

“Control Area” is an electric power system or combination of electric power systems to which a common automatic generation control scheme is applied in order to: (i) match, at all times, the power output of the generators within the electric power system(s) and capacity and energy purchased from entities outside the electric power system(s), with the load within the electric power system(s); (ii) maintain scheduled interchange with other Control Areas, within the limits of Accepted Electric Industry Practice; (iii) maintain the frequency of the electric power system(s) within reasonable limits in accordance with Accepted Electric Industry Practice and the criteria of the applicable regional reliability council or the NERC; and (iv) provide sufficient generating capacity to maintain operating reserves in accordance with Accepted Electric Industry Practice.

“DCA Peaking Unit” means a generating Resource, located in a DCA, whose capacity factor in calendar year 2002 was ten percent (10%) or less, as determined by the ISO.

“Daily Reliability Must Run Resource“ or “Daily RMR Resource“ is defined in Section 6.1 of this Market Rule.

“Day-Ahead” is the calendar day immediately preceding the Operating Day.

“Day-Ahead Adjusted Load Obligation” is defined in Section 3.2(iii) of this Market Rule.

“Day-Ahead Congestion Revenue” is defined in Section 3.(f) of this Market Rule.

“Day-Ahead Energy Market” shall mean the schedule of commitments for the purchase or sale of energy, payment of Congestion Costs, and payment for losses developed by the ISO as a result of the offers and specifications submitted in accordance with Section 1.10 of this Market Rule.

“Day-Ahead Energy Market Congestion Charge/Credit” is defined in Section 3.(d) of this Market Rule.

“Day-Ahead Energy Market Energy Charge/Credit” is defined in Section 3.(d) of this Market Rule.

“Day-Ahead Energy Market Loss Charge/Credit” is defined in Section 3.(d) of this Market Rule.

“Day-Ahead Generation Obligation” is defined in Section 3.2(ii) of this Market Rule.

“Day-Ahead Load Obligation” is defined in Section 3.2(i) of this Market Rule.

“Day-Ahead Locational Adjusted Net Interchange” is defined in Section 3.2(iv) of this Market Rule.

“Day-Ahead Loss Charges or Credits” is defined in Section 3.(h) of this Market Rule.

“Day-Ahead Loss Revenue” is defined in Section 3.(g) of this Market Rule.

“Day-Ahead Prices” shall mean the Locational Marginal Prices resulting from the Day-Ahead Energy Market.

“Decrement Bid” shall mean a bid to purchase energy at a specified Location in the Day-Ahead Energy Market which is not associated with a physical load.  An accepted Decrement Bid results in scheduled load at the specified Location in the Day-Ahead Energy Market.

“Deficiency Rate” shall mean the rate determined by the ISO that serves as a cap on the Deficiency Auction’s clearing price as defined in Section 8.(d).  The Deficiency Rate is also used to charge those Participants that are deficient in meeting their Unforced Capacity requirement.

“Delivered 10-Minute Off-Line Forward Reserve” shall be calculated pursuant to Section 9.6.6(a) of this Market Rule.

“Delivered 10-Minute On-Line Forward Reserve” shall be calculated pursuant to Section 9.6.6(a) of this Market Rule.

“Delivered 30-Minute Off-Line Forward Reserve” shall be calculated pursuant to Section 9.6.6(b) of this Market Rule.

“Delivered 30-Minute On-Line Forward Reserve” shall be calculated pursuant to Section 9.6.6(b) of this Market Rule.

“Demand Bid” shall mean a request to purchase a fixed amount of energy, at a specified Location, or an amount of energy at a specified price, that is associated with a physical load.  An accepted Demand Bid results in scheduled load at the specified Location in the Day-Ahead Energy Market.

“Demand Resource” shall mean any resource associated with the Load Response Program as defined in Appendix E to this Market Rule.

“Designated Congestion Area“ is defined in Exhibit 2 to Appendix A to this Market Rule.

“Dispatch Rate” shall mean the control signal, expressed in dollars per MWh and/or megawatts, calculated and transmitted to direct the output level of each generating Resource dispatched by the ISO in accordance with the Offer Data.

“Dispatchable Load” is any portion of the load of a Participant that meets the requirements of the NEPOOL Manuals to have its energy consumption modified in Real-Time because of its ability to respond to remote dispatch instructions from the ISO.  An External Transaction sale shall, if scheduled, be considered a Dispatchable Load for the purposes of the Day-Ahead Energy Market and the Real-Time Energy Market.

“Economic Maximum Limit“ or “Economic Max“ shall be the maximum generation, in MW, of a Participant’s generating unit during non-Emergency Condition.  This represents the highest available output from the unit for economic dispatch and is based on the physical operating characteristics and operating permits of the unit as submitted as part of a Resources Offer Data.

“Economic Minimum Limit“ or “Economic Min” shall be the maximum of the following values:  (i) the Emergency Minimum Limit; (ii) a level supported by environmental and/or operating permit restrictions; or (iii) a level that addresses any significant economic penalties associated with operating at lower levels that can not be adequately represented by three part bidding (Start-Up Fee, No-Load Fee and incremental energy price).  In no event shall the Economic Minimum Limit submitted as part of a generating unit’s Offer Data be higher than the generation level at which a generating unit's incremental heat rate is minimized (i.e., transitioning from decreasing as output increases to increasing as output increases) except that a Self-Scheduled Resource may modify its Economic Minimum Limit on an hourly basis, as part of its Supply Offer, in order to indicate the desired level of Self-Scheduled MWs.

“Effective Offer Price” is defined in Sections 3.2.3 (l), (m), and (n).

“Eligible Customer“ is defined in the NEPOOL Tariff.

“Eligible FTR Bidder” is an entity that has satisfied applicable financial assurance criteria, and shall not include the auctioneer, its affiliates, and their officers, directors, employees, consultants and other representatives.

“Emergency” is an abnormal system condition requiring manual or automatic action to maintain system frequency, or to prevent the involuntary loss of load, equipment damage, or tripping of system elements that could adversely affect the reliability of an electric system or the safety of persons or property; or a fuel shortage requiring departure from normal operating procedures in order to minimize the use of such scarce fuel; or a condition that requires implementation of Emergency procedures as defined in the NEPOOL Manuals.

“Emergency Condition” shall mean an Emergency has been declared by the ISO in accordance with the procedures set forth in the NEPOOL Manuals and ISO Administrative Procedures.

“Emergency Minimum Limit“ or “Emergency Min“ shall mean the minimum generation amount, in MWs, that a generating unit can deliver for a limited period of time without exceeding specified limits of equipment stability and operating permits.

“Energy Component“ shall mean the Locational Marginal Price at the reference point.

“Equivalent Demand Forced Outage Rate” shall mean the portion of time a unit is in demand, but is unavailable due to forced outages.

“Excepted Transaction“ is a transaction specified in Section 25 of the NEPOOL Tariff for the applicable period specified in that Section, or in Sections 25A and 25B of the NEPOOL Tariff.

“External Node” is a proxy bus or buses used for establishing a Locational Marginal Price for energy received by Participants from, or delivered by Participants to, a neighboring Control Area or for establishing Locational Marginal Prices associated with energy delivered through the NEPOOL Control Area by Non-Participants for use in calculating Non-Participant Congestion Costs and loss costs.

“External Resource” shall mean a generation resource located outside the metered boundaries of the NEPOOL Control Area.

“External Transaction” is a purchase by a Participant of energy external to the NEPOOL Control Area or a sale by a Participant of energy external to the NEPOOL Control Area in the Day-Ahead Energy Market and/or Real-Time Energy Market or a through transaction scheduled by a Non-Participant in the Real-Time Energy Market.

“Failure to Activate Reserve” shall be determined in accordance with Section 9.7.2 of this Market Rule.

“Failure to Reserve” is defined in Section 9.7.1 of this Market Rule.

“Failure to Reserve Penalty” is defined in Section 9.7.1 of this Market Rule.

“Fast Start Generator“ shall mean a generating unit that the ISO may dispatch within the hour through electronic dispatch and that meets the following criteria:  (i) minimum run time does not exceed one hour; (ii) minimum down time does not exceed one hour; (iii) time to start does not exceed 30 minutes; (iv) available for dispatch and manned or have automatic remote dispatch capability; (v) capable of receiving and acknowledging a start-up or shut-down dispatch instruction electronically; and (vi) has satisfied its minimum down time.

“Financial Transmission Right” or “FTR” shall mean a financial instrument that evidences the rights and obligations specified in Section 5.2.2 of this Market Rule.

“Formal Warning” is defined in Section 4.1.1 of Appendix B of this Market Rule.

“Formula-Based Sanctions” is defined in Section 4.1.3 of Appendix B of this Market Rule.

“Forward Reserve” shall mean 10-Minute Forward Reserve or 30-Minute Forward Reserve as appropriate.

“Forward Reserve Auction” is the periodic auction of Off-Line and On-Line Forward Reserve conducted by the ISO in accordance with Section 9 of this Market Rule.

“Forward Reserve Auction Offers” are offers to supply Forward Reserve in accordance with Section 9 of this Market Rule.

“Forward Reserve Charges or Credits” shall be determined in accordance with Section 9.8 of this Market Rule.“

“Forward Reserve Delivery Requirement” shall mean a 10-Minute Forward Reserve Delivery Requirement or a 30-Minute Forward Reserve Delivery Requirement as appropriate.

“Forward Reserve Fuel Index” is defined in Section 9.6.2 of this Market Rule.

“Forward Reserve Heat Rate” is defined in Section 9.6.2 of this Market Rule.

“Forward Reserve Payments” shall be determined in accordance with Section 9.8 of this Market Rule.“

“Forward Reserve Resource” refers to either an Off-Line Forward Reserve Resource or an On-Line Forward Reserve Resource.

“Forward Reserve Service Period” is defined in Section 9.5.2 of this Market Rule.

“Forward Reserve Shortfall” is defined in Section 9.7 of this Market Rule.

“Forward Reserve Strike Price” is defined in Section 9.6.2 of this Market Rule.

“FTR Auction” is the periodic auction of FTRs conducted by the ISO in accordance with Section 7 of this Market Rule.

“FTR Auction Revenue“ is the revenue collected from the sale of FTRs in FTR Auctions.  FTR Auction Revenue is payable to FTR Holders who submit their FTRs for sale in the FTR Auction in accordance with Section 7 of this Market Rule and to ARR Holders in accordance with Appendix C of this Market Rule.

“FTR Holder” is an entity that acquires an FTR through the FTR Auction or a subsequent bilateral arrangement pursuant to Section 7 of this Market Rule and registers with the ISO as the holder of the FTR in accordance with Section 7 of this Market Rule and applicable NEPOOL Manuals.

“GADS Data” shall mean data submitted to the NERC for collection into the NERC’s Generating Availability Data System (“GADS”).

“Generator Forced Outage” shall mean an immediate reduction in output or capacity or removal from service, in whole or in part, of a generating unit by reason of an Emergency or threatened Emergency, unanticipated failure, or other cause beyond the control of the owner or operator of the facility, as specified in the relevant portions of the NEPOOL Manuals and ISO Administrative Procedures.  A reduction in output or removal from service of a generating unit in response to changes in market conditions that is approved by the ISO shall not constitute a Generator Forced Outage.  See Section 8.(c) for additional exceptions.

“Generator Maintenance Outage” shall mean the scheduled removal from service, in whole or in part, of a generating unit in order to perform necessary repairs on specific components of the facility, if removal of the facility meets the guidelines specified in the NEPOOL Manuals and ISO Administrative Procedures.

“Generator Planned Outage” shall mean the scheduled removal from service, in whole or in part, of a generating unit for inspection, maintenance or repair with the approval of the ISO in accordance with the NEPOOL Manuals and ISO Administrative Procedures.

“Hub” is a specific set of pre-defined Nodes for which a Locational Marginal Price will be calculated for the Day-Ahead Energy Market and Real-Time Energy Market and which can be used to establish a reference price for energy purchases and the transfer of Day-Ahead Adjusted Load Obligations and Real-Time Adjusted Load Obligations and for the designation of FTRs.

“Hub Price” is calculated in accordance with Section 2.8 of this Market Rule.

“Hydro Quebec Interconnection“ is the United States segment of the transmission interconnection which connects the systems of Hydro-Quebec and the Participants.  “Phase I” is the United States portion of the 450 kV HVDC transmission line from a terminal at the Des Cantons Substation on the Hydro-Quebec system near Sherbrooke, Quebec to a terminal having an approximate rating of 690 MW at a substation at the Comerford Generating Station on the Connecticut River.  “Phase II” is the United States portion of the facilities required to increase to approximately 2000 MW the transfer capacity of the HQ Interconnection, including an extension of the HVDC transmission line from the terminus of Phase I at the Comerford Station through New Hampshire to a terminal at the Sandy Pond Substation in Massachusetts.  The Hydro Quebec Interconnection does not include any Pool Transmission Facilities installed or modified to effect reinforcements of the New England AC transmission system required in connection with the HVDC transmission line and terminals.

“Hydro Quebec Interconnection Capability Credits“ are credits that are granted to a Participant or group of Participants in accordance with the NEPOOL System Rules, which may be used to satisfy the Participant's Unforced Capacity requirement as determined in accordance with Section 8.2 of this Market Rule 1, where the value of such credits is determined in accordance with the NEPOOL Manuals.

“ICAP Effective Date“ shall be the date fixed by the ISO that is (i) the first day of a calendar month; (ii) at least one full calendar month after the SMD Effective Date; and (iii) at least thirty (30) days after the ISO has provided notice to the Commission that NEPOOL System Rules and computer programs necessary to implement the Installed Capacity Market are fully in place and functional.

“ICAP Resource” or “Installed Capacity Resource” shall mean a generating unit, a Dispatchable Load, an External Resource, an External Transaction or Demand Resource that meets the qualification requirements of this Market Rule and has been designated as an ICAP Resource by a Participant, in accordance with the NEPOOL Manuals.

“Inadvertent Energy Revenue“ is defined in Section 3.(k) of this Market Rule.

“Inadvertent Energy Revenue Charges or Credits“ is defined in Section 3.(l) of this Market Rule.

“Inadvertent Interchange“ shall mean the difference between net actual energy flow and net scheduled energy flow into or out of the Control Area.

“Increment Offer” shall mean an offer to sell energy at a specified Location in the Day-Ahead Energy Market.  An accepted Increment Offer results in scheduled generation at the specified Location in the Day-Ahead Energy Market.

“Independent Market Advisor“ shall mean the independent advisor retained by the ISO Board for market monitoring and mitigation matters.

“Installed Capacity” shall mean a MW capability from a qualifying ICAP Resource that meets the requirements set forth in this Market Rule and the NEPOOL Manuals.

“Installed Capacity Equivalent” shall mean the Resource capability that corresponds to Unforced Capacity, calculated in accordance with ISO Procedures.

“Installed Capacity Requirement” shall mean the level of capacity required to meet the reliability requirements defined for the NEPOOL Control Area and calculated in accordance with Section 8 of this Market Rule.  The Installed Capacity Requirement is used to determine an Unforced Capacity Requirement for NEPOOL and individual Participants.

“Intermittent Power Resource” shall mean Resources whose output amount and availability are not subject to the control of the ISO or the plant operator because of the source of fuel (e.g., wind, solar), or contractual obligations (e.g. qualifying facilities) or Resources less than 5 MWs operating within the distribution system.

“ISO” shall mean ISO New England Inc.

“ISO Administrative Procedures” shall mean procedures adopted by the ISO to fulfill its responsibilities to apply and implement NEPOOL System Rules.

“ISO Tariff” is the ISO New England Inc. Tariff for Transmission Dispatch and Power Administration Services.

“Limited Energy Resource” shall mean resources that, due to design considerations, environmental restriction on operations, cyclical requirements, such as the need to recharge or refill or manage water flow, or fuel limitations, are unable to operate continuously at full output on a daily basis.

“Load Asset” shall mean a physical load that has been registered in accordance with the Asset Registration Process.

“Load Response Program” shall mean the program implemented and administered by the ISO to promote demand side response as described in Appendix E to this Market Rule.

“Load-shifting” shall mean the movement of load between Participants, where one Participant’s Real-Time Load Obligation decreases as load leaves to obtain service from another Participant whose Real-Time Load Obligation increases.

“Load Zone” is a Reliability Region, except as otherwise provided for in Section 2.7 of this Market Rule.

“Location” is a Node, External Node, Load Zone or Hub.

“Locational Marginal Price” or “LMP” as defined in Section 2 of this Market Rule.  The Locational Marginal Price for a Node is the nodal price at that Node; the Locational Marginal Price for an External Node is the nodal price at that External Node; the Locational Marginal Price for a Load Zone or Reliability Region is the Zonal Price for that Load Zone or Reliability Region, respectively; and the Locational Marginal Price for a Hub is the Hub Price for that Hub.

“Loss Component“ is the component of the nodal LMP at a given Node or External Node on the PTF that reflects the cost of losses at that Node or External Node relative to the reference point.  The Loss Component of the nodal LMP at a given Node on the non-PTF system reflects the relative cost of losses at that Node adjusted as required to account for losses on the non-PTF system already accounted for through tariffs associated with the non-PTF.  When used in connection with Hub Price or Zonal Price, the term Loss Component refers to the Loss Components of the nodal LMPs that comprise the Hub Price or Zonal Price, which Loss Components are averaged or weighted in the same way that nodal LMPs are averaged to determine Hub Price or weighted to determine Zonal Price.

“Market Monitoring Unit“ shall mean the staff of the ISO designated to implement Mitigation Measures.

“Minimum Generation Emergency“ shall mean an Emergency declared by the ISO in which the ISO anticipates requesting one or more generating Resources to operate at or below Economic Minimum Limit, in order to manage, alleviate, or end the Emergency.

“Mitigation Measures“ is defined in Section 1.1 of Appendix A of this Market Rule.

“MW“ is megawatt.

“MWh“ is megawatt-hour.

“NEMA“ is The Northeast Massachusetts Reliability Region.

“NEMA Contract“ is a contract described in Appendix C of this Market Rule and listed in Exhibit 1 of Appendix C of this Market Rule.

“NEMA Load Serving Entity“ or “NEMA LSE“ is a Transmission Customer or Congestion Paying LSE Entity that serves load within NEMA.

“NEPOOL“ is the New England Power Pool, the power pool created under and governed by the Agreement, and the entities collectively participating in the New England Power Pool as Participants.

“NEPOOL Control Area“ is the integrated electric power system to which a common automatic generation control scheme and various operating procedures are applied by or under the supervision of the ISO in order to:  (i) match, at all times, the power output of the generators within the electric power system and capacity and energy purchased from entities outside the electric power system, with the load within the electric power system; (ii) maintain scheduled interchange with other interconnected systems, within the limits of Accepted Electric Industry Practice; (iii) maintain the frequency of the electric power system within reasonable limits in accordance with Accepted Electric Industry Practice and the criteria of the NPCC and NERC; and (iv) provide sufficient generating capacity to maintain operating reserves in accordance with Accepted Electric Industry Practice.

“NEPOOL Filed Documents” shall mean the NEPOOL Tariff, the Agreement, and this Market Rule or other documents that affect the rates, terms and conditions of service.

“NEPOOL Manuals” shall mean the NEPOOL operating procedures, the manuals implementing this Market Rule and other materials that are approved by NEPOOL pursuant to the Agreement; provided, however, that if Manuals relating to  Market Operations, M-11, Market Rule 1 Accounting, M-28, Financial Transmission Rights, M-06, Billing, M-29, Definitions, M-35 and Tariff Accounting, M-27 have not been approved by NEPOOL prior to the SMD Effective Date or if Manual M-20, relating to the Installed Capacity Market, has not been approved prior to the ICAP Effective

Date, then the draft manuals posted on the ISO web site as of such dates shall remain in effect until each such manual has been approved or modified.  Any further changes to such manuals made by the ISO shall only be pursuant to that certain Interim Independent System Operator Agreement dated July 1, 1997, by and among the Participants and the ISO, as amended, and any agreement that supercedes such Interim ISO Agreement.  Any elements of the NEPOOL Manuals that substantially affect rates, terms, and/or conditions of service shall be filed with the Commission under Section 205 of the Federal Power Act.

“NEPOOL Market” is the market for electric energy, capacity and certain ancillary services within the NEPOOL Control Area.

“NEPOOL Supply Margin” is defined in Section 5.2.2 of Appendix A of this Market Rule.

“NEPOOL System Rules” are this Market Rule, the NEPOOL Information Policy, the ISO Administrative Procedures, the Reliability Standards, the NEPOOL Manuals and any other system rules, procedures or criteria for the operation of the NEPOOL System and administration of the NEPOOL Market, the NEPOOL Agreement and the NEPOOL Tariff.

“NEPOOL Tariff” is the NEPOOL Open Access Transmission Tariff.

“NEPOOL Transmission System” is the system of transmission facilities within the NEPOOL Control Area under the ISO’s operational jurisdiction.

“NERC” is the North American Electric Reliability Council.

“No-Load Fee” is the amount, in dollars per hour, for a generating unit that must be paid to Participants with an Ownership Share in the unit for being scheduled in the NEPOOL Market, in addition to the Start-Up Fee and price offered to supply energy, for each hour that the generating unit is scheduled in the NEPOOL Market.

“Node” is a point on the NEPOOL Transmission System at which LMPs are calculated.

“Non-Participant” is any entity which is not a Participant but may be a Transmission Customer.

“NPCC” is the Northeast Power Coordinating Council.

“Obligation Month” shall mean a time period of one calendar month for which Participants are required to satisfy their Unforced Capacity Requirements.

“Offer Data” shall mean the scheduling, operations planning, dispatch, new Resource, and other data, including generating unit operating limits based on physical unit characteristics, and information necessary to schedule and dispatch generating Resources for the provision of energy and other services and the maintenance of the reliability and security of the transmission system in the NEPOOL Control Area, and specified for submission to the NEPOOL Market for such purposes by the ISO.

“Off-Line Forward Reserve Resource” refers to a generating Resource that has been awarded a Forward Reserve Delivery Requirement through the Forward Reserve Auction, and designated itself as an Off-Line Forward Reserve Resource in its Supply Offer.  It shall fulfill the obligations of delivery prescribed in Section 9 for either 10 Minute or 30 Minute Off-Line Forward Reserve.

“Off-Line Qualifying Megawatts” refer to the capability offered at cost above the Forward Reserve Strike Price in accordance with Section 9.6.4 of this rule.

“On-Line Forward Reserve Resource” refers to a generating Resource that has been awarded a Forward Reserve Delivery Requirement through the Forward Reserve Auction, and has designated itself as an On-Line Forward Reserve Resource in its Supply Offer.   It shall fulfill the obligations of delivery prescribed in Section 9 for either 10 Minute or 30 Minute On-Line Forward Reserve.

“On-Line Qualifying Megawatts” refer to the capability offered at cost above the Forward Reserve Strike Price in accordance with Section 9.6.5 of this rule.

“Operating Data” shall mean, pursuant to Section 8.3.5 of this Market Rule,  GADS Data, data equivalent to GADS Data, CARL Data, metered Load data, or actual system failure occurrences data, all as described in the ISO Procedures.

“Operating Day” shall mean the calendar day period beginning at midnight for which transactions on the NEPOOL Market are scheduled.

“Operating Reserve” shall mean ten-minute and thirty-minute reserve as defined in the NEPOOL Manuals.

“Operating Reserve Credit” means the payment made to a Resource as provided in Section 3.2.3, Section 6.4 and Appendix F.

“Operating Reserve Charge” means the charges to Participants as provided in Section 3.2.3, Section 6.4 and Appendix F.

“Opportunity Cost” shall have the meaning specified in Sections 3.(d) and 3.(e) of this Market Rule.

“Ownership Share” is a right or obligation, for purposes of settlement, to a percentage share of all credits or charges associated with a generating unit asset or Load Asset, where such unit or load is interconnected to the NEPOOL Transmission System.

“Participant” is an eligible Entity (or group of Entities which has elected to be treated as a single Participant pursuant to Section 4.1 of the Agreement) which is a signatory to the Agreement and has become a Participant in accordance with Section 3.1 of the Agreement until such time as such Entity’s status as a Participant terminates pursuant to Section 21.2 of the Agreement.

“Participant Obligations” is defined in Section 1.1 of Appendix B of this Market Rule.

“Pivotal Supplier“ is defined in Section 5.2.2 of Appendix A of this Market Rule.

“Pool RNS Rate“ is the transmission rate determined in accordance with paragraph (2) of Schedule 9 to the NEPOOL Tariff.

“Pool-Scheduled Resources” has the meaning specified in Section 1.10.2 of this Market Rule.

“Pool Transmission Facilities“ or “PTF“ are the pool transmission facilities defined in Section 15.1 of the Agreement, and any other new transmission facilities which the Reliability Committee determines, in accordance with criteria approved by the Participants Committee and subject to review by the ISO, should be included in PTF.

“PUSH Reference Level” means the Reference Level for a DCA Peaking Unit, as calculated by the ISO pursuant to Exhibit 2 to Appendix A to this Market Rule.

“Qualified Upgrade Awards“ are revenues associated with the additional FTRs made possible in an FTR Auction by transmission upgrades, which increase transfer capability on the NEPOOL Transmission System, where such transmission upgrades are initially placed in-service on or after March 1, 1997 and paid for by an entity and are not paid for through the Pool RNS Rate.

“Real-Time” is a period in the current Operating Day for which the ISO dispatches Resources for energy and Regulation, designates Resources for Regulation and, if necessary, commits additional Resources.

“Real-Time Adjusted Load Obligation” is defined in Section 3.2(iii) of this Market Rule.

“Real-Time Commitment Periods” are periods of continuous operation bounded by a start-up and the earlier to occur of a shut-down or a unit trip used to determine eligibility for Real-Time Operating Reserve Credit.

“Real-Time Congestion Revenue” is defined in Section 3.(f) of this Market Rule.

“Real-Time Energy Market” shall mean the purchase or sale of energy, payment of Congestion Costs, and payment for losses for quantity deviations from the Day-Ahead Energy Market in the Operating Day.

“Real-Time Energy Market Deviation Congestion Charge/Credit” is defined in Section 3.(e) of this Market Rule.

“Real-Time Energy Market Deviation Energy Charge/Credit” is defined in Section 3.(e) of this Market Rule.

“Real-Time Energy Market Deviation Loss Charge/Credit” is defined in Section 3.(e) of this Market Rule.

“Real-Time Generation Obligation” is defined in Section 3.2(ii) of this Market Rule.

“Real-Time Generation Obligation Deviation” is defined in Section 3.2(ii) of this Market Rule.

“Real-Time Load Obligation” is defined in Section 3.2(i) of this Market Rule.

“Real-Time Load Obligation Deviation” is defined in Section 3.2(i) of this Market Rule.

“Real-Time Locational Adjusted Net Interchange” is defined in Section 3.2(iv) of this Market Rule.

“Real-Time Locational Adjusted Net Interchange Deviation” is defined in Section 3.2(iv) of this Market Rule.

“Real-Time Loss Revenue” is defined in Section 3.(i) of this Market Rule.

“Real-Time Loss Revenue Charges or Credits” is defined in Section 3.(m) of this Market Rule.

“Real-Time Prices” shall mean the Locational Marginal Prices resulting from the ISO’s dispatch of the NEPOOL Market in the Operating Day.

“Reference Level” is defined in Section 5.6.1 of Appendix A of this Market Rule.

“Regulation” shall mean the capability of a specific generating unit with appropriate telecommunications, control and response capability to increase or decrease its output in response to a regulating control signal, in accordance with the specifications in the NEPOOL Manuals and ISO Administrative Procedures.

“Regulation Clearing Price” shall have the meaning specified in Section 3.(c) of this Market Rule.

“Reliability Region” shall mean any one of the regions identified on the ISO’s website.  Reliability Regions are intended to reflect the operating characteristics of, and the major transmission constraints on, the NEPOOL Transmission System.

“Reserve Shortage Condition“ exists when one of the following conditions occurs:

#### The NEPOOL Control Area is experiencing a deficiency in total ten-minute Operating Reserves; or

#### The NEPOOL Control Area is experiencing a deficiency in total Operating Reserves that has lasted for longer than four hours.

“Reserve Shortage Condition Energy Price“ shall be $1,000/MWh.

“Reserve Shortage Condition Pricing Event“ shall be declared when a Reserve Shortage Condition exists, the ISO has begun taking actions to maintain or restore Operating Reserves, and the ISO has indicated that the LMP pricing methodology as described in Section 2.5(d) of this Market Rule is to be in effect.

- “Reserve Shortage Opportunity Cost“ The Reserve Shortage Opportunity Cost to be paid to eligible generating Resources for any hour during the Operating Day for which the ISO has declared a Reserve Shortage Condition Pricing Event shall be determined by the ISO as provided for in Market Rule 1 and NEPOOL Manual 11 on the basis of the expected value of energy sales foregone by the generating Resource because it was supplying Operating Reserves.

“Resource” means a generating unit, a Dispatchable Load, an External Resource or an External Transaction.

“Replacement Reserve” shall mean reserve other than ten-minute reserve or thirty minute reserve as defined in the NEPOOL Manuals.

“Reviewable Action” is defined in Section 1.1 of Appendix D of this Market Rule.

“Sanctionable Behavior” is defined in Section 3 of Appendix B of this Market Rule.

“Self-Schedule” is the action of a Participant in committing and/or scheduling its Resource, in accordance with applicable NEPOOL Manuals, to provide service in an hour, whether or not in the absence of that action the Resource would have been scheduled or dispatched by the ISO to provide the service.

“Self-Scheduled MW” is an amount, in megawatts, that is Self-Scheduled and is equal to the greater of: (i) the Resource’s Economic Minimum Limit; or (ii) for a Resource for which the Regulation Self-Schedule flag is set for the hour and the unit was on Regulation for at least 20 minutes during the applicable hour of the Operating Day, the median value of all Regulation setpoints (Desired Dispatch Point) used by the Resource while regulating.

“SMD Effective Date“ shall be the date fixed by the ISO and posted on its website as the effective date for this Market Rule.  The SMD Effective Date may not occur until (i) a date after a Commission order is issued permitting this Market Rule and any other documents that have been filed that substantially affect rates, terms, and conditions to go into effect; (ii) at least two weeks after the ISO has given the Commission written notice that the NEPOOL System Rules and computer programs necessary to implement this Market Rule are fully in place and functional; and (iii) at least 48 hours after the ISO has posted the date on its website.

“SPD” means the ISO’s Scheduling, Pricing, and Dispatch software, as more fully described in Section 5.5.3 of Appendix A of this Market Rule.

“Special Case Resources” shall mean Limited Energy Resources, certain non-dispatchable qualifying facilities as defined in NEPOOL Manual M-20, certain generating units having a capacity of less than 5 MWs as defined in NEPOOL Manual M-20 and Intermittent Power Resources treated in accordance with Section 8.3.11 of this Market Rule.

“Special Constraint Resources” are Resources that provide Special Constraint Resource Service under Schedule 19 of the NEPOOL Tariff.

“Spinning Reserve” shall mean the reserve capability that can be converted fully into energy within ten minutes from the request of the ISO dispatcher, and is provided by equipment electrically synchronized to the NEPOOL Transmission System.

“Start-Up Fee” is the amount, in dollars, that must be paid for a generating unit to Participants with an Ownership Share in the unit each time the unit is scheduled in the NEPOOL Market to start-up.

“State Estimator” shall mean the computer model of power flows specified in Section 2.3 of this Market Rule.

“Stipulated ICAP Revenue” is defined in Section 3.3.2 of Exhibit 2 to Appendix A of this Market Rule.

“Submitted Offer” is defined in Section 5.6.1 of Appendix A of this Market Rule.

“Summer Capability Period“ shall mean one of two time periods defined by the ISO for the purposes of rating and auditing ICAP Resources.  The time period associated with the Summer Capability Period is defined in the NEPOOL Manuals.

“Supply Offer” is a proposal to furnish energy at a Node or Regulation from a Resource that meets the applicable requirements set forth in the NEPOOL Manuals submitted to the ISO by a Participant with authority to submit a Supply Offer for the Resource.  The Supply Offer will be submitted pursuant to this Market Rule and applicable NEPOOL Manuals, and include a price and information with respect to the quantity proposed to be furnished, technical parameters for the Resource, timing and other matters.  A Supply Offer is a subset of the information required in a Participant’s Offer Data.

“10-Minute Forward Reserve” is capability that satisfies the requirements for Delivered 10-minute Off-Line and On-Line Forward Reserve as calculated in accordance with Section 9.6.6 of this Market Rule.

“10-Minute Forward Reserve Clearing Price” is the clearing price for 10-Minute Forward Reserve calculated in accordance with Section 9 of this Market Rule.

“10-Minute Forward Reserve Delivery Requirement” is the amount of a Participant’s 10-Minute Forward Reserve supply offer that has been selected in the Forward Reserve Auction.

“10-Minute Hourly Forward Reserve Clearing Price” shall be the 10-Minute Forward Reserve Clearing Price divided by the number of delivery hours in the month.

“30-Minute Forward Reserve” is capability that satisfies the requirements for Delivered 30-minute Off-Line and On-Line Forward Reserve as calculated in accordance with Section 9.6.6 of this Market Rule

“30-Minute Forward Reserve Clearing Price” is the clearing price for 30-Minute Forward Reserve calculated in accordance with Section 9 of this Market Rule.

“30-Minute Forward Reserve Delivery Requirement” is the amount of a Participant’s 30-Minute Forward Reserve supply offer that has been selected in the Forward Reserve Auction.

“30-Minute Hourly Forward Reserve Clearing Price” shall be the 30-Minute Forward Reserve Clearing Price divided by the number of delivery hours in the month.

“Transmission Congestion Credit” shall mean the allocated share of total Transmission Congestion Revenue credited to each holder of Financial Transmission Rights, calculated and allocated as specified in Section 5.2 of this Market Rule.

“Transmission Congestion Revenue” is defined in Section 5.(a) of this Market Rule.

“Transmission Customer” is any Eligible Customer that (i) is a Participant which is not required to sign a Service Agreement with respect to a service to be furnished to it in accordance with Section 48 of the NEPOOL Tariff or (ii) executes, on its own behalf or through its Designated Agent, a Service Agreement, or (iii) requests in writing, on its own behalf or through its Designated Agent, that NEPOOL file with the Commission a proposed unexecuted Service Agreement in order that the Eligible Customer may receive transmission service under the NEPOOL Tariff.

“Transmission Forced Outage” shall mean an immediate removal from service of a transmission facility by reason of an Emergency or threatened Emergency, unanticipated failure, or other cause beyond the control of the owner or operator of the transmission facility, as specified in the relevant portions of the NEPOOL Manuals and ISO Administrative Procedures.  A removal from service of a transmission facility at the request of the ISO to improve transmission capability shall not constitute a Transmission Forced Outage.

“Transmission Planned Outage” shall mean any transmission outage scheduled in advance for a pre-determined duration and which meets the notification requirements for such outages specified in the NEPOOL Manuals and ISO Administrative Procedures.

“Transmission Provider“ is defined in the NEPOOL Tariff.

“UCAP Deficiency Auction” shall mean an auction conducted pursuant to Section 8.(a) of this Market Rule and the NEPOOL Manuals to procure sufficient Unforced Capacity to cover the remainder of Participant’s Unforced Capacity requirements for an Obligation Month.

“UCAP Monthly Auction” shall mean an auction administered by the ISO according to Section 8.4 of this Market Rule and the NEPOOL Manuals for Participants serving load to procure Unforced Capacity and for qualified Resources to supply Unforced Capacity.

“UCS” is Unit Commitment Software as more fully defined in Section 5.5.3 of Appendix A of this Market Rule.

“UDS” is Unit Dispatch System Software, as more fully defined in Section 5.5.3 of Appendix A of this Market Rule.

“Unforced Capacity“ or “UCAP“ shall mean the measure by which: 1) Installed Capacity suppliers will be rated, in accordance with the formulae set forth in the NEPOOL Manuals, to quantify the extent of their contribution to satisfy the NEPOOL Installed Capacity Requirement, and 2) the measure to determine if a Participant has met its procurement obligations relating to the Installed Capacity Requirement.

“Unforced Capacity Requirement“ shall mean the amount of Unforced Capacity determined by the ISO in megawatts for the Summer Capability Period and Winter Capability Period for NEPOOL as a whole as calculated in accordance with the NEPOOL Manuals.

“Winter Capability Period” shall mean one of two time periods defined by the ISO for the purposes of rating and auditing ICAP Resources.  The time period associated with the Winter Capability Period is defined in the NEPOOL Manuals.

“Zonal Price” is calculated in accordance with Section 2.7 of this Market Rule.

### [Reserved]

## [Reserved.]

## [Reserved.]

## [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### NEPOOL Manuals and ISO Administrative Procedures.

The ISO shall prepare, maintain and update the NEPOOL Manuals and ISO Administrative Procedures consistent with the NEPOOL Filed Documents.  The NEPOOL Manuals and ISO Administrative Procedures shall be available for inspection by the Participants, regulatory authorities with jurisdiction over the ISO or any Participant, and the public.

## General.

### [Reserved.]

### [Reserved.]

### Agents.

A Participant may participate in the NEPOOL Market through an agent, provided that such Participant informs the ISO in advance in writing of the appointment of such agent.  A Participant using an agent shall be bound by all of the acts or representations of such agent with respect to transactions in the NEPOOL Market, and shall ensure that any such agent complies with the requirements of the NEPOOL Manuals and ISO Administrative Procedures and the NEPOOL Filed Documents.

### [Reserved.]

### [Reserved.]

### Scheduling and Dispatching.

#### The ISO shall schedule Day-Ahead and schedule and dispatch in Real-Time Resources economically on the basis of least-cost, security-constrained dispatch and the prices and operating characteristics offered by Participants.  The ISO shall schedule and dispatch sufficient Resources of the Participants to serve the NEPOOL Market energy purchase requirements under normal system conditions of the Participants and meet the requirements of the NEPOOL Control Area for ancillary services provided by such Resources.  Scheduling and dispatch shall be conducted in accordance with the NEPOOL Filed Documents.

#### The ISO shall undertake, together with NEPOOL, to identify any conflict or incompatibility between the scheduling or other deadlines or specifications applicable to the NEPOOL Market, and any relevant procedures of another Control Area, or any tariff (including the NEPOOL Tariff).  Upon determining that any such conflict or incompatibility exists, the ISO shall propose tariff or procedural changes, or undertake such other efforts as may be appropriate, to resolve any such conflict or incompatibility.

### Energy Pricing.

The price paid for energy bought and sold in the NEPOOL Market will reflect the hourly Locational Marginal Price at each Location, determined by the ISO in accordance with the NEPOOL Filed Documents.  Congestion Costs, which shall be determined by differences in the Congestion Component of Locational Marginal Prices in an hour caused by transmission constraints, shall be calculated and collected, and the resulting revenues disbursed, by the ISO in accordance with this Market Rule.  Loss costs associated with Pool Transmission Facilities, which shall be determined by the differences in Loss Components of the Locational Marginal Prices in an hour, shall be calculated and collected, and the resulting revenues disbursed, by the ISO in accordance with this Market Rule.

### Participant Resources.

A Participant may elect to Self-Schedule its Resources in accordance with and subject to the procedures specified in this Market Rule and the NEPOOL Manuals.

### [Reserved.]

### Other Transactions.

#### Participants may enter into internal bilateral transactions and External Transactions for the purchase or sale of energy or other products to or from each other or any other entity, subject to the obligations of Participants to make ICAP Resources available for dispatch by the ISO.  External Transactions that contemplate the physical transfer of energy or obligations to or from a Participant shall be reported to and coordinated with the ISO in accordance with this Market Rule and the NEPOOL Manuals.

#### [Reserved.]

#### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### Operating Reserve.

The ISO shall schedule to the Operating Reserve and load-following objectives of the NEPOOL Control Area and the NEPOOL Market in scheduling Resources pursuant to this Market Rule.  Operating Reserve objectives are determined in accordance with NEPOOL Manuals.

### Regulation.

#### Regulation shall be supplied from generators located within the metered electrical boundaries of the NEPOOL Control Area.  Participants offering Regulation shall comply with applicable standards and requirements for Regulation capability and dispatch specified in the NEPOOL Manuals and ISO Administrative Procedures.

#### The ISO shall obtain and maintain an amount of Regulation equal to the NEPOOL Control Area Regulation objective as specified in the NEPOOL Manuals and ISO Administrative Procedures.

#### The Regulation range of a unit shall be at least twice the amount of Regulation assigned and no less than the minimum specified in the NEPOOL Manuals and ISO Administrative Procedures.

#### A unit that is providing Regulation shall have its energy dispatch range reduced by twice the amount of the Regulation provided.  The amount of Regulation provided by a unit shall serve to redefine the Economic Minimum Limit and Economic Maximum Limit of that unit, in that the amount of Regulation shall be added to the unit's Economic Minimum Limit or automatic low limit while regulating, whichever is greater, and subtracted from its Economic Maximum Limit or automatic high limit, which ever is less.  Qualified Regulation must satisfy the verification tests described in the NEPOOL Manuals and ISO Administrative Procedures.

### Ramping.

A generating unit dispatched by the ISO pursuant to a control signal appropriate to increase or decrease the unit’s megawatt output level shall be able to change output at the ramping rate specified in the Offer Data submitted to the ISO for that unit and shall be subject to sanctions for failure to comply as described in Appendix B.

1.7.19A  [Reserved - Spinning Reserve.]

### Information and Operating Requirements.

#### [Reserved.]

#### Participants selling from Resources within the NEPOOL Control Area shall:  report to the ISO sources of energy available for operation; supply to the ISO all applicable Offer Data; report to the ISO units that are Self-Scheduled; report to the ISO External Transaction sales; confirm to the ISO bilateral sales to Participants within the NEPOOL Control Area; respond to the ISO’s directives to start, shutdown or change output levels of generating units, or change scheduled voltages or reactive output levels; continuously maintain all Offer Data concurrent with on-line operating information; and ensure that, where so equipped, generating equipment is operated with control equipment functioning as specified in the NEPOOL Manuals and ISO Administrative Procedures.

#### Participants selling from Resources outside the NEPOOL Control Area shall:  provide to the ISO all applicable Offer Data, including offers specifying amounts of energy available, hours of availability and prices of energy and other services; respond to ISO directives to schedule delivery or change delivery schedules; and communicate delivery schedules to the source Control Area and any intermediary Control Areas.

#### Participants, as applicable, shall:  respond or ensure a response to ISO directives for load management steps; report to the ISO the ICAP Resources they have provided to satisfy their capacity obligations that are available for pool operation; report to the ISO all bilateral purchase transactions including External Transaction purchases; and respond or ensure a response to other ISO directives such as those required during Emergency operation.

#### Participant, as applicable, shall provide to the ISO requests to purchase specified amounts of energy for each hour of the Operating Day during which it intends to purchase from the Day-Ahead Energy Market, along with Dispatch Rate levels above which it does not desire to purchase.

## [Reserved.]

## Pre-scheduling.

### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### [Reserved.]

### Participant Responsibilities.

Participants authorized and intending to request market-based Start-Up and No-Load Fees in their Offer Data shall submit a specification of such fees to the ISO for each generating unit as to which the Participant intends to request such fees.  Any such specification shall identify the applicable period and be submitted on or before the applicable deadline specified in the NEPOOL Manuals and ISO Administrative Procedures and shall remain in effect without change throughout each such period for which a specification was submitted.  The ISO shall reject any request for Start-Up and No-Load Fees in a Participant’s Offer Data that does not conform to the Participant’s specification on file with the ISO.

### [Reserved.]

## Scheduling.

### General.

#### The ISO shall administer scheduling processes to implement a Day-Ahead Energy Market and a Real-Time Energy Market.

#### The Day-Ahead Energy Market shall enable Participants to purchase and sell energy through the NEPOOL Market at Day-Ahead Prices and enable Participants to submit External Transactions conditioned upon Congestion Costs not exceeding a specified level.  Participants whose purchases and sales and External Transactions are scheduled in the Day-Ahead Energy Market shall be obligated to purchase or sell energy or pay Congestion Costs and costs for losses, at the applicable Day-Ahead Prices for the amounts scheduled.

#### In the Real-Time Energy Market,

##### Participants that deviate from the amount of energy purchases or sales scheduled in the Day-Ahead Energy Market shall be obligated to purchase or sell energy in the amount of the deviations at the applicable Real-Time Prices, unless otherwise specified by this Market Rule, and

##### Non-Participant Transmission Customers shall be obligated to pay Congestion Costs and costs for losses for the amount of the scheduled transmission uses in the Real-Time Energy Market at the applicable Real-Time

Congestion Cost Component and Loss Component price differences, unless otherwise specified by this Market Rule.

#### The following scheduling procedures and principles shall govern the commitment of Resources to the Day-Ahead Energy Market and the Real-Time Energy Market over a period extending from one week to one hour prior to the Real-Time dispatch.  Scheduling encompasses the Day-Ahead and hourly scheduling process, through which the ISO determines the Day-Ahead Energy Market schedule and determines, based on changing forecasts of conditions and actions by Participants and system constraints, a plan to serve the hourly energy and reserve requirements of the NEPOOL Control Area in the least costly manner, subject to maintaining the reliability of the NEPOOL Control Area.  If the ISO’s forecast for the next seven days projects a likelihood of Emergency Condition, the ISO may commit, for all or part of such seven day period, to the use of generating Resources with notification time greater than 24 hours as necessary in order to alleviate or mitigate such Emergency, in accordance with the Participants’ binding Supply Offers for such units, as submitted in accordance with Section 1.10.1A(f), for such periods and the specifications in the NEPOOL Manuals and ISO Administrative Procedures and such Resources shall be treated as Pool-Scheduled Resources and shall be eligible to receive Operating Reserve Credits under Section 3.2.3 in accordance with the binding Supply Offers submitted.

1.10.1A	Day-Ahead Energy Market Scheduling	.

The following actions shall occur not later than 12:00 noon on the day before the Operating Day for which transactions are being scheduled, or such other deadline as may be specified by the ISO in order to comply with the practical requirements and the economic and efficiency objectives of the scheduling process specified in this Market Rule.

#### Each Participant may submit to the ISO specifications of the amount and location of its customer loads and/or energy purchases to be included in the Day-Ahead Energy Market for each hour of the next Operating Day, such specifications to comply with the requirements set forth in the NEPOOL Manuals and ISO Administrative Procedures.  Each Participant shall inform the ISO of (i) the prices, if any, at which it desires not to include its load in the Day-Ahead Energy Market rather than pay the Day-Ahead Price, (ii) hourly schedules for Resource increments, including hydropower units, Self-Scheduled by the Participant; and (iii) the Decrement Bid at

which each such Self-Scheduled Resource will disconnect or reduce output, or confirmation of the Participant’s intent not to reduce output.  Price-sensitive Demand Bids and Decrement Bids must be equal to or greater than zero and shall not exceed the energy Supply Offer limitation specified in this Section.

#### [Reserved.]

#### All Participants shall submit to the ISO schedules for any External Transactions involving use of generating Resources or the NEPOOL Transmission System as specified below, and shall inform the ISO whether the transaction is to be included in the Day-Ahead Energy Market.  Any Participant that elects to include an External Transaction in the Day-Ahead Energy Market may specify the price (such price not to exceed the maximum price that may be specified in the NEPOOL Manuals and ISO Administrative Procedures), if any, at which it will be curtailed rather than pay Congestion Costs.  The foregoing price specification shall apply to the price difference between the Locational Marginal Prices for specified External Transaction source and sink points in the Day-Ahead scheduling process only. Any Participant that deviates from its Day-Ahead External Transaction schedule or elects not to include its External Transaction in the Day-Ahead Energy Market shall be subject to Congestion Costs in the Real-Time Energy Market in order to complete any such scheduled External Transaction.  Scheduling of External Transactions shall be conducted in accordance with the specifications in the NEPOOL Manuals and ISO Administrative Procedures and the following requirements:

##### Participants shall submit schedules for all External Transaction purchases for delivery within the NEPOOL Control Area from Resources outside the NEPOOL Control Area;

##### Participants shall submit schedules for External Transaction sales to entities outside the NEPOOL Control Area from Resources within the NEPOOL Control Area;

##### If the sum of all submitted fixed External Transaction purchases less External Transaction sales exceeds the import capability associated with the applicable External Node, the offer prices for all fixed External Transaction purchases at the applicable External Node shall be set equal to $0.0/MWh; and

##### If the sum of all submitted fixed External Transaction sales less External Transaction purchases exceeds the export capability associated

with the applicable External Node, the offer prices for all fixed External Transaction sales at the applicable External Node shall be set equal to $1,000/MWh.

#### Participants wishing to sell into the NEPOOL Market, from either internal Resources or External Resources, shall submit Supply Offers for the supply of energy (including energy from hydropower units), Regulation, Operating Reserve or other services as applicable, for the following Operating Day.  Supply Offers shall be submitted to the ISO in the form specified by the ISO and shall contain the information specified in the ISO’s Offer Data specification, as applicable.  For External Resources that have not implemented dynamic scheduling with the ISO, Participants may only submit Offer Data and Supply Offer parameters, which allow for the External Resource to be block loaded as an External Transaction at the relevant External Node on an hour-to-hour basis.  The ISO shall not consider Start-up Fees, No-Load Fees, notification times or any other inter-temporal parameters in scheduling or dispatching these resources.  Participants owning or controlling the output of an ICAP Resource that has not been rendered unavailable by a Generator Planned Outage, a Generator Maintenance Outage, or a Generator Forced Outage shall submit Supply Offers for the available capacity of such ICAP Resource, including any portion that is Self-Scheduled by the Participant claiming the Resource as an ICAP Resource.  The submission of Supply Offers for Resources that are not ICAP Resources shall be optional, but any such Supply Offers must contain the information specified in the ISO’s Offer Data specification, as applicable.  Energy offered from generating Resources that are not ICAP Resources shall not be supplied from Resources that are included in or otherwise committed to supply the operating reserve requirements of another Control Area.  The foregoing Supply Offers for ICAP Resources, as applicable and non ICAP Resources:

##### Shall specify the generating Resource and energy for each hour in the offer period;

##### Shall specify the amounts and prices for the entire Operating Day for each Resource offered by the Participant to the ISO;

##### If based on energy from a specific generating unit internal to the NEPOOL Control Area, may specify Start-Up and No-Load Fees equal to the specification of such fees for such unit on file with the ISO (Participant changes to the Start-Up Fee and No-Load Fee can only occur during the open periodic bidding enrollment periods (twice a month));

##### Shall set forth any special conditions upon which the Participant proposes to supply a Resource increment;

##### Shall specify a minimum run time to be used for scheduling purposes that does not exceed 24 hours;

##### Shall constitute an offer to submit the Resource increment to the ISO for scheduling and dispatch in accordance with the terms of the Supply Offer, where such Supply Offer, with regard to operating limits, shall specify changes to the Economic Maximum Limit, Economic Minimum Limit and Emergency Minimum Limit from those submitted as part of the Resource’s Offer Data to reflect the physical operating characteristics and/or availability of the Resource, except that, for a Self-Scheduled Resource, the Economic Minimum Limit may be revised to reflect the Self-Scheduled output level of the Resource and for a Limited Energy Resource, the Economic Maximum Limit may be revised to reflect maximum energy available for the Operating Day, which offer shall remain open through the Operating Day for which the Supply Offer is submitted;

##### Shall be final as to the price or prices at which the Participant proposes to supply energy or other services to the NEPOOL Market, such price or prices for Resources or portions of Resources scheduled in the Day-Ahead Energy Market being guaranteed by the Participant for the period extending through the end of the following Operating Day or, in the case of a Pool-Scheduled Resource continuing to run into the second Operating Day to satisfy its minimum run time, such price or prices being guaranteed by the Participant for the period extending into the second Operating Day that satisfies the Resource’s minimum run time; and

##### Shall not specify  an energy offer price below $0/MWh or above $1,000/MWh.

#### A Participant that wishes to make a Resource available to sell Regulation service shall submit a Supply Offer for Regulation that shall specify the megawatts of Regulation being offered, the price of the offer in dollars per MWh, and such other information specified by the ISO as may be necessary to evaluate the Supply Offer and the Resource’s Opportunity Costs.  The price of the Supply Offer shall not exceed $100/MWh.  Qualified Regulation capability must satisfy the verification tests specified in the NEPOOL Manuals and ISO Administrative Procedures. Regulation capability amounts will be adjusted as necessary in the case where a generating unit’s compliance rating is less than 90%.  The audited Regulation capability will be deemed equal to the most recently calculated compliance rating times the 5-minute Regulation capability quantities utilized in that compliance rating calculation, rounded to the nearest whole megawatt.

#### Each Participant owning or controlling the output of an ICAP Resource shall submit a forecast of the availability of each such ICAP Resource for the next seven days.  A Participant (i) may submit a non-binding forecast of the price at which it expects to offer a generating Resource increment to the ISO over the next seven days, and (ii) shall submit a binding Supply Offer for energy, along with Start-Up and No-Load Fees, if any and if applicable, for the next seven days, for any ICAP Resource with a minimum notification time greater than 24 hours.

#### Each Supply Offer by a Participant of a Resource shall remain in effect for subsequent Operating Days until superseded or canceled except in the case of an External Resource and an External Transaction purchase, in which case, the Supply Offer shall remain in effect for the applicable Operating Day and shall not remain in effect for subsequent Operating Days.

#### The ISO shall post on the internet the total hourly loads  including Decrement Bids scheduled in the Day-Ahead Energy Market, as well as the ISO’s estimate of the Control Area hourly load for the next Operating Day.

#### All Participants may submit Increment Offers and/or Decrement Bids that apply to the Day-Ahead Energy Market only.  Such offers and bids must comply with the requirements set forth in the NEPOOL Manuals and ISO Administrative Procedures and must specify amount, location and price, if any, at which the Participant desires to purchase or sell energy in the Day-Ahead Energy Market.

#### [Reserved – Spinning Reserve.]

### Pool-Scheduled Resources.

Pool-Scheduled Resources are those Resources for which Participants submitted Supply Offers to sell energy in the Day-Ahead Energy Market and which the ISO scheduled in the Day-Ahead Energy Market as well as generators committed by the ISO subsequent to the Day-Ahead Energy Market.  Such Resources shall be committed to provide energy in the Real-Time dispatch unless the schedules for such units are revised pursuant to Sections 1.10.9 or 1.11.  Pool-Scheduled Resources shall be governed by the following principles and procedures.

#### Pool-Scheduled Resources shall be selected by the ISO on the basis of the prices offered for energy and related services, Start-Up Fees, No-Load

Fees, and the specified operating characteristics, offered by Participants to the ISO by the offer deadline specified in Section 1.10.1A.

#### The ISO shall optimize the dispatch of energy from Limited Energy Resources by request to minimize the as-bid production cost for the NEPOOL Control Area.  In implementing the use of Limited Energy Resources, the ISO shall use its best efforts to select the most economic hours of operation for Limited Energy Resources, in order to make optimal use of such Resources in the Day-Ahead Energy Market consistent with the Supply Offers of other Resources, the submitted Demand Bids and Decrement Bids and Operating Reserve and Replacement Reserve requirements.

#### Participants offering energy from hydropower or other facilities with fuel or environmental limitations may submit data to the ISO that is sufficient to enable the ISO to determine the available operating hours of such facilities.

#### The Participant seller whose Resource is selected as a Pool-Scheduled Resource shall receive payments or credits for energy or related services, or for Start-Up and No-Load Fees, from the ISO on behalf of the Participant buyers in accordance with Section 3 of this Market Rule.  Additionally, the Participant seller shall receive for Pool-Scheduled Resources scheduled in the Real-Time Energy Market that were not Scheduled in the Day-Ahead Energy Market, a pro-rata share of its applicable Start-Up Fee if the ISO cancels its selection of the Resource as a Pool-Scheduled Resource and so notifies the Participant seller before the Resource is synchronized (“Cancellation Fee”).

#### Participants shall make available their Pool-Scheduled Resources to the ISO for coordinated operation to supply the needs of the NEPOOL Control Area for energy and ancillary services.

#### Eligibility for Operating Reserve Compensation in the Day-Ahead Market under Section 3.2.3 is affected by Resource Self-Schedules.  The specific rules related to the impact of Resource Self-Schedules on eligibility for Operating Reserve compensation may be found in Appendix F of this Market Rule.

#### Eligibility for Operating Reserve Compensation in the Real-Time Market under Section 3.2.3 is affected by Resource Self-Schedules.  The specific rules related to the impact of Resource Self-Schedules on eligibility for Operating Reserve compensation may be found in Appendix F of this Market Rule.

#### Eligibility for Operating Reserve compensation in the Real-Time Market under Section 3.2.3 may be affected by Resource trips.  The specific rules related to the impact of Resource trips on eligibility for Operating Reserve compensation may be found in Appendix F of this Market Rule.

### Self-Scheduled Resources.

Self-Scheduled Resources shall be governed by the following principles and procedures.

#### [Reserved.]

#### The offered prices of Resources or portions of Resource that are Self-Scheduled, or otherwise not following the dispatch orders of the ISO, shall not be considered by the ISO in determining Locational Marginal Prices.

#### Participants shall make available their Self-Scheduled ICAP Resources, including increments available above Self-Scheduled increments, to the ISO for coordinated operation to supply the needs of the NEPOOL Control Area for energy and ancillary services.  Participants shall submit Supply Offers for non-ICAP Resources for the entire capability range of the Resource in excess of the portion of the Resource exported as non-recallable energy or otherwise Self-Scheduled in the Real-Time Energy Market.

#### A Participant Self-Scheduling a Resource in the Day-Ahead Energy Market that does not deliver the energy in the Real-Time Energy Market, shall replace the energy not delivered with energy from the Real-Time Energy Market or an internal bilateral transaction and shall pay for such energy not delivered, net of any internal bilateral transactions, at the applicable Real-Time Price.

### ICAP Resources.

#### An ICAP Resource selected as a Pool-Scheduled Resource shall be made available for scheduling and dispatch at the direction of the ISO.  Any Resource, including an ICAP Resource, that does not deliver energy as scheduled shall be deemed to have experienced a Generator Forced Outage to the extent such energy is not delivered except that a reduction in output or removal from service of a generating unit in response to changes in market conditions that is approved by the ISO shall not constitute a Generator Forced Outage.  A Participant offering such Resource, including an ICAP Resource, in the Day-Ahead Energy Market shall replace the energy not delivered with energy from the Real-Time Energy Market or an internal bilateral transaction and shall pay for such energy not delivered, net of any internal bilateral transactions, at the applicable Real-Time Price.

#### Energy from an ICAP Resource that has not been scheduled in the Day-Ahead Energy Market may be sold on a bilateral basis by the Participant, may be Self-Scheduled, or may be offered for dispatch during the Operating Day in accordance with the procedures specified in this Market Rule.  An ICAP Resource that has not been scheduled in the Day-Ahead Energy Market and that has been scheduled as an External Transaction sale must be made available upon request to the ISO for scheduling and dispatch during the Operating Day if the ISO declares an Emergency Condition.  Any such Resource so scheduled and dispatched shall receive the applicable Real-Time Price for energy delivered.

### External Resources.

#### Participants with External Resources that have dynamic scheduling and dispatch capability with the ISO may submit Supply Offers to the NEPOOL Market, in accordance with the Day-Ahead and Real-Time scheduling processes specified above.  An External Resource with dynamic scheduling and dispatch capability with the ISO selected as a Pool-Scheduled Resource shall be made available for scheduling and dispatch at the direction of the ISO and shall be compensated on the same basis as other Pool-Scheduled Resources.  Resources external to the NEPOOL Control Area that are not capable of dynamic scheduling and dispatch shall, if selected by the ISO on the basis of the Participant’s Supply Offer, be block loaded on an hourly scheduled basis and shall be compensated on the same basis as External Transactions.  Participants shall offer External Resources to the NEPOOL Market on a Resource-specific basis.  A Participant whose External Resource does not deliver the energy scheduled in the Day-Ahead Energy Market shall replace such energy not delivered as scheduled in the Day-Ahead Energy Market with energy from the Real-Time Energy Market or an internal bilateral transaction and shall pay for such energy not delivered, net of any internal bilateral transactions, at the applicable Real-Time Price.

#### Supply Offers for External Resources with dynamic scheduling and dispatch capability with the ISO shall specify the Resource being offered, along with the information specified in the Offer Data as applicable.

### [Reserved.]

### External Transactions.

#### External Transactions scheduled in the Real-Time Energy Market shall continue to be implemented during periods of congestion, except as may be necessary to respond to emergencies.

#### All Real-Time External Transactions shall be scheduled and curtailed in accordance with the NEPOOL Manuals and all applicable tariffs.

### ISO Responsibilities.

#### The ISO shall use its best efforts to determine (i) the least-cost means of satisfying hourly purchase requests for energy, the projected hourly requirements for Operating Reserve, Replacement Reserve and other ancillary services of the Participants, including the reliability requirements of the NEPOOL Control Area, of the Day-Ahead Energy Market, and (ii) the least-cost means of satisfying the Operating Reserve, Replacement Reserve and other ancillary service requirements for any portion of the load forecast of the ISO for the Operating Day in excess of that scheduled in the Day-Ahead Energy Market.  In making these determinations, the ISO shall take into account:  (i) the ISO’s forecasts of NEPOOL Market and NEPOOL Control Area energy requirements, giving due consideration to the energy requirement forecasts and purchase requests submitted by Participants for the Day-Ahead Energy Market; (ii) the offers submitted by Participants; (iii) the availability of Limited Energy Resources; (iv) the capacity, location, and other relevant characteristics of Self-Scheduled Resources; (v) the objectives of the NEPOOL Control Area for Operating Reserve and Replacement Reserve, as specified in the NEPOOL Manuals and ISO Administrative Procedures; (vi) the requirements of the NEPOOL Control Area for Regulation and other ancillary services, as specified in the NEPOOL Manuals and ISO Administrative Procedures; (vii) the benefits of avoiding or minimizing transmission constraint control operations, as specified in the NEPOOL Manuals and ISO Administrative Procedures; and (viii) such other factors as the ISO reasonably concludes are relevant to the foregoing determination.  The ISO shall develop a Day-Ahead Energy schedule based on the applicable portions of the foregoing determination, and shall determine the Day-Ahead Prices resulting from such schedule.

#### Not later than 4:00 p.m. of the day before each Operating Day, or such earlier deadline as may be specified by the ISO in the NEPOOL Manuals and ISO Administrative Procedures or such later deadline as necessary to account for software failures or other events, the ISO shall:  (i) post the aggregate Day-Ahead Energy schedule; (ii) post the Day-Ahead Prices; and (iii) inform the Participants of their scheduled injections and withdrawals.  In the event of an Emergency, the ISO will notify Participants as soon as practicable if the Day-Ahead Energy Market can not be operated.

#### Following posting of the information specified in Section 1.10.8(b), the ISO shall revise its schedule of generating Resources to reflect updated projections of load, conditions affecting electric system operations in the NEPOOL Control Area, the availability of and constraints on limited energy and other Resources, transmission constraints, and other relevant factors.

#### Participants shall pay and be paid for the quantities of energy scheduled in the Day-Ahead Energy Market at the Day-Ahead Prices.

### Hourly Scheduling.

#### Following the initial posting by the ISO of the Locational Marginal Prices resulting from the Day-Ahead Energy Market, and subject to the right of the ISO to schedule and dispatch Pool-Scheduled Resources and to direct that schedules be changed in an Emergency, a Resource re-offer period shall exist from 4:00 p.m. to 6:00 p.m. on the day before each Operating Day or such other re-offer period as necessary to account for software failures or other events.  During the re-offer period, Participants may submit revisions to generation Supply Offers for any generating Resource that was not scheduled in the Day-Ahead Energy Market.  Generation Resources scheduled subsequent to the closing of the re-offer period shall be settled at the applicable Real-Time Prices, and shall not affect the obligation to pay or receive payment for the quantities of energy scheduled in the Day-Ahead Energy Market at the applicable Day-Ahead Prices.

#### A Participant may adjust the schedule of a Resource under its dispatch control on an hour-to-hour basis beginning at 10:00 p.m. of the day before each Operating Day, provided that the ISO is notified not later than 60 minutes prior to the hour in which the adjustment is to take effect (or such shorter period, up to 20 minutes prior to the hour, when the ISO has notified the Participants that it has the necessary hardware, software, and procedures in place to implement the shorter notice period), as follows:

##### A Participant may Self-Schedule any of its Resources consistent with the NEPOOL Manuals and ISO Administrative Procedures;

##### A Participant may request the scheduling of an External Transaction; or

##### [Reserved]; or

##### A Participant may remove from service a Resource increment, that it had previously designated as Self-Scheduled, provided that the ISO shall have the option to schedule energy from any such Self-Scheduled Resource increment that is an ICAP Resource at the price offered in the scheduling process, with no obligation to pay any Start-Up Fee.

#### With respect to a Pool-Scheduled Resource that is scheduled in the Day-Ahead Energy Market, a Participant may not change or otherwise modify its offer to sell energy.

#### [Reserved.]

#### For each hour in the Operating Day, as soon as practicable after the deadlines specified in the foregoing subsection of this Section 1.10, the ISO shall provide Participants and parties to External Transactions with any revisions to their schedules for the hour.

## Dispatch.

The following procedures and principles shall govern the dispatch of the Resources available to the ISO.

### Resource Output.

The ISO shall have the authority to direct any Participant to adjust the output of any Pool-Scheduled Resource increment within the operating characteristics specified in the Participant’s Offer Data and Supply Offer.  The ISO may cancel its selection of, or otherwise release, Pool-Scheduled Resources, subject to an obligation to pay any applicable Start-Up, No-Load or Cancellation Fees.  The ISO shall adjust the output of Pool-Scheduled Resource increments as necessary:  (a) to maintain reliability, and subject to that constraint, to minimize the cost of supplying the energy, reserves, and other services required by the Participants and the operation of the NEPOOL Control Area; (b) to balance load and generation, maintain scheduled tie flows, and provide frequency support within the NEPOOL Control Area; and (c) to minimize unscheduled interchange that is not frequency related between the NEPOOL Control Area and other Control Areas.

### Operating Basis.

In carrying out the foregoing objectives, the ISO shall conduct the operation of the NEPOOL Control Area and shall, in accordance with the NEPOOL Manuals and ISO Administrative Procedures, (i) utilize available generating reserves and replace such generating reserves when utilized; and (ii) monitor the availability of adequate reserves.

### Pool-dispatched Resources.

#### The ISO shall optimize the dispatch of energy from Limited Energy Resources by request to minimize the as-bid production cost for the NEPOOL Control Area.  In implementing the use of Limited Energy Resources, the ISO shall use its best efforts to select the most economic hours of operation for Limited Energy Resources, in order to make optimal use of such Resources consistent with the dynamic load-following requirements of the NEPOOL Control Area and the availability of other Resources to the ISO.

#### The ISO shall implement the dispatch of energy from other Pool-Scheduled Resource increments, including generation increments from ICAP Resources the remaining increments of which are Self-Scheduled, by sending appropriate signals and instructions to the entity controlling such Resources, in accordance with the NEPOOL Manuals and ISO Administrative Procedures.  Each Participant shall ensure that the entity controlling a Pool-Scheduled Resource offered or made available by that Participant complies with the energy dispatch signals and instructions transmitted by the ISO.

#### The ISO shall have the authority to modify a Participant’s operational related Offer Data if the ISO observes that the Participant’s Resource is not operating in accordance with such Offer Data.  The ISO shall modify such operational related Offer Data based on observed performance and such modified Offer Data shall remain in effect until either (i) the affected Participant requests a test to be performed and the results of the test justify a change to the Participant’s Offer Data or (ii) the ISO observes, through actual performance, that modification to the Participant’s Offer Data is justified.  Procedures related to the ISO’s modification of operational related Offer Data and Participant requests for testing shall be as defined in the NEPOOL Manuals.

#### Participants shall exert all reasonable efforts to operate, or ensure the operation of, their Resources in the NEPOOL Control Area as close to desired output levels as practical, consistent with Accepted Electric Industry Practice.

1.11.3A  Emergency Condition.

If the ISO anticipates or declares an Emergency Condition, all sales out of the NEPOOL Control Area from ICAP Resources may be interrupted, in accordance with the NEPOOL Manuals, in order to serve load and Operating Reserve in the NEPOOL Control Area.

### Regulation.

#### A Participant may satisfy its Regulation obligation from its own Resources capable of performing Regulation service, by contractual arrangements with other Participants, or by purchases from the NEPOOL Market at the rates set forth in Section 3.2.2.

#### The ISO shall obtain Regulation service from the least-cost alternatives available from either Pool-Scheduled or Self-Scheduled Resources as needed to meet NEPOOL Control Area requirements not otherwise satisfied by the Participants.  Resources offering to sell Regulation shall be selected to provide Regulation on the basis of each Resource’s Regulation offer and the estimated Opportunity Cost of the Resource providing Regulation and in accordance with the ISO’s obligation to minimize the total cost of energy, Operating Reserve, Regulation, and other ancillary services.  Estimated Opportunity Costs shall be determined by the ISO on the basis of the expected value of the energy sales that would be foregone or uneconomic energy that would be produced by the Resource in order to provide Regulation, in accordance with procedures specified in the NEPOOL Manuals and ISO Administrative Procedures.  If the ISO is not able to distinguish Resources offering Regulation on the basis of their Regulation offers and estimated Opportunity Costs, Resources shall be selected on the basis of the quality of Regulation provided by the Resource as determined by tests administered by the ISO or as needed to preserve system reliability.

#### The ISO shall dispatch Resources for Regulation by sending Regulation signals and instructions to Resources from which Participants, in accordance with the NEPOOL Manuals and ISO Administrative Procedures, have offered Regulation service.  Participants shall comply with Regulation dispatch signals

and instructions transmitted by the ISO and, in the event of conflict, Regulation dispatch signals and instructions shall take precedence over energy dispatch signals and instructions.  Participants shall exert all reasonable efforts to operate, or ensure the operation of, their Resources supplying load in the NEPOOL Control Area as close to desired output levels as practical, consistent with Accepted Electric Industry Practice.

1.11.4A  [Reserved - Spinning Reserve.]

## Dynamic Scheduling.

Dynamic Scheduling can be requested and may be implemented in accordance with the following procedures:

#### An entity that owns or controls a generating Resource in the NEPOOL Control Area may electrically remove all or part of the generating Resource’s output from the NEPOOL Control Area through dynamic scheduling of the output to load outside the NEPOOL Control Area.  Such output shall not be available for economic dispatch by the ISO.

#### An entity that owns or controls a generating Resource outside of the NEPOOL Control Area may electrically include all or part of the generating Resource’s output into the NEPOOL Control Area through dynamic scheduling of the output to load inside the NEPOOL Control Area.  Such output shall be available for economic dispatch by the ISO.

#### An entity requesting dynamic scheduling shall be responsible for arranging for the provision of signal processing and communication from the generating unit and other participating Control Area and complying with any other procedures established by the ISO regarding dynamic scheduling as set forth in the NEPOOL Manuals.  Allocation of costs associated with dynamic scheduling shall be determined and filed with the Commission following the first request.

#### An entity requesting dynamic scheduling shall be responsible for reserving amounts of appropriate transmission service necessary to deliver the range of the dynamic transfer and any ancillary services.

# CALCULATION OF LOCATIONAL MARGINAL PRICES

## Introduction.

The ISO shall calculate the price of energy at Nodes, Load Zones and Hubs in the NEPOOL Control Area and at External Nodes on the basis of Locational Marginal Prices as determined in accordance with this Market Rule.  Locational Marginal Prices shall be calculated on a Day-Ahead basis for each hour of the Day-Ahead Energy Market, and every five minutes during the Operating Day for the Real-Time Energy Market.

## General.

The ISO shall determine the least cost security-constrained unit commitment and dispatch, which is the least costly means of serving load at different Locations in the NEPOOL Control Area based on scheduled or actual conditions, as applicable, existing on the power grid and on the prices at which Participants have offered to supply and consume energy in the NEPOOL Market.  Day-Ahead and Real-Time Locational Marginal Prices for the applicable Locations will be calculated based on the unit commitment and economic dispatch and the prices of energy offers and bids.  Except as further provided in Section 2.6, the process for the determination of Locational Marginal Prices shall be as follows:

#### To determine operating conditions, in the Day Ahead Energy Market or Real-Time Energy Market, on the NEPOOL Transmission System, the ISO shall use a computer model of the interconnected grid that uses scheduled quantities or available metered inputs regarding generator output, loads, and power flows to model remaining flows and conditions, producing a consistent representation of power flows on the network.  The computer model employed for this purpose in the Real-Time Energy Market, referred to as the State Estimator program, is a standard industry tool and is described in Section 2.3.  It will be used to obtain information regarding the output of generation supplying energy to the NEPOOL Control Area, loads at busses in the NEPOOL Control Area, transmission losses, penalty factors, and power flows on binding transmission constraints for use in the calculation of Day-Ahead and Real-Time Locational Marginal Prices.  Additional information used in the calculation of Real-Time Locational Marginal Prices, including Dispatch Rates and Real-Time schedules for External Transactions, will be obtained from the ISO’s dispatch software and dispatchers.

#### Using the prices at which Participants offer and bid energy to the NEPOOL Market, the ISO shall determine the offers and bids of energy that will be considered in the calculation of Day-Ahead Prices and Real-Time Prices.  As described in Section 2.4, every offer of energy by a Participant from a generating Resource and External Transaction purchase Resource that is following economic dispatch instructions of the ISO will be utilized in the calculation of Real-Time Locational Marginal Prices.  As described in Section 2.6, every offer and bid by a Participant that is scheduled in the Day Ahead Energy Market will be utilized in the calculation of Day Ahead Locational Marginal Prices.

## Determination of System Conditions Using the State Estimator.

Power system operations, including, but not limited to, the determination of the least costly means of serving load, depend upon the availability of a complete and consistent representation of generator outputs, loads, and power flows on the network.  In calculating Day-Ahead Prices, the ISO shall base the system conditions on the expected transmission system configuration and the set of offers and bids submitted by Participants.  In calculating Real-Time Locational Marginal Prices, the ISO shall obtain a complete and consistent description of conditions on the electric network in the NEPOOL Control Area by using the most recent power flow solution produced by the State Estimator, which is also used by the ISO for other functions within power system operations.  The State Estimator is a standard industry tool that produces a power flow model based on available Real-Time metering information, information regarding the current status of lines, generators, transformers, and other equipment, bus load distribution factors, and a representation of the electric network, to provide a complete description of system conditions, including conditions at Nodes and External Nodes for which Real-Time information is unavailable.  In calculating Real-Time Locational Marginal Prices, the ISO shall obtain a State Estimator solution every five minutes, which shall provide the megawatt output of generators and the loads at Locations in the NEPOOL Control Area, transmission line losses, penalty factors, and actual flows or loadings on constrained transmission facilities.  External Transactions between NEPOOL and other Control Areas shall be included in the Real-Time Locational Marginal Price calculation on the basis of the Real-Time transaction schedules implemented by the ISO’s dispatcher.

## Determination of Energy Offers Used in Calculating Real-Time Prices.

#### During the Operating Day, Real-Time nodal Locational Marginal Prices derived in accordance with this Section shall be determined every five minutes and integrated hourly values of such determinations shall be the basis of the settlement of sales and purchases of energy in the Real-Time Energy Market and of Congestion Costs and costs for losses under the NEPOOL Tariff not covered by the Day-Ahead Energy Market.

#### To determine the energy offers submitted to the NEPOOL Market that shall be used during the Operating Day to calculate the Real-Time nodal Locational Marginal Prices, the ISO shall determine which generating unit and External Transaction purchases are following its economic dispatch instructions.  A generating unit or External Transaction purchase will be considered to be following economic dispatch instructions and shall be included in the calculation of Real-Time Prices if:

##### the applicable Supply Offer price submitted by a Participant for energy from the generating unit or External Transaction purchase is less than or equal to the Dispatch Rate associated with that generating unit or External Transaction purchase; and

##### the generating unit, other than a Fast Start Generator, is operating above its Economic Minimum Limit; or

##### the Fast Start Generator is operating at or above its Economic Minimum Limit and the applicable Supply Offer price submitted by a Participant for energy from the Fast Start Generator is less than or equal to the Dispatch Rate associated with that Fast Start Generator; or

##### the generating unit or External Transaction purchase is specifically requested to operate by the ISO’s dispatcher and the associated energy offers submitted are otherwise eligible to be included in the calculation of Real-Time Location Marginal Prices.

#### In determining whether a generating unit or External Transaction purchase satisfies the condition described in 2.4(b), the ISO will determine the Supply Offer price associated with an energy offer by comparing the actual megawatt output of the generating unit or External Transaction purchase with the Participant’s Supply Offer price curve for that generating unit or External Transaction purchase.  Because of practical generator response limitations, a generating unit whose megawatt output is not more than ten percent above the megawatt level specified in the Supply Offer price curve for the applicable Dispatch Rate shall be deemed to be following economic dispatch instructions, but the energy price offer used in the calculation of Real-Time Prices shall not exceed the applicable Dispatch Rate.

## Calculation of Real-Time Nodal Prices.

#### The ISO shall determine the least costly means of obtaining energy to serve the next increment of load at each Node internal to the NEPOOL Control Area represented in the State Estimator and each External Node Location between the NEPOOL Control Area and an adjacent Control Area, based on the system conditions described by the most recent power flow solution produced by the State Estimator program and the energy offers that are the basis for the Day-Ahead Energy Market, or that are determined to be eligible for consideration under Section 2.4 in connection with the Real-Time dispatch, as applicable.  This calculation shall be made by applying an incremental linear optimization method to minimize energy, congestion and transmission loss costs, given actual system conditions, a set of energy offers, and any binding transmission constraints that may exist.  In performing this calculation, the ISO shall calculate the cost of serving an increment of load at each Node and External Node from all available generating units and External Transaction purchases with an eligible energy offer as the sum of:  (1) the price at which the Participant has offered to supply an additional increment of energy from the generating unit or External Transaction purchase; (2) the effect on transmission Congestion Costs (whether positive or negative) associated with increasing the output of the generating unit or External Transaction purchase, based on the effect of increased generation from that generating unit or External Transaction purchase on transmission line loadings; and (3) the effect on transmission losses caused by the increment of load and generation. The energy offer or offers that can serve an increment of load at a Location at the lowest cost, calculated in this manner, shall determine the Real-Time Price at that Node or External Node.

#### During the Operating Day, the calculation set forth in this Section 2.5 shall be performed every five minutes, using the ISO’s Locational Marginal Price program, producing a set of nodal Real-Time Prices based on system conditions during the preceding interval.  The prices produced at five-minute intervals during an hour will be integrated to determine the nodal Real-Time Prices for that hour.

#### For any interval during any hour in the Operating Day that the ISO has declared a Minimum Generation Emergency, the affected nodal Real-Time Prices calculated under this Section 2.5. shall be set equal to zero for all Nodes within the NEPOOL Control Area and all External Nodes if the Minimum Generation Emergency was declared on a NEPOOL Control Area wide basis or shall be set equal to zero for all Nodes and External Nodes within a sub-region if the Minimum Generation Emergency was declared within the sub-region.

#### For any interval during any hour in the Operating Day for which the ISO has declared a Reserve Shortage Condition Pricing Event, the affected nodal Real-Time Prices calculated under this Section 2.5 shall be set as follows:

##### The Energy Component of the affected nodal Real-Time Price shall be set to the higher of the Reserve Shortage Condition Energy Price or the Energy Component of the nodal Real-Time Price if the actions by the System Operator have not been sufficient to restore the Operating Reserves to the required levels as defined in NEPOOL Operating Procedure No. 8,

##### If an External Dispatchable Transaction purchase has been scheduled and the MWs of the External Dispatchable Transaction purchase are sufficient to restore the Operating Reserves to the required levels, then the Energy Component of the affected nodal Real-Time Prices shall be set to the higher of the Offer Price of the External Dispatchable Transaction purchase or the Energy Component of the nodal Real-Time Prices.

###### If a Reserve Shortage Condition Pricing Event has been declared on a NEPOOL Control Area wide basis, the Energy Component of the Real-Time Price for all Nodes within the NEPOOL Control Area and all External Nodes shall be used in the calculation above.  If a Reserve Shortage Condition Pricing Event has been declared that excludes one or more Reliability Regions of the NEPOOL Control Area from the Reserve Shortage Condition Pricing Event, the nodal Real-Time Prices for all Nodes and External Nodes within the excluded Reliability Region(s) shall not be adjusted.

###### A Reliability Region will be excluded from the Reserve Shortage Condition Pricing Event when the Interface Limit Calculation software indicates that the Reliability Region is export-constrained.  A Reliability Region is export-constrained when the sum of the actual flows on the transmission lines that comprise the interface is equal to the transfer limit for the interface.  When a Reliability Region is export-constrained, the output of generating Resources within the region can only be increased to satisfy increases in load that is also located within the region.  The output of generating Resources within

###### the region cannot be increased to serve load in the remainder of the NEPOOL Control Area.

The Loss Component of any nodal Real-Time Prices adjusted under Section (d) will be adjusted by the factor [Loss Componentinitial/Energy Componentinitial] times Energy Componentadjusted.

The affected nodal Real-Time Prices will then be calculated by summing the Energy, Loss, and Congested Components and those nodal Real-Time Prices will be used to calculate the hourly Locational Marginal Prices.

If the Market Monitoring Unit, in conjunction with the Independent Market Advisor, determines that the Reserve Shortage Condition and the subsequent adjustments to the affected Energy and Loss Components of the nodal Real-Time Prices as set under this Section 2.5(d) were the result of physical withholding (as defined in Appendix A to Market Rule 1) by one or more Participants, the affected nodal Real-Time Prices will not be adjusted in the price finalization as set forth in Section 2.9.

## Calculation of Day-Ahead Nodal Prices.

(a)	For the Day-Ahead Energy Market, Day-Ahead Prices shall be determined on the basis of the least-cost, security-constrained unit commitment and dispatch, model flows and system conditions resulting from the load specifications submitted by Participants, Supply Offers and Demand Bids for Resources, Increment Offers, Decrement Bids, and External Transactions submitted to the ISO and scheduled in the Day-Ahead Energy Market.  Such prices shall be determined in accordance with the provisions of this Section applicable to the Day-Ahead Energy Market and shall be the basis for the settlement of purchases and sales of energy, costs for losses and Congestion Costs resulting from the Day-Ahead Energy Market.  This calculation shall be made for each hour in the Day-Ahead Energy Market by applying a linear optimization method to minimize energy, congestion and transmission loss costs, given scheduled system conditions, scheduled transmission outages, and any transmission limitations that may exist.  In performing this calculation, the ISO shall calculate the cost of serving an increment of load at each Node and External Node from each Resource associated with an eligible energy offer or bid as the sum of: (1) the price at which the Participant has offered to supply an additional increment of energy from the Resource or reduce consumption from the Resource; (2) the effect on transmission Congestion Costs (whether positive or negative) associated with increasing the output of the Resource or reducing consumption of the Resource, based on the effect of increased generation from that Resource or reduced consumption from a Dispatchable Load Resource on transmission line loadings; and (3) the effect on transmission losses caused by the increment of load and generation.  The energy offer or offers and energy bid or bids that can serve an increment of load at a Node or External Node at the lowest cost, calculated in this manner, shall determine the Day-Ahead Price at that Node.

(b)	Energy deficient conditions. If the sum of Day-Ahead fixed Demand Bids and fixed External Transaction sales cannot be satisfied with the sum of all scheduled External Transaction purchases, cleared Increment Offers, and available generation at its Economic Maximum Limit, the technical software issues an Emergency

Condition warning message due to a shortage of economic supply in the Day-Ahead Energy Market. The following steps shall then be performed to achieve power balance:

(i)	All fixed External Transaction sales are considered to be dispatchable at $1,000/MWh;

(ii)	Reduce any remaining price-sensitive Demand Bids (including External Transaction sales) and Decrement Bids from lowest price to highest price to zero MW until power balance is achieved (there may be some price sensitive bids that are higher priced than the highest Supply Offer or Increment Offer price cleared).  Set LMP values equal to the highest price-sensitive Demand Bid or Decrement Bid that was cut in this step.  If no price-sensitive Demand Bid or Decrement Bid was reduced in this step, the LMP values are set equal to highest offer price of all on-line generation, Increment Offers or External Transaction purchases; and

(iii)	If power balance is not achieved after step (ii), reduce all remaining fixed Demand Bids proportionately (by ratio of load MW) until balance is achieved.  Set LMP values equal to the highest offer price of all on-line generation, Increment Offers or External Transaction purchases or the price from step (ii), whichever is higher.

(c)	Excess energy conditions. If the sum of Day-Ahead cleared Demand Bids, Decrement Bids and External Transaction sales is less than the total system wide generation MW (including Fixed External Transaction purchases) with all possible generation off and with all remaining generation at their Economic Minimum Limit, the technical software issues a Minimum Generation Emergency warning message due to an excess of economic generation in the Day-Ahead Energy Market.  The following steps shall then be performed to achieve power balance:

(i)	All fixed External Transaction purchases are considered to be dispatchable at $0/MWh and reduced pro-rata, as applicable, until power balance is reached;

(ii)	If power balance is not reached in step (i), reduce all committed generation down proportionately by ratio of Economic Minimum Limits but not below Emergency Minimum Limits.  If power balance is achieved prior to reaching Emergency Minimum Limits, set LMP values equal to the lowest offer price of all on-line generation; and

(iii)	If power balance not achieved in step (ii), set LMP values to zero and reduce all committed generation below Emergency Minimum Limits proportionately (by ratio of Emergency Minimum Limits) to achieve power balance.

## Reliability Regions, Load Zones, Zonal Prices and External Nodes.

#### The ISO shall calculate Zonal Prices for each Load Zone for both the Day-Ahead and Real-Time Energy Markets for each hour using a load-weighted average of the Locational Marginal Prices for the Nodes within that Load Zone.  The load weights used in calculating the Day-Ahead Zonal Prices for the Load Zone shall be determined in accordance with applicable Market Rules and shall be based on historical load usage patterns.  The load weights do not reflect Demand Bids or Decrement Bids that settle at the Node level in the Day-Ahead Market.  The ISO shall determine, in accordance with applicable NEPOOL Manuals, the load weights used in Real-Time based on the actual Real-Time load distribution as calculated by the State Estimator, and shall exclude any Dispatchable Load that settles at the Node from the load weights used to calculate the applicable Real-Time Zonal Prices.

#### Each Load Zone shall initially be approximately coterminous with a Reliability Region.

#### After consulting with the Participants, the ISO may reconfigure Reliability Regions and Load Zones and add or subtract Reliability Regions and Load Zones as necessary over time to reflect changes to the grid, patterns of usage and intrazonal Congestion.  The ISO shall file any such changes with the Commission.

#### In the event the ISO makes changes to a Reliability Region or Load Zone or adds or subtracts Reliability Regions and Load Zones, for settlement purposes and to the extent practicable, Load Assets that are physically located in one Reliability Region and electrically located within another Reliability Region shall be located within the Reliability Region to which they are electrically located.

#### External Nodes are the nodes at which External Transactions settle.  As appropriate and after consulting with Participants, the ISO will establish and re-configure External Nodes taking into consideration appropriate factors, which may include:  tie line operational matters, FTR modeling and auction assumptions, market power issues associated with external contractual arrangements, impacts on Locational Marginal Prices, and inter-regional trading impacts.

#### On or about the 20th calendar day of each month, the ISO shall publish the Real-Time nodal load weights (expressed in MW) used to calculate the load-weighted Real-Time Zonal Prices for the preceding month.  Nodal load weights will be published for all nodes used in the calculations except for those nodes identified by customers as nodes for which publication would provide individual customer usage data.  Any individual customer whose usage data would be revealed by publication of load weight information associated with a specific Node must submit a written request to the ISO to omit the applicable Node from the publication requirement.  The request must identify the affected Node and, to the best of the customer’s knowledge, the number of customers taking service at the affected Node and the estimated percentage of the total annual load (MWh) at the affected Node period that is attributable to the customer.  The information contained in the request must be certified in writing by an officer of the customer’s company (if applicable), by an affidavit signed by a person having knowledge of the applicable facts, or by representation of counsel for the customer.  The ISO will grant a customer request if it determines based on the information provided that no more than two customers are taking service at the affected Node or that the percentage of the customer’s annual load (MWh) at the affected Node is greater than 75 prevent of the total load (MWh) at the affected Node.  If a customer request is granted and that customer request is the only such customer request within a Load Zone, then the ISO shall randomly select one other Node and not disclose hourly load information for the randomly selected Node unless and until another customer request within that Load Zone is granted.  A request to suspend publication for a month must be received by the ISO on or before the 10th calendar day of the following month in order to be effective for that month.  Upon receipt of a request, the ISO will suspend publication of the load weight data for the specified Node.  The ISO may, from time to time, require customer confirmation that continued omission from publication of load weight data for a particular Node is required in order to avoid disclosure of individual customer usage data.  If customer confirmation is not received within a reasonable period not to exceed 30 days, the ISO may publish load weight data for the applicable Node.

## Hubs and Hub Prices.

#### On behalf of the Participants, the ISO shall maintain and facilitate the use of a Hub or Hubs for the Day-Ahead and Real-Time Energy Market, comprised of a set of Nodes within the NEPOOL Control Area, which Nodes shall be identified by the ISO on its internet website.  The ISO has used the following criteria to establish an initial Hub and shall use the same criteria to establish any additional Hubs:

##### Each Hub shall contain a sufficient number of Nodes to try to ensure that a Hub Price can be calculated for that Hub at all times;

##### Each Hub shall contain a sufficient number of Nodes to ensure that the unavailability of, or an adjacent line outage to, any one Node or set of Nodes would have only a minor impact on the Hub Price;

##### Each Hub shall consist of Nodes with a relatively high rate of service availability;

##### Each Hub shall consist of Nodes among which transmission service is relatively unconstrained; and

##### No Hub shall consist of a set of Nodes for which directly connected load and/or generation at that set of Nodes is dominated by any one entity or its affiliates.

#### The ISO shall calculate and publish hourly Hub Prices for both the Day-Ahead and Real-Time Energy Markets based upon the arithmetic average of the Locational Marginal Prices of the nodes that comprise the Hub.

## Final Real-Time Prices and Day-Ahead Energy Market Results.

(a)	The ISO shall post the final Real-Time Prices as soon as practicable following the Operating Day, in accordance with the timeframes specified in the NEPOOL Manuals, except that the posting of such final Real-Time Prices by the ISO shall not exceed five (5) business days from the applicable Operating Day.  Posting of final Real-Time Prices exceeding five (5) business days from the applicable Operating Day shall be approved by the ISO Board.  If the ISO is not able to calculate Real-Time Prices normally due to hardware, software, or telecommunication problems that cannot be remedied in a timely manner, the ISO will calculate Real-Time Prices as soon as practicable using the best data available.  In all cases, the ISO shall calculate prices consistent with this Market Rule.

(b)	Day-Ahead Energy Market results are final when published except as provided in this subsection.  If the ISO determines based on reasonable belief that there may be one or more errors in the Day-Ahead Energy Market results for an Operating Day due to database, software or similar errors of the ISO or its systems, the ISO shall post on the ISO website prior to 12:01 a.m. of the applicable Operating Day (4:00 p.m. for Operating Days prior to May 1, 2004), a notice that the results are provisional and subject to correction.  Any Day-Ahead Energy Market results for which no notice is posted shall be final and not subject to correction or other

adjustment and shall be used for the purposes of settlement.  The ISO shall confirm within three business days of the close of the applicable Operating Day whether there was an error in any provisional Day-Ahead Energy Market results and shall post a notice stating its findings.

(c)	The ISO will publish corrected Day-Ahead Energy Market results within three business days of the close of the applicable Operating Day or the results of the Day-Ahead Energy Market for the Operating Day will stand.  The ISO shall also publish a statement describing the nature of the error and the method used to correct the results.

(d)	If the ISO determines in accordance with subsection (b) that there are one or more errors in the Day-Ahead Energy Market results for an Operating Day, the ISO shall calculate corrected Day-Ahead Energy Market results by determining and substituting for the initial results, final results that reasonably reflect how the results would have been calculated but for the errors.  To the extent that it is necessary, reasonable, and practicable to do so, the ISO may specify an allocation of any costs that are not otherwise allocable under applicable provisions of Market Rule 1.  The ISO shall use the corrected results for purposes of settlement.

(e)	For every change in the Day-Ahead Energy Market results made pursuant to Section 2.9, the ISO will prepare and submit, as soon as practicable, an informational report to the Commission describing the nature of any errors, the precise remedy administered, the method of determining corrected prices and allocating any costs, and any remedial actions that will be taken to avoid similar errors in the future.

## Performance Evaluation.

The ISO shall undertake an evaluation of the foregoing procedures for the determination of Locational Marginal Prices, as well as the procedures for determining and awarding Financial Transmission Rights and associated Congestion Costs and Transmission Congestion Credits, not less often than every two years.  The ISO, in conjunction with the Independent Market Advisor, will conduct a review of the market after 6 months of operation, or after the first summer of operations, whichever occurs first.  To the extent practical, the ISO shall retain all data needed to perform comparisons and other analyses of locational marginal pricing.  The ISO shall report the results of its evaluation to the Participants, along with its recommendations, if any, for changes in the procedures.

# ACCOUNTING AND BILLING

## Introduction.

This Section 3 sets forth the accounting and billing principles and procedures for the purchase and sale of services in the NEPOOL Market and for the operation of the NEPOOL Control Area.

## Participants.

### NEPOOL Market Energy.

#### For each Participant for each hour, the ISO will determine a Day-Ahead Energy Market position representing that Participant’s net purchases from or sales to the NEPOOL Day-Ahead Energy Market.  To accomplish this, the ISO will perform calculations to determine the following.

##### Day-Ahead Load Obligation – Each Participant shall have for each hour a Day-Ahead Load Obligation for energy at each Location equal to the MWhs of its Demand Bids, Decrement Bids and External Transaction sales accepted by the ISO in the Day-Ahead Energy Market at that Location and such Day-Ahead Load Obligation shall have a negative value.

##### Day-Ahead Generation Obligation – Each Participant shall have for each hour a Day-Ahead Generation Obligation for energy at each Location equal to the MWhs of its generation Supply Offers, Increment Offers and External Transaction purchases accepted by the ISO in the Day-Ahead Energy Market at that Location and such Day-Ahead Generation Obligation shall have a positive value.

##### Day-Ahead Adjusted Load Obligation – Each Participant shall have for each hour a Day-Ahead Adjusted Load Obligation at each Location equal to the Day-Ahead Load Obligation adjusted by any applicable Day-Ahead internal bilateral transactions at that Location.

##### Day-Ahead Locational Adjusted Net Interchange – Each Participant shall have for each hour a Day-Ahead Locational Adjusted Net Interchange at each Location equal to the Day-Ahead Adjusted Load Obligation plus the Day-Ahead Generation Obligation at that Location.

#### For each Participant for each hour, the ISO will determine a Real-Time Energy Market position.  To accomplish this, the ISO will perform calculations to determine the following:

##### Real-Time Load Obligation – Each Participant shall have for each hour a Real-Time Load Obligation for energy at each Location equal to the MWhs of load, where such MWhs of load shall include External Transaction sales and shall have a negative value, at that Location, adjusted for any applicable internal bilateral transactions which transfer Real-Time load obligations.

##### Real-Time Generation Obligation – Each Participant shall have for each hour a Real-Time Generation Obligation for energy at each Location.  The Real-Time Generation Obligation shall equal the MWhs of energy, where such MWhs of energy shall have positive value, provided by generating Resources, External Resources, and External Transaction purchases at that Location.

##### Real-Time Adjusted Load Obligation – Each Participant shall have for each hour a Real-Time Adjusted Load Obligation at each Location equal to the Real-Time Load Obligation adjusted by any applicable energy related internal Real-Time bilateral transactions at that Location.

##### Real-Time Locational Adjusted Net Interchange – Each Participant shall have for each hour a Real-Time Locational Adjusted Net Interchange at each Location equal to the Real-Time Adjusted Load Obligation plus the Real-Time Generation Obligation at that Location.

#### For each Participant for each hour, the ISO will determine the difference between the Day-Ahead Energy Market position (Section 3.2.1(a)) and the Real-Time Energy Market position (Section 3.2.1(b)) representing that Participant’s net purchases from or sales to the Real-Time Energy Market.  To accomplish this, the ISO will perform calculations to determine the following:

##### Real-Time Load Obligation Deviation – Each Participant shall have for each hour a Real-Time Load Obligation Deviation at each Location equal to the difference in MWhs between the Real-Time Load Obligation and the Day-Ahead Load Obligation.

##### Real-Time Generation Obligation Deviation – Each Participant shall have for each hour a Real-Time Generation Obligation Deviation at each Location equal to the difference in MWhs between the Real-Time Generation Obligation and the Day-Ahead Generation Obligation.

##### Real-Time Adjusted Load Obligation Deviation – Each Participant shall have for each hour a Real-Time Adjusted Load Obligation Deviation at each Location equal to the difference in MWhs between the Real-Time Adjusted Load Obligation and the Day-Ahead Adjusted Load Obligation.

##### Real-Time Locational Adjusted Net Interchange Deviation – Each Participant shall have for each hour a Real-Time Locational Adjusted Net Interchange Deviation at each Location equal to the difference in MWhs between the Real-Time Locational Adjusted Net Interchange and the Day-Ahead Locational Adjusted Net Interchange.

#### For each Participant for each hour, the ISO will determine Day-Ahead Energy Market monetary positions representing a charge or credit for its net purchases from or sales to the NEPOOL Day-Ahead Energy Market.  The Day-Ahead Energy Market Energy Charge/Credit shall be equal to the sum of its Location specific Day-Ahead Locational Adjusted Net Interchanges multiplied by the Energy Component of the associated Day-Ahead Locational Marginal Prices.  The Day-Ahead Energy Market Congestion Charge/Credit shall be equal to the sum of its Location specific Day-Ahead Locational Adjusted Net Interchanges multiplied by the Congestion Component of the associated Day-Ahead Locational Marginal Prices.  The Day-Ahead Energy Market Loss Charge/Credit shall be equal to the sum of its Location specific Day-Ahead Locational Adjusted Net Interchanges multiplied by the Loss Component of the associated Day-Ahead Locational Marginal Prices.

#### For each Participant for each hour, the ISO will determine Real-Time Energy Market monetary positions representing a charge or credit to the Participant for its net purchases from or sales to the Real-Time Energy Market.  The Real-Time Energy Market Deviation Energy Charge/Credit shall be equal to the sum of its Location specific Real-Time Locational Adjusted Net Interchange Deviations multiplied by the Energy Component of the associated Real-Time Locational Marginal Prices.  The Real-Time Energy Market Deviation Congestion Charge/Credit shall be

equal to the sum of its Location specific Real-Time Locational Adjusted Net Interchange Deviations multiplied by the Congestion Component of the associated Real-Time Locational Marginal Prices.  The Real-Time Energy Market Deviation Loss Charge/Credit shall be equal to the sum of its Location specific Real-Time Locational Adjusted Net Interchange Deviations multiplied by the Loss Component of the associated Real-Time Locational Marginal Prices.

#### For each hour, the ISO will determine the total revenues associated with transmission congestion on the NEPOOL Transmission System.  To accomplish this, the ISO will perform calculations to determine the following.  The Day-Ahead Congestion Revenue shall equal the sum of all Participants’ Day-Ahead Energy Market Congestion Charge/Credits.  The Real-Time Congestion Revenue shall equal the sum of all Participants’ Real-Time Energy Market Congestion Charge/Credits.

#### For each hour, the ISO will determine the excess or deficiency in Loss Revenue associated with the Day-Ahead Energy Market.  The Day-Ahead Loss Revenue shall be equal to the sum of all Participants’ Day-Ahead Energy Market Energy Charge/Credits and Day-Ahead Energy Market Loss Charge/Credits.

#### For each hour for each Participant, the ISO shall calculate a Day-Ahead payment or charge associated with the excess or deficiency in Loss Revenue (Section 3.2.1(g)).  The Day-Ahead Loss Charges or Credits shall be equal to the Day-Ahead Loss Revenue multiplied by the Participant’s pro rata share of the sum of all Participants’ Real-Time Adjusted Load Obligations.

#### For each hour, the ISO will determine the excess or deficiency in Loss Revenue associated with the Real-Time Energy Market.  The Real-Time Loss Revenue shall be equal to the sum of all Participants’ Real-Time Energy Market Deviation Energy Charge/Credit and Real-Time Energy Market Deviation Loss Charge/Credit plus Non-Participant Transmission Customer loss costs.  The ISO will then adjust Real-Time Loss Revenue to account for Inadvertent Energy Revenue, as calculated under Section 3.2.1(k) and Emergency transactions as described under Section 4.3(a).

#### Non-Participant Transmission Customer loss costs shall be assessed for transmission use scheduled in the Real-Time Energy Market, calculated

as the amount to be delivered in each hour multiplied by the difference between the Loss Component of the hourly Real-Time Price at the delivery point or NEPOOL Control Area boundary delivery interface and the Loss Component of the hourly Real-Time Price at the source point or NEPOOL Control Area boundary source interface.

#### For each hour, the ISO will calculate an excess or deficiency in Inadvertent Energy Revenue by multiplying the Inadvertent Interchange at each External Node by the associated Real-Time Locational Marginal Price and then summing these values for all External Nodes.

#### For each hour for each Participant, the ISO shall calculate a Real-Time payment or charge associated with the excess or deficiency in Inadvertent Energy Revenue (Section 3.2.1(k)).  The Inadvertent Energy Revenue Charges or Credits shall be equal to the Inadvertent Energy Revenue multiplied by the Participant’s pro rata share of the sum of all Participants’ Real-Time Load Obligations and Real-Time Generation Obligations over all Locations, measured as absolute values.

#### For each hour for each Participant, the ISO shall calculate a Real-Time payment or charge associated with the excess or deficiency in Real-Time Loss Revenue (Section 3.2.1(i)).  The Real-Time Loss Revenue Charges or Credits shall be equal to the Real-Time Loss Revenue multiplied by the Participant’s pro rata share of the sum of all Participants’ Real-Time Adjusted Load Obligations.

### Regulation.

#### Each Participant shall have an hourly Regulation obligation equal to its pro rata share of the NEPOOL Control Area Regulation requirements for the hour, based on the Participant’s total Real-Time Load Obligation in the NEPOOL Control Area for the hour.  A Participant that does not meet its hourly Regulation obligation through its own Resources or through an appropriate bilateral transaction, shall be charged for Regulation dispatched by the ISO to meet such obligation at the Regulation Clearing Price determined in accordance with paragraph (c) of this Section, plus the amounts, if any, described in paragraph (f) of this Section.

#### A Participant supplying Regulation at the direction of the ISO in excess of its hourly Regulation obligation shall be credited for each increment of such Regulation at the higher of (i) the Regulation Clearing Price or (ii) the sum of the regulation offer and the unit-specific Opportunity Cost of the generating Resource supplying the increment of Regulation, as determined by the ISO in accordance with procedures specified in the NEPOOL Manuals and ISO Administrative Procedures.

#### The Regulation Clearing Price shall be determined by the ISO.  The Regulation Clearing Price for each hour shall be equal to the highest sum, for any generating Resources selected to provide Regulation, of a generating Resource’s Regulation offer plus its estimated unit-specific Opportunity Costs.

#### In determining the Regulation Clearing Price, the estimated unit-specific Opportunity Costs of a generating Resource offering to sell Regulation each hour shall be equal to the product of (i) the deviation of the set point of the generating Resource that is expected to be required in order to provide Regulation from the generating Resource’s expected output level if it had been dispatched in economic merit order multiplied by (ii) the absolute value of the difference between the expected Real-Time Price at the generation Node for the generating Resource and the offer price for energy from the generating Resource (at the megawatt level of the Regulation set point for the Resource) in the NEPOOL Market.

#### In determining the credit under subsection (b) to a Participant that is selected to provide Regulation and that actively follows the ISO’s Regulation signals and instructions, the unit-specific Opportunity Cost of a Resource shall be determined for each hour that the ISO requires a generating Resource to provide Regulation and shall be equal to the product of (i) the deviation of the generating Resource’s output necessary to follow the ISO’s Regulation signals from the generating Resource’s expected output level if it had been dispatched in economic merit order multiplied by (ii) the absolute value of the difference between the Real-Time Price at the generation Node for the generating Resource and the offer price for energy from the generating Resource (at the megawatt level of the Regulation set point for the Resource) in the NEPOOL Market.

#### The Regulation Clearing Price calculated Day-Ahead does not change based upon regulating unit adjustments made in Real-Time.  Any Real-Time Opportunity Costs plus Regulation Supply Offer costs that exceed the Day-Ahead RCP multiplied by the amount of Regulation provided are credited after the fact on a unit-specific basis.

#### Any amounts credited for Regulation in an hour in excess of the Regulation Clearing Price in that hour shall be allocated and charged to each Participant that does not meet its hourly Regulation obligation in proportion to its purchases of Regulation in MWhs during that hour.

### Operating Reserve Credits.

The following paragraphs describe the calculation of Operating Reserve Credits and Operating Reserve Charges.  Additional information on these calculations may be found in Appendix F of this Market Rule.

#### Except as otherwise provided for under Section 3.2.3(f), Participants’ Pool-Scheduled Resources capable of providing Operating Reserve or Replacement Reserve shall be credited as specified below (an “Operating Reserve Credit”) based on the prices offered for the operation of such Resources, provided that the Resources were available for the entire time specified in the Offer Data for such Resource.

#### The following determination shall be made for each Pool-Scheduled generating Resource that is scheduled in the Day-Ahead Energy Market: the total offered price for Start-Up and No-Load Fees and energy, determined on the basis of the Resource’s scheduled output, shall be compared to the total value of that Resource’s scheduled energy output as determined by the Day-Ahead Energy Market and the Day-Ahead Prices applicable to the relevant generation Node or External Node in the Day-Ahead Energy Market.  If the total offered price summed over all hours for the Operating Day exceeds the total value summed over all hours for the Operating Day, the difference shall be credited to the Participant.  In the case of a Dispatchable Load, the total bid price for energy consumption, determined on the basis of the Resource’s scheduled consumption, shall be compared to the total cost of that Resource’s scheduled energy consumption as determined by the Day-Ahead Energy Market and the Day-Ahead Prices applicable to the relevant load Node or External Node in the Day-Ahead Energy Market.  If the total cost summed over all hours for the Operating Day exceeds the total bid price summed over all hours for the Operating Day, the difference shall be credited to the Participant as an Operating Reserve Credit.

#### Except as otherwise provided for under Section 6.4.4, the sum of the foregoing credits calculated in accordance with Section 3.2.3(b) shall be the “Operating Reserve Charge” in the Day-Ahead Energy Market in each Operating Day.

#### The Operating Reserve Charge in the Day-Ahead Energy Market shall be allocated and charged to each Participant in proportion to the sum of its hourly Day-Ahead Load Obligation in for that Operating Day except that, any Operating Reserve Charge associated with Pool-Scheduled Resources scheduled in the Day-Ahead Energy Market for the provision of voltage or VAR support are charged in accordance with the provisions of Schedule 2 of the NEPOOL Tariff.

#### At the end of each Operating Day, the following determination shall be made for each synchronized Pool-Scheduled Resource of each Participant that operates as requested by the ISO and that is not committed for synchronized condensing:  the total offered price for Start-Up and No-Load Fees and market energy, determined on the basis of the lesser of the Resource’s (i) hourly metered output or (ii) requested output as determined by the ISO dispatch.  The total offered price shall be compared to the total value of that Resource’s energy in the Day-Ahead Energy Market plus any credit or charge for quantity deviations, at the ISO dispatch direction, from the Day-Ahead Energy Market during the Operating Day plus any credit(s) for Reserve Shortage Opportunity Costs during periods of declared Reserve Shortage Condition Pricing Events during the Operating Day.  If the total offered price exceeds the total value, the difference less any credit as determined pursuant to Section 3.(b) and less any amounts credited for Regulation in excess of the Regulation offer plus the generating Resource’s Opportunity Cost shall be credited to the Participant.  In the case of a Dispatchable Load, the total bid price shall be compared to the total cost of that Resource’s energy in the Day-Ahead Energy Market plus any credit or charge for quantity deviations, at the ISO dispatch direction, from the Day-Ahead Energy Market during the Operating Day.  The difference between a Participant’s Real-Time bid price and the sum of its Day-Ahead and Real-Time costs less any Day-Ahead Operating Reserve Credit shall be credited to the Participant as a Real-Time Operating Reserve Credit.  For purposes of this Section, a Participant is deemed to operate as requested by the ISO if it follows dispatch instructions.  A generator is considered to be following a dispatch instruction if the actual output of the generator is not greater than 10% above its desired dispatch point and is not less than 10% below its desired dispatch point for each interval in the hour.  Generators with an Economic Max less than or equal to 50 MWs are considered to be following a dispatch instruction if the actual output of the generator is not greater than 5 MWs above its desired dispatch point and is not less than 5 MWs below its desired dispatch point for each interval in the hour.  If the generator violates this criterion in any interval during the hour, the generator is considered to be not following dispatch instructions for the entire hour.

#### A Participant’s Pool-Scheduled Resource or Self-Scheduled Resource, the output of which is reduced or suspended at the request of the ISO for the purpose of maintaining appropriate levels of capacity or Spinning Reserve, shall be credited in an amount equal to the following:

For generating Resources, the credit for each hour of reduced or suspended operation is:

Posturing Credit = (PAG - AG)   x (ULMP - UB) – RC

where:

PAG equals the estimated hourly generation had the generator not responded to dispatch orders to reduce or suspend operation taking any limited energy restrictions into account, such estimated hourly generation to be determined in accordance with procedures defined in the NEPOOL Manuals;

AG equals the actual output of the generating Resource;

ULMP equals the Real-Time Price associated with the generating Resource that is reduced or suspended for each hour;

UB equals the Supply Offer price associated with PAG for that generating Resource whose output is reduced or suspended;

RC equals any Regulation credits from Section 3.(f); and

where ULMP - UB shall not be negative and Posturing Credit shall not be negative.

#### Except as otherwise provided for under Section 6.4.4, the sum of the foregoing Operating Reserve Credits, plus any Cancellation Fees paid in accordance with Section 1.(d), such Cancellation Fees to be applied to the Operating Day for which the unit was scheduled, shall be the non-synchronized condensing Operating Reserve Charge for the Real-Time Energy Market in each Operating Day.

#### The non-synchronized condensing and synchronized condensing Operating Reserve Charge for the Real-Time Energy Market for each Operating Day shall be allocated and charged to each Participant in proportion to the sum of the absolute values of its (i) Real-Time Load Obligation Deviations in MWhs during that Operating Day; (ii) generation deviations for Pool-Scheduled Resources not following ISO dispatch instructions, Self-Scheduled Resources with dispatchable increments above their Self-Scheduled amounts not following ISO dispatch instructions and Self-Scheduled Resources not following their Day-Ahead Self-Scheduled amounts other than those Self-Scheduled Resources that are following ISO dispatch instructions, including External Resources, in MWhs during the Operating Day; and (iii) deviations from the Day-Ahead Energy Market for External Transaction purchases in MWhs during the Operating Day except that, any Operating Reserve Charge associated with Pool-Scheduled Resources scheduled in the Real-Time Energy Market for the provision of voltage or VAR support are charged in accordance with the provisions of Schedule 2 of the NEPOOL Tariff. For the purposes of calculating generation deviations, if a

#### generation resource has been scheduled in the Day-Ahead Energy Market and the ISO determines that the unit should not be run in order to avoid a Minimum Generation Emergency, the generation owner will be responsible for all Real-Time Energy Market Deviation Energy Charges but will not incur generation related deviations for the purpose of allocating Real-Time Operating Reserve Charges.  For the purposes of calculating Real-Time Operating Reserve Charges, any difference between the actual consumption (Real-Time Load Obligation) of Dispatchable Loads and Dispatchable Load Demand Bids that clear in the Day-Ahead Energy Market that result from operation in accordance with the ISO’s instructions shall be excluded from the Participant Real-Time Load Obligation Deviation.

#### At the end of each Operating Day, Participants shall be credited on the basis of their offered prices for synchronized condensing for any hydropower or combustion turbine units operated as synchronous condensers but producing no energy.

#### The sum of the foregoing Operating Reserve Credits as specified in Section 3.(i) shall be the Operating Reserve Charge for synchronized condensing in the Real-Time Energy Market for the Operating Day in the NEPOOL Control Area.

#### [Reserved – Spinning Reserve.]

#### For any Operating Day in either, as applicable, the Real-Time Energy Market or the Day-Ahead Energy Market for which, for all or any part of such Operating Day, the ISO (i) declares an Emergency Condition; (ii) issues an alert that an Emergency Condition may be declared (“Emergency Condition Alert”); or (iii) schedules units based on the anticipation of an Emergency Condition or an Emergency Condition Alert, the Operating Reserve Credit otherwise provided by Section 3.(b) or Section 3.(e) shall be limited as provided in paragraphs (m) or (n), respectively.  The ISO shall provide timely notice on its internet site of the commencement and termination of any of the actions described in clause (i), (ii), or (iii) of this paragraph (l).

#### For the Real-Time Energy Market, if the Effective Offer Price (as defined below) is greater than $1,000/MWh, then such amounts above $1,000/MWh shall be used to reduce any Operating Reserve Credits received for generating Resources under Section 3.(e).  For purposes of this paragraph (m), the Effective Offer Price shall be the amount calculated for total offered price for such Operating Day pursuant to Section 3.(e) divided by the lesser of (i) hourly metered output or (ii) requested output as determined by the ISO dispatch, during the pool-scheduled hours determined under Section 3.(e).

#### For the Day-Ahead Energy Market, if the Effective Offer Price (as defined below) is greater than $1,000/MWh, then such amounts above $1,000/MWh shall be used to reduce any Operating Reserve credits received for generating Resources under Section 3.(b).  For purposes of this paragraph (n), the Effective Offer Price shall be the amount calculated for total offered price for such Operating Day pursuant to Section 3.(b) divided by the scheduled MWhs during the pool-schedule hours determined under Section 3.(b).

#### During Reserve Shortage Condition Pricing Events, certain Generators that provided Operating Reserve will be eligible for credits for Reserve Shortage Opportunity Costs.  Eligibility requirements for Generators are set forth in the NEPOOL Manuals. The Reserve Shortage Opportunity Cost credits for eligible Generators are calculated by comparing the Economic Maximum Limit of the Generator to the greater of its Desired Dispatch Point or its actual output. The difference is then multiplied by the difference between the LMP and the weighted average Offer Price for those MW.

The sum of the Reserve Shortage Opportunity Cost credits for each hour shall be allocated and charged to Participants based on each Participant’s ratio share of hourly Real-Time Load Obligation.

### 3.2.3A	  [Reserved - Spinning Reserve.]

### Transmission Congestion.

Participants shall be charged or credited for Congestion Costs as specified in Section 3.(f) of this Market Rule.

### [Reserved.]

### Emergency Energy.

#### Hourly net costs in excess of Real-Time Prices attributable to the purchase of Emergency energy by the ISO from other Control Areas shall be allocated to Participants based on the following hourly deviations where such deviations are negative:  (i) Real-Time Adjusted Load Obligation Deviations during that Operating Day; (ii) generation deviations for Pool-Scheduled Resources not following ISO dispatch instructions, Self-Scheduled Resources with dispatchable increments above their Self-Scheduled amounts not following ISO dispatch instructions and Self-Scheduled Resources not following their Day-Ahead Self-Scheduled amounts other than those Self-Scheduled Resources that are following ISO dispatch instructions, including External Resources, in MWhs during the Operating Day; and (iii) deviations from the Day-Ahead Energy Market for External Transaction purchases in MWhs during the Operating Day.  As provided for in the NEPOOL Manuals, generation Resources shall have a 5% or 5 MWh threshold when determining such deviations.

#### Hourly net revenues (excluding the cost of Ancillary Services and transmission) in excess of Real-Time Prices attributable to the sale of Emergency energy to other Control Areas shall be credited to Participants based on the following deviations where such deviations are negative:  (i) Real-Time Adjusted Load Obligation Deviations in MWhs during that Operating Day; (ii) generation deviations for Pool-Scheduled Resources following ISO dispatch instructions and Self-Scheduled generating Resources with dispatchable increments above their Self-Scheduled amounts following ISO dispatch instructions , including External Resources, in MWhs during the Operating Day; and (iii) deviations from the Day-Ahead Energy Market for External Transaction purchases in MWhs during the Operating Day. As provided for in the NEPOOL Manuals, generation Resources shall have a 5% or 5 MWh threshold when determining such deviations.

### Billing.

The ISO shall prepare a billing statement each billing cycle, in accordance with Attachment N to the NEPOOL Tariff, for each Participant in accordance with the charges and credits specified in Sections 3.2.1 through 3.2.6 of this Market Rule, and showing the net amount to be paid or received by the Participant. Billing statements shall provide sufficient detail, as specified in the NEPOOL Manuals, ISO Administrative Procedures and Attachment N to the NEPOOL Tariff, to allow verification of the billing amounts and completion of the Participant’s internal accounting.  Billing disputes shall be settled in accordance with procedures specified in Section 5 of Attachment N to the NEPOOL Tariff.

## [Reserved.]

## Non-Participant Transmission Customers.

### Transmission Congestion.

Non-Participant Transmission Customers shall be charged or credited for Congestion Costs as specified in Section 5 of this Market Rule.

### Transmission Losses.

Non-Participant Transmission Customers shall be charged or credited for transmission losses in an amount equal to the product of (i) the Transmission Customer’s MWhs of deliveries in the Real-Time Energy Market, multiplied by (ii) the difference between the Loss Components of the Real-Time Locational Marginal Prices at the point-of-receipt and the point-of-delivery Locations.

### Billing.

The ISO shall prepare a billing statement each billing cycle, in accordance with Attachment N to the NEPOOL Tariff, for each Non-Participant Transmission Customer in accordance with the charges and credits specified in Sections 3.4.1 through 3.4.2 of this Market Rule, and showing the net amount to be paid or received by the Non-Participant Transmission Customer.  Billing statements shall provide sufficient detail, as specified in the NEPOOL Manuals, the ISO Administrative Procedures and Attachment N to the NEPOOL Tariff, to allow verification of the billing amounts and completion of the Non-Participant Transmission Customer’s internal accounting.  Billing disputes shall be settled in accordance with procedures specified in Section 5 of Attachment N to the NEPOOL Tariff.

## [Reserved.]

## Data Reconciliation.

### Data Correction Billing.

The ISO will reconcile Participant data errors and corrections after the correction limit for such data has passed.  The correction limit for Participant supplied meter data and for ISO errors in the processing of meter and other Participant data is ninety (90) days from the date of initial billing of the last Operating Day of the month to which the data applied.

### Eligible Data.

The ISO will accept revised hourly asset meter readings from assigned meter readers and new or revised internal bilateral transactions from Participants.  No other revised data will be accepted for use in settlement recalculations.  The ISO will correct data handling errors associated with other Participant supplied data to the extent that such data did not impact unit commitment or the Real-Time dispatch.  Data handling errors that impacted unit commitment,  the Real-Time dispatch or ICAP auctions will not be corrected.

### Data Revisions.

The ISO will accept revisions to asset specific meter data and internal bilateral transactions at any time prior to the correction limit.  No revisions to other Participant data will be accepted after the deadlines for submittal of that data have passed.  If the ISO discovers a data error or if a Participant discovers and notifies the ISO of a data error prior to the correction limit, revised hourly data will be used to recalculate energy, Operating Reserve and Regulation.  No settlement recalculations or other adjustments may be made if the correction limit for the Operating Day to which the error applied has passed.

### Meter Corrections Between Control Areas.

For revisions to meter data associated with assets that connect the NEPOOL Control Area to other Control Areas, the ISO will, in addition to performing settlement recalculations, adjust the actual interchange between the NEPOOL Control Area and the other Control Area to maintain a proper record of inadvertent energy flow.

### Meter Correction Data.

#### Unless otherwise specified in the NEPOOL Manuals and ISO Administrative Procedures, revised meter data shall be submitted to the ISO as soon as it is available and not later than the correction limit.

#### Unless otherwise specified in the NEPOOL Manuals or the ISO Administrative Procedures, errors on the part of the ISO in the administration of Participant supplied data shall be brought to the attention of the ISO as soon as possible and not later than the correction limit.

## Eligibility for Billing Adjustments.

### Errors in Participant’s statements resulting from errors in settlement software, errors in data entry by ISO personnel, and Settlement production problems, that do not affect the day-ahead schedule or real-time system dispatch, will be corrected as promptly as practicable.  If errors are identified prior to the issuance of final statements, the market will be resettled based on the corrected information.

### Calculations made by scheduling or dispatch software, operational decisions involving ISO discretion which affect scheduling or real-time operation, and the ISO’s execution of mandatory dispatch directions, such as self-schedules or external contract conditions, are not subject to retroactive correction and resettlement.  The ISO will settle and bill day-ahead market as actually scheduled and the real-time market as actually dispatched.  Any post-settlement issues raised concerning operating decisions related to these markets will be corrected through revision of operations procedures and guidelines on a prospective basis.

### While errors in reporting hourly metered data may be corrected (see Section 3.6), Participants have the responsibility to ensure the correctness of all other data they submit to the market system and no adjustment will be made.

### Disputes between Participants regarding Settlement of internal bilateral transactions shall not be subject to adjustment by the ISO, but shall be resolved directly by the Participants unless they involve an error by the ISO that is subject to resolution under Section 3.7.1) above.

### Billing disputes between Participants and the ISO or Non-Participants and the ISO shall be settled in accordance with procedures specified in Section 5 of Attachment N to the NEPOOL Tariff.

# RATE TABLE

## Offered Price Rates.

Day-Ahead energy, Real-Time energy, Regulation, Operating Reserve, Congestion Cost and transmission loss costs are based on offers and bids submitted to the ISO as specified in this Market Rule.

## [Reserved.]

## Emergency Energy Transaction.

The pricing for Emergency energy purchases and sales will be determined in accordance with:

#### an applicable agreement with an adjacent Control Area for Emergency purchases and sales, or

#### arrangements made by the ISO with Participants, in accordance with procedures defined in the NEPOOL Manuals, to purchase Emergency energy offered by such Participant from Resources that are not ICAP Resources.  The ISO shall select offers to sell Emergency energy made by Participants to the ISO on a least cost basis and the selected Participants shall receive payment for energy delivered at the higher of their offer price or the Real-Time Price at the applicable External Node.  Such Emergency energy purchases from Participants shall not be eligible to set Real-Time Prices.

# CALCULATION OF TRANSMISSION CONGESTION REVENUE AND CREDITS

## Non-Participant Transmission Congestion Cost Calculation.

### Calculation by ISO.

When the transmission system is operating under constrained conditions, the ISO shall calculate Congestion Costs.

### General.

The basis for the Congestion Costs shall be the differences in the Congestion Component of the Locational Marginal Prices between points of delivery and points of receipt, as determined in accordance with Section 2 of this Market Rule.

### [Reserved.]

### Non-Participant Transmission Customer Calculation.

Congestion Costs shall be assessed for transmission use scheduled in the Real-Time Energy Market, calculated as the amount to be delivered in each hour multiplied by the difference between the Congestion Component of the hourly Real-Time Price at the delivery point or NEPOOL Control Area boundary delivery interface and the Congestion Component of the hourly Real-Time Price at the source point or NEPOOL Control Area boundary source interface.  Non-Participants using Point-to-Point Transmission Service for deliveries through the NEPOOL Control Area shall be included in the determination of the Congestion Costs.

## Transmission Congestion Credit Calculation.

### Eligibility.

Except as provided in Section 6.5 of Appendix A, each holder of a Financial Transmission Right shall receive as a Transmission Congestion Credit a proportional share of the total monthly Transmission Congestion Revenues collected.

### Financial Transmission Rights.

#### Transmission Congestion Credits will be calculated based upon the FTRs held at the time of the constrained hour.

#### FTRs shall be awarded to winning bidders in the FTR Auctions pursuant to Section 7 and may be acquired in the subsequent bilateral market from FTR Holders.

##### An entity that acquires an FTR through the FTR Auction shall automatically be recognized by the ISO as the registered FTR Holder of that FTR, subject to having already met the eligibility criteria for bidding in the FTR Auction.  The registered FTR Holder shall be entitled to receive or be obligated to make FTR payments arising from such FTR in accordance with Section 5.2.

##### An entity that acquires an FTR through the FTR Auction or through a subsequent bilateral transaction may elect to hold it, sell it in the FTR Auction or sell it bilaterally.  The registered FTR Holder of an FTR sold in a bilateral transaction will continue to be the FTR Holder for that FTR unless it submits a confirmation of the sale to the ISO in accordance with the NEPOOL Manuals and ISO Administrative Procedures.  The ISO upon receipt of such a confirmation will transfer record ownership.  The purchaser of an FTR in a bilateral transaction that is not recorded by the ISO receives only a contractual right against the seller of the FTR and has no rights or obligations in settlement or in the Energy market.  An entity who subsequently acquires an FTR from an FTR Holder through a bilateral transaction must meet applicable financial assurance criteria to be the FTR Holder of that FTR and secure the associated rights and obligations.  The ISO shall settle FTRs only with the registered FTR Holders.  At any given time, each FTR shall have only one registered FTR Holder.

### [Reserved.]

### Target Allocation to FTR Holders.

A target allocation of Transmission Congestion Credits for each FTR Holder shall be determined for all applicable FTRs.  Each FTR shall be multiplied by the Day-Ahead Price Congestion Component differences for the associated receipt and delivery points. This calculation will result in a positive or negative FTR target allocation.  All negative target allocations are obligations to pay.  All positive target allocations will be summed for an FTR Holder for the month and compensated as Transmission Congestion Credits under Section 5.2.5.

### Calculation of Transmission Congestion Credits.

#### The total Transmission Congestion Revenue available for the month shall be equal to the sum of:  (i) the hourly Day-Ahead Congestion Revenue amounts for the month, (ii) the hourly Real-Time Congestion Revenue amounts for the month plus Congestion Costs collected under Section 5.1.4, (iii) the negative FTR target allocations calculated under Section 5.2.4, and (iv) any excess Transmission Congestion Revenue from prior months.

#### The sum of all monthly positive target allocations, as determined in Section 5.2.4, shall be compared to the Transmission Congestion Revenue for the current month.  If the sum of all monthly positive target allocations is less than the Transmission Congestion Revenue for the current month, the Transmission Congestion Credit for each FTR Holder shall be equal to its total monthly positive target allocation.  All remaining Transmission Congestion Revenues from the current month shall be carried over into the following month.

#### If the sum of all the monthly positive target allocations is greater than the Transmission Congestion Revenue for the current month, each FTR Holder shall be assigned a share of the total Transmission Congestion Revenue for the current month in proportion to its total monthly positive target allocations.

### Distribution of Excess Congestion Revenue.

If there is any remaining Transmission Congestion Revenue at the end of the calendar year, this amount shall be proportionally allocated to any remaining unpaid monthly positive target allocations in any month of that year, but shall not exceed the amount of each unpaid monthly positive target allocation.  Any remaining surplus Transmission Congestion Revenue over and above the total of all unpaid monthly positive target allocations for the year shall be allocated to the entities who paid Congestion Costs in that calendar year in proportion to the amount of total Congestion Costs paid during the year.

# RELIABILITY MUST RUN RESOURCES

## Definition.  “

Daily RMR Resources” are those Resources identified by the ISO on a daily basis as necessary for the provision of Operating Reserve requirements and adherence to NERC, NPCC and NEPOOL reliability criteria over and above those Resources required to meet first contingency reliability criteria within a Reliability Region.

## Day-Ahead and Real-Time Energy Market.

When establishing operating schedules, the ISO will select and identify Resources (“Daily RMR Resources”) on a not unduly discriminatory basis in accordance with the procedures defined in the NEPOOL Manuals.  Appendix A will determine which, if any, Supply Offers will be adjusted.  The ISO will also record, in an auditable log, the reason the Resource was selected.

### Special Constraint Resources.

When establishing operating schedules, at the request of a Transmission Owner or distribution company in order to maintain area reliability, the ISO will commit and dispatch generating Resources to provide relief for constraints not reflected in the ISO’s systems for operating the NEPOOL Transmission System or the ISO’s operating procedures in accordance with the procedures defined in the NEPOOL Manuals.  The ISO will also record, in an auditable log, the designation of such generating Resource as a Special Constraint Resource and the name of the requesting Transmission Owner or distribution company. Any Operating Reserve Charge associated with the Real-Time operation of the Special Constraint Resource is charged in accordance with the provisions of Schedule 19 of the NEPOOL Tariff.

## [Reserved.]

## RMR Operating Reserve Charges.

### [Reserved.]

### [Reserved.]

### Calculation of RMR Operating Reserve Payments.

Day-Ahead and Real-Time Operating Reserve Credits for Daily RMR Resources are determined in accordance with the provisions of paragraphs (a), (b), (c) and (e) in Section 3.2.3, as applied to Pool-Scheduled Resources, but such credits shall not be included in Operating Reserve Charges pursuant to Section 3.2.3 and shall instead be allocated and charged in accordance with Section 6.4.4.  The Day-Ahead and Real-Time Operating Reserve Credits for Daily RMR Resources are subject to market power review and mitigation.

### Calculation of Daily RMR Resource Operating Reserve Charges and Allocation of Fixed Cost Charges Associated with RMR Contracts.

#### The Day-Ahead Operating Reserve Credits calculated in accordance with Section 6.4.3 for Daily RMR Resources are aggregated into an Operating Reserve Charge and charged pro rata to each Participant in proportion to the sum of its Day-Ahead Load Obligations in MWhs for that Operating Day for Locations within the affected Reliability Region.

#### The Real-Time Operating Reserve Credits calculated in accordance with Section 6.4.3 for Daily RMR Resources are aggregated into an Operating Reserve Charge and charged to each Participant in proportion to the sum of the absolute values of its (i) Real-Time Load Obligation Deviations in MWhs (excluding any difference between cleared Demand Bids in the Day-Ahead Energy Market and revenue quality meter readings for Dispatchable Load pumps that result from operation in accordance with the ISO’s instructions) during that Operating Day within the affected Reliability Region; and (ii) generation deviations from the Day-Ahead Energy Market for Pool-Scheduled Resources not following ISO dispatch instructions, Self-Scheduled Resources with dispatchable increments above their Self-Scheduled amounts not following ISO dispatch instructions and Self-Scheduled Resources not following their Day-Ahead Self-Scheduled amounts other than those Self-Scheduled Resources that are following ISO dispatch instructions  in MWhs during the Operating Day within the affected Reliability Region.  For hours for which an RMR Operating Reserve Charge is calculated and an Emergency energy sale is being made by NEPOOL, the amount (MWh) of Emergency energy sales will be included in the above calculation, with a proportional share attributable to the Emergency energy sale being added to the purchasing Control Area’s cost for Emergency energy.

#### Any monthly fixed-cost charges paid to Resources pursuant to Agreements negotiated under Appendix A, Section 6 and Exhibit 2 shall be allocated and charged pro rata to Participants and Non-Participants with Network Load in proportion to the sum of their Network Load during that month within the affected Reliability Region.

# FINANCIAL TRANSMISSION RIGHTS AUCTIONS

## Auctions of Financial Transmission Rights.

Periodic auctions (“FTR Auctions”) to allow Eligible FTR Bidders to acquire or FTR Holders to sell FTRs shall be conducted by the ISO in accordance with the provisions of this Section.  Non-Participants that want to participate in the FTR Auction or to become an FTR Holder via the secondary market and have satisfied the applicable financial assurance criteria will be charged a one time FTR Registration Fee of $5,000.  This fee may be superceded by a future provision in the ISO Tariff.

### Auction Period and Scope of Auctions.

#### Initially, FTR Auctions shall be held on a monthly basis followed by the introduction of long-term FTR Auctions.  Long-term auctions shall be introduced no later than seven (7) months after the SMD Effective Date.  The ISO shall provide notice of the initial long-term auction at least thirty (30) days prior to the opening of the auction-quoting period for such long-term auction as described in Section 7.1.2(a).  At the time of such notice, the ISO shall post a schedule for future long-term auctions and the percent of the feasible FTRs that will be available in such long-term auctions.  Such schedule shall coordinate the start and end dates of the long-term FTRs to be auctioned with those of the long-term FTRs of neighboring Control Areas.  During the period prior to the long-term auctions, the entire transfer capability of the NEPOOL Transmission System will be made available to support the sale of FTRs with a term of one month in the monthly FTR Auctions.

#### Following the initial auctions described above, FTR auctions shall be held on both an annual and monthly basis.  Fifty percent of the feasible FTRs that can be made available with a term of one year shall be made available in the annual FTR Auction.  After the annual FTR Auction has been conducted, the remaining feasible FTRs, each having a term of one month, shall be made available in the monthly FTR Auctions.  Within two years from the SMD Effective Date the ISO will evaluate making available FTRs with a term of more than one year (in one-year increments).

### Frequency and Time of FTR Auctions.

#### Annual (initial long-term) auctions:  The bid and offer period shall open five business days before the first business day of the month preceding the period in which the FTRs are to be effective for a five business day auction-quoting period that closes at noon on the fifth business day.

#### Monthly auctions:  The bid and offer period shall open beginning fifteen business days before the first business day of the month preceding the period in which the FTRs are to be effective for a five business day auction-quoting period that closes at noon on the fifth business day.

## Financial Transmission Rights Characteristics.

### Reconfiguration of Financial Transmission Rights.

Using an appropriate linear programming model, the ISO shall reconfigure the FTRs offered or otherwise available for sale in any auction to maximize the value to the bidders of the FTRs sold, provided that any FTRs acquired at auction shall be simultaneously feasible in combination with those FTRs outstanding at the time of the auction and not sold in the auction.  The linear programming model shall, while respecting transmission constraints and the maximum megawatt quantities of the bids and offers, select the set of simultaneously feasible FTRs with the highest total auction value as determined by the bids of buyers and taking into account the reservation prices of the sellers.

### Specified Locations.

Auction bids for FTRs may specify any combination of receipt and delivery locations represented in the State Estimator model for which the ISO calculates and posts Locational Marginal Prices.  Auction bids may specify receipt and delivery points from locations outside of the NEPOOL Control Area to locations inside the NEPOOL Control Area, from locations within the NEPOOL Control Area to locations outside of the NEPOOL Control Area, or to and from locations within the NEPOOL Control Area.  Congestion over interfaces associated with non-PTF external tie lines is not subject to LMP-based congestion management and, therefore, no FTRs across such interfaces will be included in the FTR Auctions.

### Transmission Congestion Revenues.

FTRs shall entitle holders thereof to credits only for Transmission Congestion Revenue, and shall not confer a right to credits for payments arising from or relating to transmission congestion made to any entity other than the ISO.

### [Reserved.]

## Auction Procedures.

### Role of the ISO.

FTRs auctions shall be conducted by the ISO in accordance with standards and procedures set forth in the NEPOOL Manuals and ISO Administrative Procedures, such standards and procedures to be consistent with the requirements of this Market Rule.

### [Reserved.]

### [Reserved.]

### On-Peak and Off-Peak Periods.

The ISO will conduct separate auctions simultaneously for on-peak and off-peak periods.  On-peak FTRs shall cover the periods from 7:00 a.m. up to the hour ending at 11:00 p.m. on Mondays through Fridays, except holidays as defined in the NEPOOL Manuals and ISO Administrative Procedures.  Off-peak FTRs shall cover the periods from 11:00 p.m. up to the hour ending 7:00 a.m. on Mondays through Fridays and all hours on Saturdays, Sundays, and NERC holidays as defined in the NEPOOL Manuals and ISO Administrative Procedures.  Each bid shall specify whether it is for an on-peak or off-peak period.

### Offers and Bids.

#### Offers to sell and bids to purchase FTRs shall be submitted during the applicable period set forth in Section 7.1.2, and shall be in the form specified by the ISO in accordance with the requirements set forth below.

#### Offers to sell shall identify the specific FTRs, by megawatt quantity and receipt and delivery points, offered for sale.  An offer to sell a specified megawatt quantity of FTRs shall constitute an offer to sell a quantity of FTRs equal to or less than the specified quantity.  An offer to sell may not specify a minimum quantity being offered.  Each offer may specify a reservation price, below which the offeror does not wish to sell the FTR.  Offers shall be subject to such applicable standards for the financial assurance of the offeror or for the posting of security for performance as the ISO shall establish.

#### Bids to purchase shall specify the megawatt quantity, price per megawatt, and receipt and delivery points of the FTR that the bidder wishes to purchase.  A bid to purchase a specified megawatt quantity of FTRs shall constitute a bid to purchase a quantity of FTRs equal to or less than the specified quantity.  A bid to purchase may not specify a minimum quantity that the bidder wishes to purchase.  A bid to purchase may not specify a negative price per MW.  A bid may specify as receipt or delivery points any Location for which the ISO calculates and posts Locational Marginal Prices in accordance with Section 2 of this Market Rule and may include FTRs for which the associated Transmission Congestion Credits may have negative values.  Bids shall be subject to such applicable standards for the financial assurance of the bidder or for the posting of security for performance as the ISO shall establish.

#### Bids and offers shall be specified to the nearest 0.1 megawatt and the quantity shall be greater than zero.

### Determination of Winning Bids and Clearing Price.

#### At the close of each bidding period, the ISO will create a base FTR power flow model that includes all outstanding FTRs that have previously been awarded for the period for which the auction was conducted and that were not offered for sale in the auction.  The base FTRs model also will include estimated uncompensated parallel flows into each interface point of the NEPOOL Control Area and estimated scheduled transmission outages.

#### In accordance with the requirements of this Section and subject to all applicable transmission constraints and reliability requirements, the ISO shall determine the simultaneous feasibility of all outstanding FTRs not offered for sale in the auction and of all FTRs that could be awarded in the auction for which bids were submitted.  The winning bids shall be determined from an appropriate linear programming model that, while respecting transmission constraints and the maximum megawatt quantities of the bids and offers, selects the set of simultaneously feasible FTRs with the highest net total auction value as determined by the bids of buyers and taking into account the reservation prices of the sellers.  In the event that there are two or more identical bids for the selected FTRs and there are insufficient FTRs to accommodate all of the identical bids, then each such bidder will receive a pro rata share of the FTRs that can be awarded.

#### FTRs shall be sold at the market-clearing price for FTRs between specified pairs of receipt and delivery points, as determined by the bid value of the marginal FTR that could not be awarded because it would not be simultaneously feasible.  The linear programming model shall determine the clearing prices of all FTR paths based on the bid value of the marginal FTRs, which are those FTRs with the highest bid values that could not be awarded fully because they were not simultaneously feasible, and based on the flow sensitivities of each FTR’s path relative to the marginal FTRs’ paths flow sensitivities on the binding transmission constraints.

### Announcement of Winners and Prices.

Within four (4) business days after the close of a monthly auction and six (6) business days after the close of an annual or initial long-term auction or such later time as may be approved by the ISO Board, the ISO shall post the winning bidders, the megawatt quantity, and the receipt and delivery points for each FTR awarded in the auction and the price at which each FTRs was awarded.  Results of the on-peak auction and off-peak auction will be posted separately.  The ISO shall not disclose the price specified in any bid to purchase or the reservation price specified in any offer to sell.

### Auction Settlements.

All buyers and sellers of FTRs between the same points of receipt and delivery shall pay or be paid the market-clearing price, as determined in the auction, for such FTRs.

### Allocation of Auction Revenues.

All auction revenues, net of payments to entities selling FTRs into the auction, shall be allocated as specified under Appendix C of this Market Rule.

### Simultaneous Feasibility.

The ISO shall make the simultaneous feasibility determinations specified herein using appropriate power flow models of contingency-constrained dispatch.  Such determinations shall take into account outages and expected configuration of transmission facilities and outages of individual generating units to the extent that such outages impact voltage or stability limits and shall be based on reasonable assumptions about the configuration and availability of transmission capability during the period covered by the auction.  The goal of the simultaneous feasibility determination shall be to ensure that there are sufficient Transmission Congestion Revenues to satisfy all FTR obligations for the auction period under expected conditions.

### [Reserved.]

### Financial Transmission Rights in the Form of Options.

When the ISO has the necessary software and hardware, the FTR Auctions shall allow for the acquisition of FTRs that do not create potential obligations to pay.

### FTR Secondary Trading Market.

FTR Holders may trade FTRs on the secondary market and have these settled using the ISO systems.  The ISO systems shall only allow FTRs to be sub-divided into multiple FTRs with; i) the same points of injection and withdrawal; ii) different megawatt amounts the sum of which does not exceed the original FTR MW amount; and iii) different start and end dates where the start and end dates are the same as or within the start and end dates of the original FTR.  FTRs may be reconfigured only through FTR Auctions.

# INSTALLED CAPACITY

## Annual Installed Capacity Requirement.

NEPOOL shall calculate the Summer Capability Period and Winter Capability Period Installed Capacity Requirement prior to the beginning of each Capability Year.  In the event that NEPOOL is unable to establish an Installed Capacity Requirement prior to the date three months prior to the start of the Capability Year, the Summer Capability Period and Winter Capability Period Installed Capacity requirement for the upcoming Capability Year shall be established by the ISO in consultation with NEPOOL Participants.  The ISO shall translate the Summer Capability Period and Winter Capability Period Installed Capacity Requirement into a Summer Capability Period and Winter Capability Period Unforced Capacity Requirement for the NEPOOL Market.  The NEPOOL Unforced Capacity Requirement for the Summer Capability period or Winter Capability Period shall equal the Installed Capacity Requirement in the Summer Capability Period or Winter Capability Period times the quantity one minus the system-wide weighted average equivalent forced outage rate of the generating assets located within the NEPOOL Control Area.  The system-wide forced outage rate shall be calculated in accordance with the NEPOOL Manuals.  The ISO shall determine the amount of Unforced Capacity that must be sited within the NEPOOL Control Area and the amount of Unforced Capacity that may be procured from areas External to the NEPOOL Control Area in a manner consistent with the NEPOOL System Rules.

## Requirements Applicable to Participants.

### Allocation of the NEPOOL Control Area Unforced Capacity Requirement to Participants.

The ISO shall assign each Participant an Unforced Capacity obligation prior to the beginning of each Capability Year, and shall update this allocation monthly throughout the Capability Year.  Each Participant’s Unforced Capacity obligation for each month of the  Capability Year will equal the product of: (i) the Summer Capability Period or Winter Capability Period Unforced Capacity Requirement, as applicable, for the Obligation Month; and (ii) the Participant’s pro-rata share of the sum of all Participant annual coincident contributions to the NEPOOL annual peak load from the calendar year immediately prior to the Capability Year, as calculated in accordance with Section 8.2.3 of this Market Rule.    Each month, as customers are gained and lost by Participants through Load-shifting, the ISO will adjust the requirement for each Participant such that (i) the total NEPOOL Unforced Capacity Requirement for the Summer Capability Period and Winter Capability Period remains constant and (ii) an individual Participant’s Unforced Capacity obligation reflects the gains and losses.  In addition, adjustments will be made to Participant Unforced Capacity obligations to account for customers entering and permanently leaving the NEPOOL Control Area.  The net impact of customer entry and exit shall be proportionately allocated to Participants in accordance with the procedures in the NEPOOL Manuals, such that the total Unforced Capacity Requirement for the Summer Capability Period and Winter Capability Period remains constant.  Given the fixed Summer Capability Period and Winter Capability Period Unforced Capacity Requirement, net increases in customer peak contribution shall reduce the Unforced Capacity obligation of individual Participants, while net decreases in customer peak contribution shall increase individual Participant Unforced Capacity obligations.

### Participant Obligations.

Each Participant must obtain Capacity Credits, determined in accordance with the NEPOOL Manuals, or self-supply, or procure Unforced Capacity in an amount equal to its Summer Capability Period or Winter Capability Period Unforced Capacity obligation, as applicable, from any ICAP Resource through bilateral transactions and/or purchases in ISO-administered installed capacity auctions.  Each Participant must demonstrate that it owns or has obtained a sufficient amount of Unforced Capacity prior to the beginning of each Obligation Month. To satisfy this obligation, each Participant must submit information on all bilateral purchases and sales of Unforced Capacity to the ISO by the date specified in the NEPOOL Manuals.  This information shall be submitted by the Participants in the format and include all the information prescribed by the NEPOOL Manuals.  Participants that fail to timely satisfy their Unforced Capacity obligations, or that fail to make timely submissions of bilateral transactions for Unforced Capacity in accordance with the schedule set forth in the NEPOOL Manuals shall be required to participate in a Deficiency Auction pursuant to Section 8.5 of this Market Rule.

### Participant Peak Load and Load-Shifting Adjustments.

The NEPOOL Manuals and ISO Administrative procedures set forth the procedures for settlement data to be submitted to the ISO, including data for identifying each Participant’s Unforced Capacity responsibility for customers at each Load Asset.  For the purposes of determining a Participant’s pro-rata share of the Unforced Capacity Requirement in Section 8.2.1, a Participant’s monthly peak contribution shall be calculated in accordance with the following two steps, which are further detailed in the NEPOOL Manuals:

#### Calculate the sum of the annual coincident peak contributions from the prior calendar year of the customers served by the Participant at each Load Asset coincident with the annual NEPOOL peak of the prior calendar year.

#### Sum the peak loads in (a) for each Participant across all Load Zones.

#### The following loads are exempt from the Unforced Capacity requirements and are assigned a peak contribution of zero for the purposes of assigning obligations and tracking load shifts:

##### Load associated with pumping of pumped hydro generators, if the resource was pumping;

##### Station service load that is modeled as a discrete Load Asset and the Resource is complying with the maintenance scheduling procedures of the ISO for ICAP Resources and non-ICAP Resources as applicable; and

##### Transmission losses associated with delivery of energy over the Control Area tie lines.

The ISO shall account for Load-shifting among Participants each month using the best available information provided to it and the affected Participants by the entities responsible for submitting settlement data in accordance with the NEPOOL Manuals.  By the time specified in the NEPOOL Manuals, load-gaining Participants must procure sufficient Unforced Capacity to meet their increased Unforced Capacity obligation for the nearest following month, and load-losing Participants may sell Unforced Capacity that is no longer needed to satisfy their Unforced Capacity obligation.  The ISO shall provide each Participant with a revised Unforced Capacity obligation for the following month.

## Requirements Applicable to ICAP Resources.

### ICAP Resource Qualification Requirements.

#### In order to qualify as an ICAP Resource in the NEPOOL Control Area generating units, Dispatchable Loads and External Resources which have agreed to certain curtailment conditions as set forth in the last paragraph of Section 8.3.2 below, and other than Special Case Resources shall:

##### provide information reasonably requested by the ISO including the name and location of generators, and Dispatchable Loads providing the Installed Capacity;

##### in accordance with the NEPOOL Manuals, perform Installed Capacity audit tests and submit the results to the ISO, or provide to the ISO appropriate historical production data;

##### for ICAP Resources internal to the NEPOOL Control Area abide by the ISO generator maintenance coordination procedures. External ICAP Resources shall comply with the maintenance coordination procedures applicable to ICAP resources in the External Control Area, and ISO-NE will attempt to coordinate maintenance schedules of External ICAP Resources with the External Control Area;

##### provide the expected return date from any outages (including partial outages) to the ISO;

##### if the Resource is a Generator or External Resource the Participant must commit that it will submit a Supply Offer into the Day-Ahead Energy Market as described in Section 1.10.1A of this Market Rule, unless and to the extent that the generator or External Resource is unable to do so due to an outage as defined in the NEPOOL Manuals or due to temperature related de-ratings. Participants must also submit Offer Data, which specifies an Economic Maximum Limit;

##### if the Resource is a Dispatchable Load, the Participant must commit that it will submit a Demand Bid which specifies the price at which it is willing to be interrupted;

##### provide Operating Data in accordance with Section 8.3.5 of this Market Rule; and

##### comply with the NEPOOL Manuals.

#### The ISO shall inform each potential ICAP Resource that is required to submit Installed Capacity audit data of its approved ratings in accordance with the NEPOOL Manuals.

### Additional Provisions Applicable to External ICAP Resources.

Requirements to qualify as an ICAP Resource, for External Resources and External Transactions associated with Control Area to Control Area ICAP transactions that have agreed not to curtail the energy associated with such Installed Capacity, shall be established here and in the NEPOOL Manuals.

#### External Resources may offer UCAP into the NEPOOL Control Area in five ways:

##### External dispatchable energy backed by a Unit;

##### External non-dispatchable energy backed by a Unit. Energy must be scheduled a minimum of 16 on-peak hours during week days that are not holidays or as specified in the NEPOOL Manuals.

##### External non-dispatchable energy backed by a Control Area.  Energy must be scheduled a minimum of 16 on-peak hours during week days that are not holidays or as specified in the NEPOOL Manuals.

##### External dispatchable energy backed by a Control Area.  The Installed Capacity Equivalent of the contract shall be offered into the Day-Ahead market consistent with the requirements for a unit-backed contract.

##### Hydro Quebec Interconnection Capability Credits backed by either an emergency interchange agreement or other emergency supply arrangement between the NEPOOL Control Area and the Hydro Quebec Control Area over the HQ Interconnection.

#### The NEPOOL Participant entering the external ICAP transaction shall be responsible for supplying either the GADS data, data equivalent to GADS data, or other Operating Data to the ISO each month in accordance with the NEPOOL Manuals necessary to calculate an EFORd for unit-backed transactions, or for directly supplying the EFORd for the unit backing the transaction calculated in accordance with the formulas in the NEPOOL Manuals, and subject to verification with the External Control Area where the unit is located.  In the case of transactions backed by an External Control Area, CARL Data shall be submitted in support of the transaction.  The CARL data shall be used to assess the ability of the External Control Area to deliver energy in support of the UCAP transaction and to calculate the EFORd of the transaction.  For an energy contract backed by a Control Area to qualify as an ICAP Resource, the Control Area shall afford the contract the same curtailment priority as its native load. The EFORd data shall be used to reduce the energy face value of the contract from an ICAP value to a UCAP value for purposes of meeting the UCAP obligations of the Participant, consistent with the treatment of internal resources.  The UCAP value of the contract is determined by multiplying the ICAP value of the contract by the quantity one minus the EFORd of the transaction, where the EFORd of the transaction is the rolling twelve month average EFORd of the External Control Area. External ICAP Resources are required to bid the Installed Capacity Equivalent of the UCAP value given to such resources, as described in the NEPOOL Manuals.

#### Certain external ICAP Resources are afforded Grandfathered status with special treatment that is described in the NEPOOL Manuals. Those Resources and their associated MW values are identified in the following table:

| Contract Description | Grandfathered (MW) | Contract End Date |
| --- | --- | --- |
| NYPA: NY – NE: CMEEC NYPA: NY – NE: MMWEC NYPA: NY – NE: Pascoag NYPA: NY – NE: VELCO | 20.9 81.8 2.4 15.0 120.1 | 10/31/2003 10/31/2003 10/31/2003 10/31/2003 |
| VJO: Highgate – NE VJO: Phase I/II – NE VJO: CCC Block Load | Up to 225* Up to 110* Up to 60* | 2020 |
| VPPSA NYSEG: NY-NE* | 6 (April-Oct) 7 (Nov-March) | 10/31/2003 |
| BED NYSEG: NY – NE* | 10 | 12/31/2009 |
| Select: New Brunswick | 224 | 2020 |

- * The total grandfathered MW for the VJO contract are limited to 335MW

### ICAP Resource Outage Scheduling Requirements.

All Participants with ICAP Resources, except for Dispatchable Load Resources, and Special Case Resources, that intend to supply Unforced Capacity to the NEPOOL Control Area shall submit a confidential notification to the ISO of their proposed generator outage schedules in accordance with the NEPOOL Manuals.  Based upon a reliability assessment, if Operating Reserve deficiencies are projected to occur in certain weeks for the upcoming calendar year, the ISO will request voluntary re-scheduling of outages.  In the case of Participants with Generators listed as ICAP Resources in accordance with the provisions of Section 8.3.4 of this Market Rule and the NEPOOL Manuals, if voluntary re-scheduling is ineffective, the ISO will invoke forced re-scheduling of their outages to ensure that projected Operating Reserve over the upcoming year is adequate.  A Participant that refuses a forced rescheduling of its outages for any Generator listed as an ICAP Resource shall be prevented from supplying Unforced Capacity in the NEPOOL Control Area with that Generator during any month where it undertakes such outages.  The re-scheduling process is described in the NEPOOL Manuals.  A Participant that intends to supply Unforced Capacity in a given month from a Resource that was not listed as an ICAP Resource prior to the beginning of the Capability Period must notify the ISO in accordance with the NEPOOL Manuals so that it may be subject to forced re-scheduling of its proposed outages in order to qualify as an ICAP Resource.  A Participant that refuses the ISO’s forced rescheduling of its proposed outages shall not qualify as an ICAP Resource for that unit for any month during which it schedules or conducts an outage. Participants with Dispatchable Load Resources shall notify the ISO in accordance with the NEPOOL Manuals of outages that would reduce their ability to interrupt.  Dispatchable Load Resources must also submit to the ISO a written commitment that any outages that would reduce their ability to interrupt without reducing their Load by a corresponding amount will only be conducted in accordance with the NEPOOL Manuals.  The external Control Area and the ISO, in accordance with the NEPOOL Manuals, shall coordinate outage schedules for External Resources.

### Required Notification That an ICAP Resource Has Been De-listed.

Each Participant must notify the ISO in accordance with the dates specified in the NEPOOL Manuals of any Unforced Capacity it wishes to de-list as a qualified ICAP Resource.  The de-listed Unforced Capacity may be sold as a capacity-based product for use in a Control Area external to the NEPOOL Control Area or the Resource may operate as an energy-only resource.  In the event that a Resource is de-listed from the UCAP market, the owner of the Resource is not excused from the requirement to offer energy from the Resource if it is available.  Energy from the de-listed Resource may be offered to either the NEPOOL Control Area in either the Day-Ahead Energy Market and/or Real-Time Energy Market or to an external Control Area  Any outage while de-listed shall be accurately accounted for in accordance with NERC GADS procedures and the NEPOOL Manuals in the event that the unit is re-listed as an ICAP Resource.

Resources may only be de-listed in accordance with the procedures and requirements specified in the NEPOOL Manuals.  Only whole Resources may be de-listed.

For a given UCAP Obligation Month, a Resource may be de-listed at any time prior to the start of the Obligation Month, subject to minimum notification requirements specified in the NEPOOL Manuals.  All listing and de-listing actions shall be effective as of the first of the applicable Obligation Month.  In the event that a listing or de-listing notification is received prior to the Deficiency Auction for a Obligation Month, subject to minimum notification requirements, the listing or de-listing action shall be binding with respect to satisfying the Participant’s UCAP obligation for that Obligation Month.  Any listing notification received prior to the Deficiency Auction but after the date established by the minimum notification requirements as specified in the NEPOOL Manuals shall not count toward satisfaction of UCAP obligations for that Obligation Month.  Any request to de-list a Resource after the Deficiency Auction but prior to the start of the Obligation Month shall be subject to confirmation of the UCAP position of the Participant requesting the de-listing.  If a de-listing request would cause a Participant to become deficient with respect to its UCAP obligation, the request will be denied.  Only requests to de-list whole Resources that are in excess of the total Resources required to meet the Participant’s UCAP obligation shall be honored after the Deficiency Auction.

### Operating Data Reporting Requirements.

To qualify as ICAP Resources in the NEPOOL Control Area, Participants shall submit GADS Data, data equivalent to GADS Data, or other Operating Data to the ISO each month in accordance with the NEPOOL Manuals for Generators, Dispatchable Loads, and Special Case Resources providing Unforced Capacity.  Participants that do not submit Operating Data in accordance with this subsection and the NEPOOL Manuals shall be subject to the sanctions provided in Appendix B of this Market Rule.  ICAP Resources that were not in operation at the beginning of a Capability Year shall submit Operating Data to the ISO no later than one month after such Resources commence commercial operation, and in accordance with the NEPOOL Manuals.

### Operating Data Default Value and Collection.

#### Monthly Calculations.  The ISO shall calculate each month for each ICAP Resource the amount of Unforced Capacity that it is qualified to supply in the NEPOOL Control Area based on a rolling twelve-month calculation in accordance with formulae provided in the NEPOOL Manuals.  The ISO shall provide this information to each Participant prior to the monthly auctions in accordance with the dates set forth in the NEPOOL Manuals.  The amount of Unforced Capacity that each generator, Dispatchable Load Resource, and External Resource is authorized to supply in the NEPOOL Control Area shall be based on the ISO’s calculations of individual Equivalent Demand Forced Outage Rates.  The amount of Unforced Capacity that each Intermittent Power Resource is authorized to supply in the NEPOOL Control Area shall be based on the individual historical capacity factor adjusted by the ISO to remove the effects of outages in accordance with the rating process defined in the NEPOOL Manuals.  The ISO shall calculate the Equivalent Demand Forced Outage Rates and capacity factors monthly using a twelve-month rolling average of Operating Data in accordance with formulae provided in the NEPOOL Manuals.

#### Default Unforced Capacity.  In its calculation of Unforced Capacity, the ISO shall deem an ICAP Resource to be completely forced out for each month for which the Participant has not submitted its Operating Data in accordance with Section 8.3.5 of this Market Rule and the NEPOOL Manuals.  For an ICAP Resource that has been deemed completely forced out for a particular month, the Participant may submit new Operating Data, for that month, to the ISO at any time. The ISO will use such new Operating Data when calculating, in a timely manner in accordance with the NEPOOL Manuals, a new rolling average for the ICAP Resource.  Upon a showing of extraordinary circumstances, the ISO retains the discretion to accept at any time Operating Data which have not been submitted in a timely manner, or which do not fully conform with the NEPOOL Manuals.

#### Exception for Certain Equipment Failures.  When a Generator, Special Case Resource, or External Resource is forced into an outage by an equipment failure that involves equipment located on the high voltage side of the electric network beyond the step-up transformer, and including such step-up transformer, the outage will not be counted for purposes of calculating that Resource’s Equivalent Demand Forced Outage Rate.

### Availability Requirements.

#### Subsequent to qualifying, each ICAP Resource shall, except as noted in Section 8.3.11 of this Market Rule, on a daily basis: (i) Self-Schedule the Resource; (ii) Offer energy in each hour of the Day-Ahead Energy Market in accordance with the applicable provisions of Section 8.3.1 of this Market Rule; or (iii) notify the ISO of any outages. The total amount of energy that an ICAP Resource Self-Schedules, offers to the pool, or declares to be unavailable on a given day must equal or exceed the seasonal Installed Capacity Equivalent of the Unforced Capacity it supplies.

(b)	Participants utilizing Hydro Quebec Interconnection Capability Credits to meet all or part of their UCAP obligations are not required to submit an offer or Self-Schedule for the energy equivalent of the Capacity Credit.

### Unforced Capacity Sales.

Each ICAP Resource will be authorized to supply an amount of Unforced Capacity during each Obligation Month based on calculations in Section 8.3.6 and on the rating process described in the NEPOOL Manuals.  During the Summer Capability Period the ISO will use the summer rating of the Resource to calculate the amount of Unforced Capacity the Resource may supply, and during the Winter Capability Period the winter rating will be used to calculate the amount of Unforced Capacity the Resource may supply.  If an ICAP Resource’s Installed Capacity rating is determined to have increased during an Obligation Month, pursuant to testing procedures described in the NEPOOL Manuals,

the amount of Unforced Capacity that it shall be authorized to supply in that or future Obligation Months shall also be increased on a prospective basis in accordance with the schedule set forth in the NEPOOL Manuals.  New Generators and Generators that have increased their capacity due to changes in their generating equipment may qualify to supply Unforced Capacity on a foregoing basis based upon an Installed Capacity audit test that is performed and reported to the ISO in accordance with the rating process described in the NEPOOL Manuals.  Any ICAP Resource, except as noted in Section 8.3.11 of this Market Rule, which fails on a daily basis to schedule, offer or bid, or declare to be unavailable in the Day-Ahead Energy Market an amount of energy equal to its Installed Capacity during the Summer Capability Period or its Installed Capacity during the Winter Capability Period, as applicable, as established through testing in accordance with the NEPOOL Manuals, rounded down to the nearest whole MW, is subject to sanctions pursuant to Appendix B of this Market Rule.  If an entity other than the owner of the ICAP Resource that is providing Unforced Capacity is responsible for fulfilling submission of Supply Offer or Demand Bid data, scheduling, and notification requirements, the owner and that entity must designate to the ISO which of them will be responsible for complying with the scheduling, Supply Offer or Demand bid data submission, and notification requirements. The designated entity shall be subject to sanctions pursuant to Appendix B of this Market Rule.

### Curtailment of External Transactions.

Any Unforced Capacity that is not out of service or scheduled in the Day-Ahead Energy Market may be scheduled to supply energy for use in External Transactions provided, however, that such External Transactions shall be subject to curtailment within the hour, consistent with NEPOOL Manuals.  Such curtailment shall not exceed the Installed Capacity Equivalent committed to the NEPOOL Control Area.  If an ICAP Resource’s External Transaction is Curtailed in-hour to resolve a reserve shortage within the NEPOOL Control Area, the Participant scheduling such transaction shall be paid, for the remainder of the hour, the Real-Time generator nodal price. New England will only recall External Transactions associated with a non-ICAP Resources due to unavailability of the resource backing that transaction, or in accordance with system emergency procedures as defined in the NEPOOL Tariff.

### [Reserved.]

### Special Case Resources.

#### Limited Energy Resources.  A Limited Energy Resource may qualify as an ICAP Resource if it offers or Self-Schedules its Installed Capacity

Equivalent into the Day-Ahead Energy Market each day and if it is able to provide the energy equivalent of the Unforced Capacity in accordance with the rating process set forth in the NEPOOL Manuals.  Limited Energy Resources shall also offer an Economic Maximum Limit, designating desired operating limits.  Limited Energy Resources that are not scheduled in the Day-Ahead Energy Market to operate at a level above their bid-in Economic Maximum Limit may be called in Real-Time pursuant to a manual intervention by ISO dispatchers, who will account for the fact that Limited Energy Resource may not be capable of responding.

#### Intermittent Power Resources.  Intermittent Power Resources may qualify as ICAP Resources without having to comply with the daily bidding and scheduling requirements set forth in Section 8.3.7 of this Market Rule and may claim up to their Installed Capacity as Unforced Capacity in accordance with the rating procedures set forth in the NEPOOL Manuals.  To qualify as ICAP Resources, Intermittent Power Resources shall comply with the notification requirements of Section 8.3.7(a)(iii) of this Market Rule.  In calculating Unforced Capacity for an Intermittent Power Resource, the historical capacity factor will be adjusted to remove the effects of outages in accordance with the NEPOOL Manuals.

## Unforced Capacity Auctions.

### General Auction Requirements.

The ISO will administer Unforced Capacity auctions to accommodate Participant’s efforts to enter into Unforced Capacity transactions and to give Participants an opportunity to satisfy their Unforced Capacity obligations.  The ISO shall conduct regular auctions at the times specified in this Section and the NEPOOL Manuals.  Unforced Capacity purchased in Unforced Capacity auctions may not be sold for the purposes of meeting Installed Capacity requirements imposed by operators of external Control Areas.  Offers to sell and bids to purchase Unforced Capacity shall be made in $/kw-month for the time period appropriate to the auction.  The ISO shall impose no limits on bids or offers in any auction, except to the extent required by any applicable market mitigation measures.  All Participants may offer to sell into the ISO-administered Unforced Capacity auctions at their discretion.  The NEPOOL Manuals shall specify the dates by which the ISO will post the results of Unforced Capacity auctions.  The NEPOOL Manuals shall ensure that there are at least four business days between the times that auction results are posted and the dates that Participants are required to demonstrate that they have procured sufficient Unforced Capacity to cover their Unforced Capacity obligations pursuant to Section 8.2.2 of this Market Rule.

### UCAP Monthly Auctions.

Monthly Auctions will be held each Obligation Month during which Unforced Capacity may be purchased and sold for the forthcoming month, and any other month as specified in the NEPOOL Manuals.  The exact dates of each Monthly Auction shall be established in the NEPOOL Manuals.  Offers to sell and bids to purchase Unforced Capacity in the Monthly Auction shall be made in $/kW-month for the time period appropriate to the auction.  The ISO shall impose no limits on bids or offers in the auction, except to the extent required by any applicable market mitigation measures.  All Participants may offer to sell into Monthly Auctions at their discretion and such offers need not be associated with a specific ICAP Resource.  Each Monthly Auction shall establish the Market-Clearing Price for Unforced Capacity for the month in question.  In each Monthly Auction, Participants that are awarded Unforced Capacity shall pay the Market-Clearing Price for Unforced Capacity. Participants selected to supply Unforced Capacity shall receive the Market-Clearing Price for Unforced Capacity.

## Capacity Deficiencies and Deficiency Auctions.

### Participant Deficiencies.

#### UCAP Deficiency Auction.  If a Participant violates Sections 8.2.2 or 8.2.3 of this Market Rule by failing to procure sufficient Unforced Capacity to cover its Unforced Capacity obligation for an Obligation Month, the ISO shall attempt to procure sufficient Unforced Capacity to cover the remainder of the Participant’s Unforced Capacity obligation for that Obligation Month through a Deficiency Auction.  The ISO shall conduct a Deficiency Auction preceding the start of an Obligation Month.  The exact date of the Deficiency Auction shall be established in the NEPOOL Manuals.  The offers used in the Deficiency Auction shall be as follows:

##### The ISO shall use the sum of all calculated Participant deficiencies for the month in question as the total demand in the auction;

##### Participants shall submit offers for all Unforced Capacity that has not been sold either bilaterally or in the Monthly Auction for the month in question and may submit offers not linked to a specific ICAP Resource;

##### In the event that a Participant has Unforced Capacity that is not bid into the Deficiency Auction in accordance with Section 8.5.1 (a) (iii) above, the ISO shall enter an offer of $0.00/kW-month for the excess UCAP held by the Participant;

##### The mandatory offer provisions described in section 8.5.1 (a)(ii) shall not apply to resources that are de-listed from the ICAP market in accordance with the applicable procedures.

#### Deficiency Auction Clearing Price.  The clearing price for the Deficiency Auction shall be calculated based on this set of offers and total demand but shall not be permitted to exceed the Deficiency Rate.  To the extent all Participant deficiencies cannot be met in this auction, the Unforced Capacity procured in the auction shall be allocated to deficient Participants on a proportional basis, and the remaining deficiencies shall be charged in accordance with Section 8.5.1(c).  The ISO shall pay Participants with ICAP Resources selected to provide Unforced Capacity the clearing price associated with the Deficiency Auction for such Unforced Capacity.  Deficient Participants that are awarded Unforced Capacity shall pay to the ISO the clearing price associated with the Deficiency Auction for such Unforced Capacity.

#### Additional Deficiency Charges Imposed.  In the event that insufficient resources are available to meet the total demand in the Deficiency Auction, any Participants that are still deficient after the completion of a Deficiency Auction shall pay a monthly deficiency charge to the ISO equal to the Deficiency Rate specified in Section 8.5.1(d) multiplied by the number of MWs by which they are deficient. The ISO shall not reveal the number of MWs that Participants are deficient prior to a Deficiency Auction.

#### Deficiency Rate.  The Deficiency Rate applied in Section 8.5.1(c) shall be based on the calculated carrying cost of a peaking capacity unit, adjusted to an Unforced Capacity value.  As of the ICAP Effective Date, the calculated carrying cost of a simple cycle peaking capacity unit shall be $6.15/ kW-month measured on an ICAP basis.  The unit-specific equivalent forced outage rate used to convert this charge to a UCAP basis shall be 7.67%, which is the NERC GADS five year EFORd for a Gas Turbine, 50 MW or greater.  The Deficiency Rate shall be converted to a UCAP basis by dividing the ICAP rate by the quantity one minus the EFORd.  The resulting Deficiency Rate, expressed on a UCAP basis and applied in Section 8.5.1(c), shall be $6.66/ kW-month.

ISO-NE will allocate any Deficiency Charge revenues to: (a) all Participants with a Settlement Obligation for Unforced Capability that are not deficient prior to the Deficiency Auction and (b) all Participants with a surplus prior to the Deficiency Auction as follows:

Participant’s	=	Aggregate	x         Participant’s       +	Participant’s

Allocation		Deficiency Payments	UCAP Obligation (1)          UCAP Surplus

Aggregate          +	Aggregate

UCAP Obligations(1)         UCAP Surplus

(1) Only Participants who have met their obligation are included.

# FORWARD RESERVE MARKET

The Forward Reserve Market is a market to acquire Operating Reserves.  It is anticipated that when a spot market for reserve is implemented, the Forward Reserve Market will be redesigned to settle against the spot market.

## Forward Reserve Purchase Amount and Timing.

The ISO shall conduct an advance purchase of capability to satisfy the expected 10-minute non-synchronous and 30-minute Operating Reserve requirements.  The amount to be purchased is the expected full NPCC requirement for each of these respective products.  If the ISO expects to hold Replacement Reserve over and above the 30-minute Operating Reserve Requirement and above efficiencies available to the Control Area such as reserve sharing with neighboring Control Areas, that amount of Replacement Reserve will be added to the 30-minute requirement in the Forward Reserve Auction.

## Timing of Forward Reserve Auctions.

Forward Reserve Auctions will be held in advance of the Forward Reserve Service Period as specified in the NEPOOL Manuals.  Initially, auctions will be held 1 to 6 months in advance of the Service Period. The Service Period is the period during which the Resource must provide the Forward Reserve.

## Forward Reserve Auction Offers.

Forward Reserve Auction Offers shall be (a) made on a $/MW/Month basis and (b) Resource specific.  Separate Forward Reserve Auction Offers may be made to supply 10 and 30-Minute Forward Reserve.  Forward Reserve Auction Offers shall be submitted to the ISO by the Lead Participant for a generating Resource.  The Lead Participant is also responsible for complying with the requirements of this Section if the Forward Reserve Auction Offer is accepted.  Forward Reserve Auction Offers shall only be accepted from generating Resources.  The Forward Reserve Auction Offer must specify at the time of the Forward Reserve Auction whether the generating Resource shall be treated as an Off-Line or On-Line Forward Reserve Resource for the purposes of the Forward Reserve Market.

## Evaluation of Forward Reserve Auction Offers and Forward Reserve Clearing Prices.

There will be a 10-Minute Forward Reserve Clearing Price and a 30-Minute Forward Reserve Clearing Price.

Forward Reserve Auction Offers will be evaluated minimizing the total cost of the Forward Reserve Auction Offer quantities multiplied by the Forward Reserve Auction Offer prices.  The ISO shall substitute higher quality 10-Minute Forward Reserve for

lower quality 30-Minute Forward Reserve when it is economical to do so.  In addition, if the 30-Minute Forward Reserve Clearing Price is higher than the highest accepted offer for 10-Minute Forward Reserve, then the 10-Minute Forward Reserve Clearing Price will be set to the 30-Minute Forward Reserve Clearing Price.

A generating Resource may be selected in the Forward Reserve Auction to supply either 10-Minute Forward Reserve or 30-Minute Forward Reserve, consistent with its Forward Reserve Auction Offer.  Winners in the auction will receive either a 10-Minute Forward Reserve Delivery Requirement, or a 30-Minute Forward Reserve Delivery Requirement.  Winners are also required to adhere to the requirements, set forth below, of their chosen status of On-Line or Off-Line Forward Reserve Resources when satisfying the Delivery Requirements.

### Insufficient Offers.

In the event that the Forward Reserve Auction Offers are insufficient to satisfy the purchase requirement of 10-Minute Forward Reserve or 30-Minute Forward Reserve, then the 10-Minute Forward Reserve or 30-Minute Forward Reserve Clearing Price will be determined at the level of the highest accepted offer cleared for that product and that auction, consistent with the price determination of Section above.

### Forward Reserve Offer Cap.

Offers for 10-Minute Forward Reserve and 30-Minute Forward Reserve may not exceed the ICAP offer or price cap applicable to the Resource as defined in this Market Rule.

### Forward Reserve Supply Analysis.

The ISO shall publish the quantities of 10-minute and 30-minute Forward Reserve procured in the Auction and shall publish the quantities of such Forward Reserves that are available for use in satisfying the Operating Reserve requirements of the NEPOOL Control Area used in the Day-Ahead Energy Market, the Reserve Adequacy Analyses (as defined in the NEPOOL Manuals), and for each hour of the Real-Time Energy Market, in a monthly report.

## Product Definition and Resource Requirements.

### General Requirements.

#### Off-Line Forward Reserve Resources Flexibility.  Off-Line Forward Reserve Resources must satisfy the following criteria: (i) have a minimum run-time of not more than one hour, (ii) have a minimum down time of not more than one hour, and (iii) be able to produce energy within 30 minutes or less beginning from an off-line state.

#### On-Line Forward Reserve Resources Forego Day-Ahead Operating Reserve Credits.  If selected in the Forward Reserve Auction, On-Line Forward Reserve Resources are required to forego Day-Ahead Operating Reserve Credits applicable to Operating Days for which they have a Forward Reserve Delivery Requirement.  This will permit the ISO to evaluate the cost trade-off between (i) generating Resources that must be committed on-line, at extra operating cost, in order to provide Operating Reserve, and (ii) flexible resources which can provide Operating Reserve when off-line without extra operating cost.

#### ICAP Resource Requirement.  In order to qualify as Forward Reserve Resources, generating Resources must be listed as NEPOOL ICAP Resources in accordance with Section 8 of this Market Rule during the Forward Reserve Service Period, provided, however, that they do not have to be listed as ICAP Resources at the time of the Forward Reserve Auction.  In the event that the generating Resource fails to be listed as an ICAP Resource during an hour for which it has a 10-Minute or 30-Minute Forward Reserve Delivery Requirement, such generating Resource’s Delivered Forward Reserve shall be calculated as zero for the purposes of Sections  9.6.6 and  9.7 below.  Forward Reserve Resources may sell UCAP within NEPOOL in addition to Forward Reserve.

Forward Reserve Resources must also be under energy dispatch control of the ISO in Real-Time, be able to follow price-based energy dispatch signals, and have a Remote Intelligent Gateway.

External resources will be permitted to participate in the Forward Reserve Market when the respective Control Areas implement the technology and processes necessary to support recognition of operating reserve from external Resources.

### Delivery Service Period.

The Forward Reserve Service Period is the period during which delivery is made.  The beginning and end dates of the Forward Reserve Service Periods shall be provided in the Forward Reserve Auction announcement.  The length of the Forward Reserve Service Periods shall be provided in the NEPOOL Manuals.

## Delivery of Reserve.

### Dispatch and Energy Bidding of Reserve.

10-Minute and 30-Minute Forward Reserve shall be delivered by generating Resources offering the capability into both the Day-Ahead and Real-Time Energy Markets through Supply Offers at or above the Forward Reserve Strike Price (as calculated pursuant to Section 9.6.2 of this Market Rule).

The Forward Reserve Strike Price shall be set in accordance with the NEPOOL Manuals so that Forward Reserve capability has (a) a low probability of being dispatched for energy and (b) a high probability of being held for reserve purposes.

Forward Reserve Resources are scheduled and operated in accordance with Section 1 of this Market Rule; no distinction is made due to their status as Forward Reserve Resources.  Forward Reserve Resources may be eligible to set the Locational Marginal Price in accordance with Section 2 of this Market Rule.

### Forward Reserve Strike Prices.

The formula for determining the Forward Reserve Strike Price shall be fixed for the duration of the Forward Reserve Service Period.  The ISO will reevaluate the Forward Reserve Strike Price level for successive Forward Reserve Auctions on the basis of experience, expected operating conditions and other relevant information.

Forward Reserve Strike Price:  is calculated as the Forward Reserve Heat Rate multiplied by the Forward Reserve Fuel Index.

Forward Reserve Heat Rate:  shall be fixed for the duration of the Forward Reserve Service Period and announced in the announcement for the Forward Reserve Auction.  New Forward Reserve Heat Rates shall be specified for successive auctions.

Forward Reserve Fuel Index:  is the fuel index, or combination of indices, specified in the announcement of the Forward Reserve Auction.  The Forward Reserve Fuel Index for a Forward Reserve Service Period shall be specified in the Forward Reserve Auction announcement.

### Market Monitoring Unit.

The Market Monitoring Unit will receive information that will identify Forward Reserve Resources, the Forward Reserve Strike Price, and the 10 or 30-Minute Forward Reserve Delivery Requirement.  Forward Reserve Resources with Energy Offers at or above the Forward Reserve Strike Price which satisfy the 10 or 30-Minute Forward Reserve Delivery Requirement will not be required to pay the Failure to Reserve Penalty if mitigated below the Forward Reserve Strike Price.  Delivery accounting in Section 9.6.6 will use the mitigated offer.  Prior to mitigation, the Market Monitoring Unit consults with the Participant in accordance with Market Rule 1, Appendix A, Section 3.  The Market Monitoring Unit and the Participant shall consider the effect of any Forward Reserve Delivery Requirement in those consultations.

### Off-Line Forward Reserve Resources.

Off-Line Forward Reserve Resources shall be capable of satisfying the NERC and NPCC criteria for providing either 10 or 30-minute non-synchronous Operating Reserve. Their Day-Ahead and Real-Time Energy Supply Offers and other Resource characteristics must be capable of satisfying the NERC and NPCC criteria for providing either 10 or 30-minute non-synchronous Operating Reserve. An Off-Line Forward Reserve Resource is a generating Resource that has been awarded a Forward Reserve Delivery Requirement through the Forward Reserve Auction, and has designated itself as an Off-Line Forward Reserve Resource in its Forward Reserve Auction Offer.  An Off-Line Forward Reserve Resource shall be required to be able to satisfy the NERC and NPCC criteria for providing either 10 or 30-minute non-synchronous Operating Reserve, in accordance with its 10 or 30-Minute Forward Reserve Delivery Requirement.

Off-Line Qualifying Megawatts.  Off-Line Qualifying Megawatts are the amount of capability offered by an Off-Line Forward Reserve Resource at or above the Forward Reserve Strike Price.  The generating Resource must satisfy this requirement in both the Day-Ahead and the Real-Time Energy Markets.  The Off-Line Qualifying Megawatts shall be calculated as the lesser of the amount of capability offered by a generating Resource at  or above the Forward Reserve Strike Price in the Day-Ahead and Real-Time Energy Markets when there is a difference.  In the case of Off-Line Qualifying Megawatts, the offer measure shall include both the energy Supply Offer and a pro-rated amount of Start-Up and No-Load costs as defined below.

An Off-Line Forward Reserve Resource must offer its capability so that the following holds:

StartUpEcoMax×1 hour+NoLoadEcoMax+Energy Offeri≥ForwardReserveStrikePrice

where:

StartUp 	= 	the generating Resource’s Cold Start-Up fee.

NoLoad 	= 	the generating Resource’s No-Load fee.

EnergyOfferi = 	the generating Resource’s Energy Offer for Energy Offer block i.

EcoMax 	= 	the  Economic Maximum Limit.

The Off-Line Qualifying Megawatts must also be below the Resource-specific Economic Maximum Limit.

### On-Line Forward Reserve Resources.

On-Line Forward Reserve Resources shall be capable of satisfying the NERC and NPCC criteria for providing either 10 or 30-minute synchronous Operating Reserve.  Their Day-Ahead and Real-Time Energy Supply Offers and other Resource characteristics must be capable of satisfying the NERC and NPCC criteria for providing either 10 or 30-minute synchronous Operating Reserve for some portion of their unit capability above the Resource-specific Economic Minimum Limit.

To comply with the Delivery Requirement it has been awarded, an On-Line Forward Reserve Resource must either (a) be scheduled in the Day-Ahead Energy Market for all hours during which it has a 10-Minute or 30-Minute Forward Reserve Delivery Requirement, or (b) be Self-Scheduled in Real-Time at or above its Economic Minimum Limit during those hours.

On-Line Forward Reserve Resources will forego eligibility for Day-Ahead Operating Reserve Credits.

On-Line Qualifying Megawatts: is the capability offered at or above the Forward Reserve Strike Price by an On-Line Forward Reserve Resource.  The On-Line Forward Reserve Resource must satisfy this requirement in both the Day-Ahead and the Real-Time Energy Markets.  The On-Line Qualifying Megawatts for a generating Resource shall be calculated as the lesser of the amount of capability offered by a generating Resource at or above the Forward Reserve Strike Price and below the resource specific Economic Maximum in the Day-Ahead and Real-Time Energy Markets.

### Delivery Accounting.

“Delivered Forward Reserve” refers to the quantity of 10 or 30-Minute Forward Reserve delivered.

#### Delivered 10-Minute Forward Reserve

##### Delivered Off-Line 10-Minute Forward Reserve.  Delivered Off-Line 10-Minute Forward Reserve is calculated in megawatts as the minimum of:

- MW of NPCC 10 minute non-synchronous reserve that the generating Resource can provide, given real-time characteristics of the ramping rate, and Economic Maximum Limit, and

- Off-Line Qualifying Megawatts (energy at or above the Forward Reserve Strike Price per Section 9.6.4).

Delivered Off-Line 10-Minute Forward Reserve excludes any Self-Scheduled megawatts.

##### Delivered On-Line 10-Minute Forward Reserve.  Delivered On-Line 10-Minute Forward Reserve is calculated as the minimum of:

- 10 times the MW/minute ramping rate of the generating Resource.

- On-Line Qualifying Megawatts (MW offered at or above the Forward Reserve Strike Price per Section 9.6.5).

Delivered On-Line 10-Minute Forward Reserve excludes Self-Scheduled megawatts and megawatts below the Economic Minimum Limit.

#### Delivered 30-Minute Forward Reserve.  Delivered Forward Reserve for both On-Line and Off-Line 30-Minute Forward Reserve is reduced by the amount of Delivered 10-Minute Forward Reserve already calculated for the generating Resource.  The Delivered 10-Minute Forward Reserve can be applied to a 30-Minute Reserve Delivery Requirement as discussed further below.

##### Delivered Off-Line 30-Minute Forward Reserve.  Delivered Off-Line 30-Minute Forward Reserve is calculated as the minimum of:

- MW of NPCC 30 minute non-synchronous reserve that the generating Resource can provide, given actual real-time characteristics of ramping rate, Economic Minimum and Economic Maximum Limits, and

- Off-Line Qualifying Megawatts (energy at or above the Forward Reserve Strike Price per Section 9.6.4),

less any 10-Minute Delivered Forward Reserve calculated.

Delivered Off-Line 30-Minute Forward Reserve excludes Self-Scheduled capability.

##### Delivered On-Line 30-Minute Forward Reserve.  Delivered On-Line 30-Minute Forward Reserve is calculated as the minimum of:

- 30 times the MW/minute ramping rate of the generating Resource;

- On-Line Qualifying Megawatts (MW offered at or above the Forward Reserve Strike Price per Section 9.6.5 );

less any Delivered 10-Minute Forward Reserve calculated.

Delivered On-Line 30-Minute Forward Reserve excludes Self-Scheduled capability and capability below the Economic Minimum Limit.

Delivered 10-Minute Forward Reserve will be applied first to the higher value 10-Minute Forward Reserve Delivery Requirement.  Surplus Delivered 10-Minute Forward Reserve will be applied to any 30-Minute Forward Reserve Delivery Requirement.

## Consequences of Delivery Failure.

### Failure to Reserve.

A “Failure to Reserve” occurs when the Delivered 10 or 30-Minute Off-Line or On-Line Forward Reserve is less than the 10-Minute or 30-Minute Forward Reserve Delivery Requirement for a Forward Reserve Resource.  Under these circumstances the Forward Reserve Resource not only foregoes payment for the megawatts not delivered, but may also pay a penalty in addition to the foregone Forward Reserve Auction revenues.

Failure to Reserve:  A Forward Reserve Resource is determined to have a Failure to Reserve when either its Day-Ahead or Real-Time Supply Offer causes its Delivered 10 or 30-Minute Off-Line or On-Line Forward Reserve to fall below its 10 or 30-Minute Forward Reserve Delivery Requirement.  Failure to Reserve is measured by the megawatts of Forward Reserve Shortfall as defined below.  The failure is specific to either 10 or 30-Minute Forward Reserve Delivery Requirements.

Forward Reserve Shortfall:  The Forward Reserve Shortfall is defined as, for any hour, the amount  (MW 10 or 30-Minute Forward Reserve Delivery Requirement – MW Delivered 10 or 30 Minute On-Line or Off-Line Forward Reserve).  The shortfall is specific to either 10 or 30-Minute Delivery Requirements.  In the event that the Forward Reserve Resource is undergoing a Planned Outage that has been approved by the ISO in accordance with the maintenance scheduling provisions of the NEPOOL Manuals and Operating Procedures, this value shall be calculated as zero.  The Forward Reserve Shortfall is measured for both Day-Ahead and Real-Time offers.  The final value used is the greater of the two (Real-Time or Day-Ahead) Forward Reserve Shortfalls.

Failure to Reserve Penalty:  The Failure to Reserve Penalty is defined as:

Failure to Reserve Penalty = Max[Day-Ahead Nodal LMP – Forward Reserve Strike Price, 0] x Forward Reserve Shortfall

Consequences of Failure to Reserve:  For every MW of Forward Reserve Shortfall, or Failure to Reserve, the generating Resource will: (i) forego 100% of the payment of the Forward Reserve Market compensation for the MW of shortfall; and (ii) pay the Failure to Reserve Penalty.

### Failure to Activate Reserve.

On-Line and Off-Line Forward Reserve Resources are required to pay the Replacement Energy Cost when they fail to activate their capability per dispatch instructions, and the Real-Time Nodal LMP is at or above the Forward Reserve Strike Price (“Failure to Activate Reserve”).

Determination and measurement of Failure to Activate Reserve is set forth in the NEPOOL Manuals.

When a Failure to Activate Reserve occurs, the Forward Reserve Resource that has been determined by the ISO to have failed to activate shall be required to pay the Replacement Energy Cost.  The Replacement Energy Cost is set at:

Replacement Energy Cost = Min[Delivered Forward Reserve, Forward Reserve Delivery Requirement] x 1 hour x Max[Real-Time Nodal LMP – Strike Price, 0]

### Known Performance Limitations.

The ISO may have reason to believe that a particular Forward Reserve Resource is frequently receiving, or may frequently receive, Forward Reserve payments for a portion or all of its capability that is not capable of activating the 10 or 30-Minute Forward Reserve that has been delivered in accordance with Section 9.6.

When the ISO believes there is such a limited generating Resource in the Forward Reserve Market, then it will contact and confer with the Lead Participant before taking any action.

#### The ISO will, whenever practicable, contact the Lead Participant of the Forward Reserve Resource to request an explanation of the relevant resource Offer Data;

#### If the explanation, if available, considered together with other information available to the ISO, indicates to the satisfaction of the ISO that the questioned Forward Reserve payments are consistent with generating Resource capabilities, no further action will be taken; and

#### If no agreement is reached, or acceptable explanation is provided, the ISO may call for Resource Performance Audits as specified in NEPOOL Manuals.  If the generating Resource fails the Performance Audit, the ISO may take remedial action. Remedial actions may include, but are not limited to: (i) redeclaration, by the ISO, of any relevant operational Offer Data parameter, or (ii) removing the asset or the relevant portion of the asset’s capability from the Forward Reserve Market on a going-forward basis.

## Forward Reserve Credits and Charges.

Settlement for the Forward Reserve Auction shall be done on an hourly basis and billed monthly.

### Calculating a Resource’s Forward Reserve Position.

Payments to (or collections from if negative) a Resource for the product 10-Minute Forward Reserve during one hour are defined as:

(10-Minute Hourly Forward Reserve Clearing Price multiplied by Min[Delivered  10-Minute Forward Reserve, 10-Minute Forward Reserve Delivery Requirement])  minus

Failure to Reserve Penalty  minus

Replacement Energy Cost (Section 9.7.2).

Payments to (or collections from if negative) a Resource for the product 30-Minute Forward Reserve during one hour is defined as:

(30-Minute Hourly Forward Reserve Clearing Price multiplied by Min[Delivered 30-Minute Forward Reserve, 30-Minute Forward Reserve Delivery Requirement])  minus

Failure To Reserve Penalty minus

Replacement Energy Cost (Section 9.7.2).

### Forward Reserve Credits and Charges.

The total Forward Reserve Credits for the hour is the sum of Forward Reserve Payments for all Forward Reserve products for the hour.

The sum of the Forward Reserve Credits shall be allocated to the NEPOOL Participants as a Forward Reserve Charge in accordance with Section 9.8.3.

### Calculating a Participant’s Forward Reserve Charge.

The costs of the Forward Reserve Market shall be allocated on the basis of hourly Real-Time Load Obligations (RTLO) ratio share.

The ISO calculates for each hour the Forward Reserve Charges by allocating the total Forward Reserve Credits paid (or collected if negative) to each Participant based on the Participant’s hourly ratio share of NEPOOL-wide Real-Time Load Obligation (Participant’s Real-Time Load Obligation at all Locations / Total of all Participants’ Real-Time Load Obligations at all Locations).

10.	GAP RFPs FOR RELIABILITY PURPOSES

## 10.1	Request For Proposals for Load Response and Supplemental Generation Resources for Reliability Purposes.

(a)	Should the ISO determine that a region may have potential critical near-term power supply reliability problems for which no Participant has proposed or committed to implement a viable solution (from a timeliness or financial standpoint), the ISO may, after consultation with the NEPOOL Reliability Committee, issue a request for proposals (Gap RFP).  The Gap RFP will solicit Load Response and supplemental generating Resources to maintain near-term reliability in the identified region.  For any Gap RFP issued after December 31, 2003, the ISO shall file such Gap RFP with the Commission for approval at least 60 days prior to its issuance.  The filing shall include proposed Gap RFP terms and conditions and shall explain why market incentives were unable to solicit a market response in the absence of the Gap RFP.

(b)	The ISO may enter into contracts awarded pursuant to a competitive Gap RFP process.  Bidders that are awarded contracts through the Gap RFP process shall file those contracts with the Commission for approval of the rates to be charged thereunder to the extent that such contracts are for services that are jurisdictional to the Commission.  All other contracts entered into pursuant to a Gap RFP shall be filed with the Commission for informational purposes.

(c)	The costs for Load Response and other generation Resources selected through any Gap RFP issued by the ISO pursuant to this Section 10 shall be allocated and charged in the same manner as fixed-cost charges associated with RMR Resources under Section 6.4.4(c) of Market Rule 1.

Sheet Nos. 107 through 200 are reserved for future use.