II.44 Scheduling and Curtailment Rules

For purposes of scheduling and Curtailment of Real-Time External Transactions over interconnections between the New England Control Area and neighboring Control Areas, the following rules shall apply:

(1) For External Interfaces that are not subject to Coordinated Transaction Scheduling

- Real-Time External Transaction sales and purchases that (i) are supported by those service agreements referenced in Attachment G-3 to this OATT that have not opted for Auction Revenue Rights consideration under applicable ISO System Rules or (ii) are supported by those service agreements referenced in Attachment H to this OATT, and (iii) have been submitted into the Real-Time Energy Market prior to the Day-Ahead Energy Market Scheduling deadline established in Section III.1.10.1A of the Tariff  as a Self-Scheduled Real-Time External Transaction (“real-time without price”) at an External Node referenced in Attachment G-3 or Attachment H to this OATT shall be assigned the highest transmission priority when compared to other Real-Time External Transaction purchases or sales at that node having the same offer price or bid price. In the event that the transfer limit for a given external interface does not allow all Excepted Transactions or MEPCO Grandfathered Transactions submitted over that interface to flow, they shall be scheduled or curtailed on a pro-rata basis. For Real-Time External Transactions referenced in Attachment G-3 or Attachment H to this OATT that also require an advance physical reservation associated with a MTF or OTF external interface, the MTF or OTF transmission priority shall take precedence over the above language for the purposes of scheduling and curtailment under Sections II.44(1)(c) and II.44(1)(d) of this OATT, respectively. For Excepted Transactions or MEPCO Grandfathered Transactions that are tied within economic merit, and tied within transmission priority, such transactions cleared in the Day-Ahead Energy Market that have a corresponding Real-Time Energy Market External Transaction will have scheduling and curtailment priority in the Real-Time Energy Market before Excepted Transactions or MEPCO Grandfathered Transactions not cleared in the Day-Ahead Energy Market;

- For external interfaces where advance physical reservations are not required, in the event …