HVDC TRANSMISSION OPERATING AGREEMENT

ATTACHMENT M

Transmission Line Ratings

General:

Beginning July 12, 2025, the Asset Owners will implement HVDC-TF Transmission Line Ratings on the HVDC-TF, as provided below.

Definitions:

The following definitions apply for purposes of this Attachment:

(1) “HVDC-TF Transmission Line Rating” means the maximum transfer capability of an HVDC-TF, computed in accordance with a written HVDC-TF Transmission Line Rating methodology and consistent with Good Utility Practice, considering the technical limitations on conductors and relevant transmission equipment (such as thermal flow limits), as well as technical limitations of the New England Transmission System (such as system voltage and stability limits).  Relevant transmission equipment may include, but is not limited to, circuit breakers, line traps, and transformers.

(2) “Ambient-Adjusted Rating” (“AAR”) means an HVDC-TF Transmission Line Rating that:

(a) Applies to a time period of not greater than one hour.

(b) Reflects an up-to-date forecast of ambient air temperature across the time period to which the rating applies.

(c) Reflects the absence of solar heating during nighttime periods, where the local sunrise/sunset times used to determine daytime and nighttime periods are updated at least monthly, if not more frequently.

(d) Is calculated at least each hour, if not more frequently.

(3) “Seasonal HVDC-TF Line Rating” as further defined in any applicable ISO New England Operating Documents, means an HVDC-TF Transmission Line Rating that:

(a) Applies to a specified season, where seasons are defined  to include not fewer than four seasons in each year, and to reasonably reflect portions of the year where expected high temperatures are relatively consistent.

(b) Reflects up-to-date historical ambient air temperatures across the relevant season over which the rating applies.

(c) Is calculated annually, if not more frequently, for each season in the future for which Phase I/II HVDC-TF Service under Schedule 20A to the ISO OATT can be requested.

(4) “Near-Term Phase I/II HVDC-TF  Service” shall have the meaning set forth in Schedule 20A to the ISO OATT.

(5) “Emergency HVDC-TF Rating,” as further defined in any applicable ISO New England Operating Documents, means an HVDC-TF Transmission Line Rating that reflects operation for a specified, finite period, rather than reflecting continuous operation.  An Emergency HVDC-TF Rating may assume an acceptable loss of equipment life or other physical or safety limitations for the equipment involved.

System Reliability:

If an Asset Owner reasonably determines, consistent with Good Utility Practice, that the temporary use of an HVDC-TF Transmission Line Rating for a facility of that Asset Owner different than would otherwise be required by this Attachment is necessary to ensure the safety and reliability of the New England Transmission System, then the Asset Owner may use such an alternate rating.  The applicable Asset Owner and the ISO shall confirm the use of such an alternate rating consistent with any applicable procedures established in ISO New England Operating Documents and consistent with this HVDC TOA. The Asset Owner must document in its database of HVDC-TF Transmission Line Ratings and HVDC-TF Transmission Line Rating methodologies on OASIS or another password-protected website, as required by this Attachment, the use of an alternate HVDC-TF Transmission Line Rating under this paragraph, including the nature of and basis for the alternate rating, the date and time that the alternate rating was initiated, and (if applicable) the date and time that the alternate rating was withdrawn and the standard rating became effective again.

Obligations of the Asset Owners:

The Asset Owners will have the following obligations:

Each Asset Owner must use uniquely determined Emergency HVDC-TF Ratings, as specified in any applicable ISO New England Operating Documents and as determined consistent with the rights and obligations of the Parties under this HVDC TOA, for contingency analysis and in post-contingency simulations of constraints.  Such uniquely determined Emergency HVDC-TF Ratings must also include separate AAR calculations for each Emergency HVDC-TF Rating duration used.

The Asset Owners shall, consistent with this Attachment, this HVDC TOA, and any applicable ISO New England Operating Documents, calculate and electronically transmit AARs, Emergency HVDC-TF Ratings and Seasonal HVDC-TF Line Ratings to the ISO and Schedule 20A Service Providers.

In utilizing forecasts of ambient air temperature for AARs and evaluating historical temperatures for Seasonal HVDC-TF Line Ratings, the Asset Owners shall utilize such forecasts and historical temperatures consistent with Good Utility Practice and on a non-discriminatory basis.

Postings to OASIS or another password-protected website:  The Asset Owners must send to the Schedule 20A Service Providers to maintain on the password-protected section of the Schedule 20A Service Providers’ OASIS page or on another password-protected website a database of HVDC-TF Transmission Line Ratings and HVDC-TF Transmission Line Rating methodologies.

The database must include a full record of all HVDC-TF Transmission Line Ratings, both as used in real-time operations, and as used for all future periods for which Phase I/II HVDC-TF Service is offered.  Any postings of temporary alternate HVDC-TF Transmission Line Ratings or exceptions used under the System Reliability section above or the Exceptions section below, respectively, are considered part of the database.  The database must include records of which HVDC-TF Transmission Line Ratings and HVDC-TF Transmission Line Rating methodologies were in effect at which times over at least the previous five years, including records of which temporary alternate HVDC-TF Transmission Line Ratings or exceptions were in effect at which times during the previous five years.  Each record in the database must indicate which transmission line the record applies to, and the date and time the record was entered into the database.  The database must be maintained such that users can view, download, and query data in standard formats, using standard protocols.

Sharing with Transmission Providers:  Each Asset Owner must share, upon request by any transmission provider and in a timely manner, the following information:

(1) HVDC-TF Transmission Line Ratings for each period for which HVDC-TF Transmission Line Ratings are calculated, with updated ratings shared each time HVDC-TF Transmission Line Ratings are calculated, and

(2) Written HVDC-TF Transmission Line Rating methodologies used to calculate the Transmission Line Ratings in (1) above.

Exceptions:  Where an Asset Owner determines, consistent with Good Utility Practice, that the HVDC-TF Transmission Line Rating of a transmission line is not affected by ambient air temperature or solar heating, the Asset Owner may, consistent with the rights and obligations of the Parties under this HVDC TOA, provide an HVDC-TF Transmission Line Rating for that transmission line that is not an AAR or Seasonal HVDC-TF Line Rating.  Examples of such a transmission line may include (but are not limited to):  (1) a transmission line for which the technical transfer capability of the limiting conductors and/or limiting transmission equipment is not dependent on ambient air temperature or solar heating; or (2) a transmission line whose transfer capability is limited by a transmission system limit (such as a system voltage or stability limit) which is not dependent on ambient air temperature or solar heating.  The Asset Owner must document in its database of HVDC-TF Transmission Line Ratings and Transmission Line Rating methodologies to be posted on the OASIS or other password-protected website maintained by the Schedule 20A Service Providers any exceptions to the requirements contained in this Attachment initiated under this paragraph, including the nature of and basis for each exception, the date(s) and time(s) that the exception was initiated, and (if applicable) the date(s) and time(s) that each exception was withdrawn and the standard rating became effective again.  If the technical basis for an exception under this paragraph changes, then the applicable Asset Owner must update the relevant HVDC-TF Transmission Line Rating(s) in a timely manner.  Each Asset Owner must reevaluate any exceptions taken under this paragraph at least every five years.